ontario regulation 477/16

made under the

Environmental Protection Act

Made: November 30, 2016
Filed: December 19, 2016
Published on e-Laws: December 19, 2016
Printed in The Ontario Gazette: January 7, 2017

Amending O. Reg. 535/05

(ETHANOL IN GASOLINE)

1.  (1)  The definition of “cellulosic ethanol” in subsection 1 (1) of Ontario Regulation 535/05 is revoked and the following substituted:

“cellulosic ethanol” means ethanol derived,

(a)	solely from either or both of lignocellulosic or hemicellulosic matter that is available on a renewable or recurring basis, or

(b)	from the biogenic carbon component of solid municipal waste, other than solid municipal waste that contains,

(i)	source separated organics, except for residues generated by the processing of the waste,

(ii)	compost produced by composting,

(iii)	anaerobic digestion output, or

(iv)	leaf and yard waste collected or accepted by a leaf and yard waste system to which section 13 of Ontario Regulation 101/94 (Recycling and Composting of Municipal Waste) made under the Act applies, except for residues generated by the composting of the waste at a leaf and yard waste composting site as defined in Part V of that regulation;

(2)  The definition of “Northern Ontario” in subsection 1 (1) of the Regulation is revoked.

(3)  Subsection 1 (1) of the Regulation is amended by adding the following definition:

“source separated organics” has the same meaning as in Ontario Regulation 79/15 (Alternative Low-Carbon Fuels) made under the Act.

(4)  Section 1 of the Regulation is amended by adding the following subsection:

(1.1)  In this Regulation, the following words and expressions have the same meanings as in Regulation 347 of the Revised Regulations of Ontario, 1990 (General — Waste Management) made under the Act:

1.	Anaerobic digestion output.

2.	Composting.

3.	Municipal waste.

2.  (1)  Subsections 2 (1) and (2) of the Regulation are revoked and the following substituted:

(1)  No person shall distribute ethanol-blended gasoline for use or sale in Ontario unless the ethanol-blended gasoline meets the standards and specifications set out in Canadian General Standards Board (C.G.S.B.) document CAN/CGSB-3.511-2016 or the Canadian General Standards Board (C.G.S.B.) document CAN/CGSB-3.512-2013, each as amended from time to time, or an equivalent set of standards and specifications that is approved in writing by the Director before the distribution of the ethanol-blended gasoline.

(2)  Ethanol-blended gasoline is deemed to meet ethanol quality standards for the purposes of subsection (1) if it meets the ethanol quality standards set out in the American Society for Testing and Materials (A.S.T.M.) document ASTM D4806-16a, as amended from time to time, rather than the ethanol quality standards set out in the Canadian General Standards Board (C.G.S.B.) document CAN/CGSB-3.516-2011, as amended from time to time.

(2)  Subsections 2 (6) and (7) of the Regulation are revoked.

3.  Subsection 3 (3) of the Regulation is revoked.

4.  (1)  Subsection 4 (1) of the Regulation is amended by striking out the formula and substituting the following:

x = ([C + (C1 × 2.5)] + D – E) / (A + B – F) × 100

(2)  The definitions of “F” and “G” in subsection 4 (1) of the Regulation are revoked and the following substituted:

“F”	 is the volume of ethanol-blended gasolines and non-ethanol-blended gasolines placed in the Ontario market in the compliance year for the purposes of generating power in aircraft, marine vessels or equipment, off-road vehicles or equipment, or vehicles manufactured prior to 1980 if the fuel supplier has so elected under subsection 3 (4).

(3)  Section 4 of the Regulation is amended by adding the following subsection:

(3)  The following rules apply for the purposes of subsection (1) when calculating the volume of cellulosic ethanol in ethanol derived from solid municipal waste:

1.	The biogenic carbon component shall be determined in accordance with Method B or Method C of the American Society for Testing and Materials (A.S.T.M.) document ASTM D6866-16, as amended from time to time, or in accordance with an equivalent set of standards and specifications that is approved in writing by the Director.

2.	The biogenic carbon component shall be determined from a composite sample comprised of a volume-weighted combination of samples collected weekly during a quarterly period, where a quarterly period starts on either January 1, April 1, July 1 or October 1.

3.	The biogenic carbon component that is determined for a quarterly period applies to the ethanol produced during that quarter.

5.  Clause 5 (2) (c) of the Regulation is revoked.

Commencement

6.  This Regulation comes into force on the later of January 1, 2017 and the day it is filed.

Back to top