ontario regulation 302/17

made under the

Ontario Water Resources Act

Made: July 26, 2017
Filed: July 28, 2017
Published on e-Laws: July 31, 2017
Printed in The Ontario Gazette: August 12, 2017

Amending O. Reg. 129/04

(LICENSING OF SEWAGE WORKS OPERATORS)

1.  Subsection 4 (4) of Ontario Regulation 129/04 is amended by striking out “Schedule 1” and substituting “Schedule 1, 1.1 or 1.2”.

2.  Schedule 1 to the Regulation is revoked and the following substituted:

Schedule 1
FACILITY CLASSIFICATION POINT SYSTEMS

1.  A wastewater collection facility is classified in accordance with Table 1 of this Schedule, based on the number of points applicable to the facility under the point system described in Schedule 1.1.

2.  A wastewater treatment facility is classified in accordance with Table 1 of this Schedule, based on the number of points applicable to the facility under the point system described in Schedule 1.2.

3.  (1)  For the purpose of section 2, the number of points applicable to a package wastewater treatment facility under the point system described in Schedule 1.2 may be reduced by the Director if he or she is satisfied that the operational needs of the package wastewater treatment facility are less than the operational needs of other wastewater treatment facilities.

(2)  In subsection (1),

“package wastewater treatment facility” means a wastewater treatment facility where the treatment processes of the facility meet the following criteria:

1.	The facility has been manufactured as a complete unit.

2.	The facility has been preassembled and delivered in not more than four modules to the site where it is used.

3.	The facility has a design flow of 1,300 cubic metres per day or less.

TABLE 1
CLASSES OF FACILITIES

| Item | Column 1 Class | Column 2 Number of Points |
| --- | --- | --- |
| 1. | Class I | 30 or less |
| 2. | Class II | 31 to 55 |
| 3. | Class III | 56 to 75 |
| 4. | Class IV | 76 or more |

Schedule 1.1
WASTEWATER COLLECTION FACILITIES POINT SYSTEM

1.  The number of points applicable to a wastewater collection facility is determined by applying the Tables to this Schedule:

Table 1
points based on Population served

| Item | Column 1 Population served | Column 2 Number of Points |
| --- | --- | --- |
| 1. | 0 to 5,000 people | 0 |
| 2. | 5,001 to 50,000 people | 1 |
| 3. | 50,001 to 100,000 people | 2 |
| 4. | More than 100,000 people | 3 |

Table 2
points based on Type of service

| Item | Column 1 Type of service | Column 2 Number of Points |
| --- | --- | --- |
| 1. | Facility includes combined sanitary and storm sewers, with length of combined sewers equal to 10% or less of total | 2 |
| 2. | Facility includes combined sanitary and storm sewers, with length of combined sewage equal to more than 10% of total | 4 |
| 3. | Facility collects industrial sewage, with volume of industrial sewage equal to 20% or less of total | 2 |
| 4. | Facility collects industrial sewage, with volume of industrial sewage equal to more than 20% of total | 4 |

Table 3
points based on PUMPING STATIONS

| Item | Column 1 Pumping Stations | Column 2 Number of Points |
| --- | --- | --- |
| 1. | Facility has pumping station | 10 |
| 2. | Pumping station is generator supported | 2 |
| 3. | Pumping station has overflow chamber | 2 |
| 4. | Pumping station has grit chamber | 2 |
| 5. | Pumping station has alarm | 2 |
| 6. | Pumping station has odour control | 2 |
| 7. | Pumping station staffed 24 hours per day | 10 |

Table 4
points based on Force mains

| Item | Column 1 Force Mains | Column 2 Number of Points |
| --- | --- | --- |
| 1. | Facility has force mains | 10 |
| 2. | Total length of force mains exceeds 1 kilometre | 4 |
| 3. | Force mains have active cathodic protection | 2 |

Table 5
points based on Age of facility materials

| Item | Column1 Age of Facility Materials | Column 2 Number of Points |
| --- | --- | --- |
| 1. | More than 10% of facility materials pre-date 1950 | 4 |
| 2. | More than 50% of facility materials pre-date 1970 | 4 |

Table 6
points based on by-pass chamber

| Item | Column 1 By-pass Chamber | Column 2 Number of Points |
| --- | --- | --- |
| 1. | Facility has by-pass chamber | 10 |
| 2. | By-pass chamber has alarm | 2 |
| 3. | By-pass chamber has disinfection | 2 |

Table 7
points based on special features

| Item | Column 1 Special Features | Column 2 Number of Points |
| --- | --- | --- |
| 1. | Facility has flow equalization or fill and draw chambers | 4 |
| 2. | Facility has inverted siphons | 4 |
| 3. | Facility has access deeper than 10 metres | 6 |

SCHEDULE 1.2
WASTEWATER TREATMENT FACILITIES POINT SYSTEM

1.  The number of points applicable to a wastewater treatment facility is determined by applying the following rules and the Tables to this Schedule:

1.	For every 10,000 people served by a facility, the number of points applicable to the facility is one point, up to a maximum of 10 points.

2.	For every 4,500 cubic metres of flow at a facility, the number of points applicable to the facility is one point, up to a maximum of 10 points.  The flow at the facility is the greater of the following:

i.	The average daily design flow.

ii.	The average daily flow in peak month.

3.	The maximum number of points applicable to a facility with respect to effluent discharge, as determined by applying Table 2 of this Schedule, is six.

4.	The number of points applicable to a facility with respect to bacteriological and biological laboratory control by facility personnel, as determined by applying Table 9 of this Schedule, is the greatest of the numbers set out in Column 2 of that Table that applies to the facility.

5.	The number of points applicable to a facility with respect to chemical and physical laboratory control by facility personnel, as determined by applying Table 10 of this Schedule, is the greatest of the numbers set out in Column 2 of that Table that applies to the facility.

Table 1
points based on Raw waste flow and toxicity

| Item | Column 1 Raw Waste Flow and Toxicity | Column 2 Number of Points |
| --- | --- | --- |
| 1. | Variations in flow are less than 100% of average values and raw waste is not subject to toxic waste discharges | 0 |
| 2. | Variations in flow are 100% to 200% of average values and raw waste is not subject to toxic waste discharges | 2 |
| 3. | Variations in flow are more than 200% of average values and raw waste is not subject to toxic waste discharges | 4 |
| 4. | Raw waste is subject to toxic waste discharges | 6 |

Table 2
points based on effluent discharge

| Item | Column 1 Effluent Discharge | Column 2 Number of Points |
| --- | --- | --- |
| 1. | Receiving water sensitivity to effluent discharge: secondary treatment is adequate | 1 |
| 2. | Receiving water sensitivity to effluent discharge: more than secondary treatment is required, but a very high degree of treatment is not required | 2 |
| 3. | Receiving water sensitivity to effluent discharge: very high degree of treatment is required | 3 |
| 4. | Facility uses evaporation to dispose of all effluent | 2 |
| 5. | Facility uses land spraying to dispose of all effluent | 4 |

Table 3
points based on pretreatment

| Item | Column 1 Pretreatment | Column 2 Number of Points |
| --- | --- | --- |
| 1. | Facility uses screening or comminution | 3 |
| 2. | Facility has grit removal | 3 |
| 3. | Facility uses plant pumping of main flow | 3 |
| 4. | Facility uses chemical precipitation or pH adjustment | 6 |

Table 4
points based on primary treatment

| Item | Column 1 Primary Treatment | Column 2 Number of Points |
| --- | --- | --- |
| 1. | Facility uses primary clarifiers | 5 |
| 2. | Facility uses combined sedimentation and digestion | 5 |
| 3. | Facility uses chemical addition, other than for disinfection | 4 |

Table 5
points based on secondary treatment

| Item | Column 1 Secondary Treatment | Column 2 Number of Points |
| --- | --- | --- |
| 1. | Facility uses trickling filter with secondary clarifiers | 10 |
| 2. | Facility uses activated sludge with secondary clarifiers | 15 |
| 3. | Facility uses stabilization pond without aeration | 5 |
| 4. | Facility uses aerated lagoon | 8 |

Table 6
points based on advanced waste treatment

| Item | Column 1 Advanced Waste Treatment | Column 2 Number of Points |
| --- | --- | --- |
| 1. | Facility uses polishing pond | 2 |
| 2. | Facility uses advanced chemical or physical treatment without secondary treatment | 15 |
| 3. | Facility uses advanced chemical or physical treatment with secondary treatment | 10 |
| 4. | Facility uses advanced biological treatment | 12 |
| 5. | Facility uses ion exchange | 10 |
| 6. | Facility uses reverse osmosis or electrodialysis | 15 |
| 7. | Facility uses chemical recovery or carbon regeneration | 4 |

Table 7
points based on Solids handling

| Item | Column 1 Solids Handling | Column 2 Number of Points |
| --- | --- | --- |
| 1. | Facility uses thickening | 5 |
| 2. | Facility uses anaerobic digestion | 10 |
| 3. | Facility uses aerobic digestion | 6 |
| 4. | Facility uses evaporative sludge drying | 2 |
| 5. | Facility uses mechanical dewatering | 8 |
| 6. | Facility uses incineration, gasification, pyrolysis, plasma arc treatment or another method of thermal treatment | 12 |
| 7. | Facility uses wet oxidation | 12 |

Table 8
points based on disinfection

| Item | Column 1 Disinfection | Column 2 Number of Points |
| --- | --- | --- |
| 1. | Facility uses chlorination or a comparable process | 5 |
| 2. | Facility has on-site generation of disinfectant | 5 |

Table 9
points based on bacteriological and biological laboratory control by facility personnel

| Item | Column 1 Bacteriological and Biological Laboratory Control by Facility Personnel | Column 2 Number of Points |
| --- | --- | --- |
| 1. | Lab work is done outside the facility | 0 |
| 2. | Facility personnel do membrane filter procedures | 3 |
| 3. | Facility personnel use fermentation tubes or other dilution methods or do fecal coliform determinations | 5 |
| 4. | Facility personnel do biological identification | 7 |
| 5. | Facility personnel do virus studies or similar work | 10 |

Table 10
points based on chemical and physical laboratory control by facility personnel

| Item | Column 1 Chemical and Physical Laboratory Control by Facility Personnel | Column 2 Number of Points |
| --- | --- | --- |
| 1. | Lab work is done outside the facility | 0 |
| 2. | Facility personnel use push-button or visual methods, to do simple determinations such as pH or settleable solids | 3 |
| 3. | Facility personnel do DO, COD, BOD, volatile content or solids determinations, gas analysis, tirations, or similar tests | 5 |
| 4. | Facility personnel do determinations for specific constituents, nutrients, total oils or phenols, or similar determination | 7 |
| 5. | Facility personnel perform procedures using very sophisticated instrumentation, such as atomic absorption or gas chromatography | 10 |

Commencement

3.  This Regulation comes into force on the day it is filed.

Back to top