ontario regulation 313/17

made under the

Environmental Protection Act

Made: July 26, 2017
Filed: July 28, 2017
Published on e-Laws: July 31, 2017
Printed in The Ontario Gazette: August 12, 2017

Amending O. Reg. 63/95

(EFFLUENT MONITORING AND EFFLUENT LIMITS - ORGANIC CHEMICAL MANUFACTURING SECTOR)

1.  (1)  The definition of “assessment parameter” in subsection 1 (1) of Ontario Regulation 63/95 is amended by adding “the Table for the plant in” before “Schedule 6”.

(2)  The definition of “limited parameter” in subsection 1 (1) of the Regulation is revoked and the following substituted:

“limited parameter”,

(a)	in relation to a plant to which Table 1 or 2 in Schedule 4 applies, means a parameter for which a limit is specified in Column 3 of the Table in Schedule 4, and

(b)	in relation to any other plant, means a parameter for which a limit is specified in Column 3 or 4 of the Table for the plant in Schedule 2;

(3)  The definition of “merged parameter” in subsection 1 (1) of the Regulation is amended by adding “the Table for the plant in” before “Schedule 5”.

2.  (1)  Subsection 8 (1) of the Regulation is amended by striking out “Schedule 2 for the discharger’s plant” at the end and substituting “the heading of the Table for the discharger’s plant in Schedule 2”.

(2)  Subsection 8 (2) of the Regulation is amended by striking out “Schedule 4 for the discharger’s plant” at the end and substituting “the heading of the Table for the discharger’s plant in Schedule 4”.

(3)  Subsection 8 (3) of the Regulation is amended by striking out “Schedule 5 for the discharger’s plant” at the end and substituting “the heading of the Table for the discharger’s plant in Schedule 5”.

(4)  Subsection 8 (4) of the Regulation is amended by striking out “Schedule 6 for the discharger’s plant” at the end and substituting “the heading of the Table for the discharger’s plant in Schedule 6”.

3.  (1)  Subsections 16 (1) and (2) of the Regulation are amended by adding “the Table for the plant in” before “Schedule 2” wherever it appears.

(2)  Subsection 16 (3) of the Regulation is amended by adding “the Table for the plant in” before “Schedule 4”.

4.  Section 17 of the Regulation is revoked and the following substituted:

17.  Each discharger shall control the quality of each stream at the discharger’s plant for which a sampling point is listed opposite the plant in Column 2 of Schedule 7 to ensure that each rainbow trout acute lethality test and each Daphnia magna acute lethality test performed on any grab sample collected at an applicable sampling point for the plant results in mortality for no more than 50 per cent of the test organisms in 100 per cent effluent.

5.  (1)  Subsection 19 (1) of the Regulation is amended by adding “the Table in” before “Schedule 2”.

(2)  Subsection 19 (2) of the Regulation is revoked and the following substituted:

(2)  A discharger for a plant for which there is a Table in Schedule 3 does not need to analyze a set of samples collected at a process effluent sampling point for the parameter if the Table indicates that the discharger does not need to analyze for the parameter.

(3)  Clause 19 (3) (b) of the Regulation is amended by adding “the Table in” before “Schedule 2”.

(4)  Clauses 19 (6) (a) and (b) of the Regulation are amended by adding “the Table in” before “Schedule 2” wherever it appears.

6.  (1)  Subsection 20 (1) of the Regulation is amended by adding “the Table in” before “Schedule 2”.

(2)  Subsection 20 (2) of the Regulation is revoked and the following substituted:

(2)  A discharger for a plant for which there is a Table in Schedule 3 does not need to analyze a set of samples collected at a process effluent sampling point for the parameter if the Table indicates that the discharger does not need to analyze for the parameter.

7.  (1)  Subsection 21 (1) of the Regulation is amended by adding “the Table in” before “Schedule 2”.

(2)  Subsection 21 (2) of the Regulation is revoked and the following substituted:

(2)  A discharger for a plant for which there is a Table in Schedule 3 does not need to analyze a set of samples collected at a process effluent sampling point for the parameter if the Table indicates that the discharger does not need to analyze for the parameter.

8.  Subsection 22 (1) of the Regulation is amended by “the Table in” before “Schedule 4”.

9.  Subsection 23 (1) of the Regulation is amended by adding “the Table in” before “Schedule 5”.

10.  Subsection 24 (1) of the Regulation is amended by adding “the Table in” before “Schedule 2”.

11.  (1)  Subsection 26 (3) of the Regulation is amended by striking out “in Schedule 7 for the discharger’s plant” and substituting “opposite the discharger’s plant in Column 2 of Schedule 7”.

(2)  Subsection 26 (17) of the Regulation is revoked and the following substituted:

(17)  Subsections (2) to (16) apply with necessary modifications to each process effluent batch sampling point listed in Column 2 of Schedule 7 opposite the discharger’s plant and, for the purpose, the reference in subsection (3) to each process effluent sampling point shall be deemed to be a reference to each process effluent batch sampling point and the reference in subsections (4) and (9) to subsection 20 (1) shall be deemed to be a reference to subsection 22 (1).

(3)  Subsection 26 (18) of the Regulation is revoked and the following substituted:

(18)  Subsections (2) to (16) apply with necessary modifications to each merged effluent sampling point listed in Column 2 of Schedule 7 opposite the discharger’s plant and, for the purpose, the reference in subsection (3) to each process effluent sampling point shall be deemed to be a reference to each merged effluent sampling point and the reference in subsections (4) and (9) to subsection 20 (1) shall be deemed to be a reference to subsection 23 (1).

(4)  Subsection 26 (19) of the Regulation is revoked and the following substituted:

(19)  Subsections (2) to (13) apply with necessary modifications to each cooling water effluent sampling point listed in Column 2 of Schedule 7 opposite the discharger’s plant and, for the purpose, the reference in subsection (3) to each process effluent sampling point shall be deemed to be a reference to each cooling water effluent sampling point and the reference in subsections (4) and (9) to subsection 20 (1) shall be deemed to be a reference to section 32.

12.  Subsection 28 (1) of the Regulation is revoked and the following substituted:

(1)  Beginning on February 16, 1998, on one day in each month, on a day on which samples are picked up at the plant under section 23, each discharger shall, if the discharger’s plant is listed in Column 1 of Schedule 8, collect and immediately pick up a grab sample at each sampling point that is listed opposite the discharger’s plant in Column 2 of Schedule 8 and shall perform a rainbow trout acute lethality test on each sample.

13.  Subsection 29 (3) of the Regulation is revoked and the following substituted:

(3)  Beginning on February 16, 1998, on one day in each month, on a day on which samples are picked up at the plant under section 23, each discharger shall, if the discharger’s plant is listed in Column 1 of Schedule 8, collect and immediately pick up a grab sample at each sampling point that is listed opposite the discharger’s plant in Column 2 of Schedule 8 and shall perform a Daphnia magna acute lethality test on each sample.

14.  Subsection 31 (3) of the Regulation is revoked and the following substituted:

(3)  On one day in each semi-annual period, on a day on which samples are picked up at the plant under section 20 or, in a semi-annual period in which no samples are picked up at the plant under section 20, on a day on which samples are picked up at the plant under section 22, each discharger shall, if the discharger’s plant is listed in Column 1 of Schedule 9, collect and immediately pick up a grab sample from each sampling point listed that is listed opposite the discharger’s plant in Column 2 of Schedule 9, and shall perform a seven-day fathead minnow growth inhibition test and a seven-day Ceriodaphnia dubia reproduction inhibition and survivability test on each sample.

15.  Subsection 32 (1) of the Regulation is amended by adding “the Table in” before “Schedule 6”.

16.  Subsection 36 (1.2) of the Regulation is amended by adding “the Table in” before “Schedule 10”.

17.  Subsections 37 (8) and (9) of the Regulation are amended by striking out “in Schedule 11 for the discharger’s plant” wherever it appears and substituting in each case “opposite the discharger’s plant in Column 2 of Schedule 11”.

18.  Subsection 39 (4) of the Regulation is amended by striking out “in Schedule 11 for the discharger’s plant” and substituting “opposite the discharger’s plant in Column 2 of Schedule 11”.

19.  Schedules 2 to 11 to the Regulation are revoked and the following substituted:

SCHEDULE 2
PROCESS EFFLUENT: DESIGNATED SAMPLING POINTS, LIMITS

Table 1
Amoco Canada Petroleum Company Limited (Sarnia)

Designated Process Effluent Sampling Point: 0100, API Separator Effluent to River

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Daily Plant Loading Limit in kilograms per day | Column 4 Monthly Average Plant Loading Limit in kilograms per day |
| --- | --- | --- | --- | --- | --- |
| 1. | 5a | DOC | Daily | 9.0 | 3.0 |
| 2. | 6 | Total phosphorus | Weekly | 1.2 | 0.41 |
| 3. | 8 | Total suspended solids | Daily | 17 | 3.7 |
| 4. | 9 | Aluminum | Quarterly | 1.1 | Not applicable |
| 5. | 9 | Copper | Quarterly | 0.21 | Not applicable |
| 6. | 9 | Molybdenum | Weekly | 0.38 | 0.16 |
| 7. | 9 | Zinc | Quarterly | 0.17 | Not applicable |
| 8. | 10 | Arsenic | Quarterly | 0.011 | Not applicable |
| 9. | 12 | Mercury | Quarterly | 0.0010 | Not applicable |
| 10. | 14 | Phenolics (4AAP) | Weekly | 0.0067 | 0.0023 |
| 11. | 15 | Sulphide | Weekly | 0.12 | 0.044 |
| 12. | 16 | 1,2-Dichlorobenzene | Weekly | 0.16 | 0.056 |
| 13. | 17 | Benzene | Weekly | 0.013 | 0.0051 |
| 14. | 25 | Oil and grease | Weekly | 6.0 | 4.7 |

Table 2
Bayer Inc. (Sarnia)

Designated Process Effluent Sampling Points:

0900, Butyl II Effluent to 66 inch Sewer to River

1300, Butyl holdup Pond to River

1700, Neutralization Sump to River

1800, Biox Plant Effluent to River

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Daily Plant Loading Limit in kilograms per day | Column 4 Monthly Average Plant Loading Limit in kilograms per day |
| --- | --- | --- | --- | --- | --- |
| 1. | 4a | Ammonia plus Ammonium | Weekly | 264 | 92 |
| 2. | 4b | Nitrate + Nitrite | Weekly | 390 | 142 |
| 3. | 4a | Total Kjeldahl nitrogen | Daily | 230 | 67 |
| 4. | 5a | DOC | Daily | 1145 | 447 |
| 5. | 6 | Total phosphorus | Weekly | 69 | 26 |
| 6. | 8 | Total suspended solids | Daily | 1639 | 540 |
| 7. | 9 | Aluminum | Weekly | 170 | 57 |
| 8. | 9 | Cobalt | Weekly | 0.40 | 0.28 |
| 9. | 9 | Zinc | Weekly | 15 | 4.9 |
| 10. | 14 | Phenolics (4AAP) | Weekly | 0.66 | 0.27 |
| 11. | 16 | Bromoform | Weekly | 1.2 | 0.37 |
| 12. | 16 | Bromomethane | Daily | 1.4 | 0.32 |
| 13. | 16 | Chloroform | Weekly | 1.2 | 0.39 |
| 14. | 16 | Chloromethane | Daily | 2.1 | 0.60 |
| 15. | 16 | Methylene chloride | Quarterly | 0.21 | Not applicable |
| 16. | 17 | Benzene | Weekly | 0.39 | 0.14 |
| 17. | 25 | Oil and grease | Weekly | 251 | 126 |

Table 3
Celanese Canada Inc. (Millhaven)

Designated Process Effluent Sampling Point:
0400, Treatment Plant Effluent to Centre Outfall

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Daily Plant Loading Limit in kilograms per day | Column 4 Monthly Average Plant Loading Limit in kilograms per day |
| --- | --- | --- | --- | --- | --- |
| 1. | 4a | Ammonia plus Ammonium | Weekly | 55 | 20 |
| 2. | 4b | Nitrate + Nitrite | Weekly | 110 | 41 |
| 3. | 4a | Total Kjeldahl nitrogen | Daily | 54 | 16 |
| 4. | 5a | DOC | Daily | 57 | 31 |
| 5. | 6 | Total phosphorus | Weekly | 23 | 8.5 |
| 6. | 8 | Total suspended solids | Daily | 130 | 39 |
| 7. | 10 | Antimony | Weekly | 1.5 | 0.53 |
| 8. | 14 | Phenolics (4AAP) | Weekly | 0.032 | 0.0080 |
| 9. | 25 | Oil and grease | Weekly | 42 | 14 |

Table 4
Chinook Group Limited (Sombra)

Designated Process Effluent Sampling Point: 0100, Sump Effluent to River

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Daily Plant Loading Limit in kilograms per day | Column 4 Monthly Average Plant Loading Limit in kilograms per day |
| --- | --- | --- | --- | --- | --- |
| 1. | 4a | Ammonia plus Ammonium | Weekly | 1.5 | 0.48 |
| 2. | 4b | Total Kjeldahl nitrogen | Daily | 3.1 | 1.1 |
| 3. | 5a | DOC | Daily | 8.2 | 3.2 |
| 4. | 6 | Total phosphorus | Weekly | 0.52 | 0.20 |
| 5. | 8 | Total suspended solids | Daily | 7.5 | 2.4 |
| 6. | 9 | Chromium | Weekly | 0.021 | 0.0079 |
| 7. | 9 | Zinc | Weekly | 0.066 | 0.023 |
| 8. | 25 | Oil and grease | Weekly | 0.90 | 0.34 |
| 9. | 24 | 2,3,7,8-tetrachlorodibenzo-para-dioxin | Quarterly | Not applicable | Not applicable |
| 10. | 24 | 2,3,7,8-tetrachlorodibenzofuran | Quarterly | Not applicable | Not applicable |
| 11. | 24 | Total toxic equivalent of 2,3,7,8 substituted dioxin and furan congeners | Quarterly | Not applicable | Not applicable |

Table 5
Dow Chemical Canada Inc. (Sarnia) — Scott Road

Designated Process Effluent Sampling Point: 2100, Scott Road Treated Runoff to River

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Daily Plant Loading Limit in kilograms per day | Column 4 Monthly Average Plant Loading Limit in kilograms per day |
| --- | --- | --- | --- | --- | --- |
| 1. | 4a | Total Kjeldahl Nitrogen | Weekly | 1.7 | 0.62 |
| 2. | 5a | DOC | Daily | 27 | 6.1 |
| 3. | 6 | Total phosphorus | Weekly | 0.14 | 0.053 |
| 4. | 8 | Total suspended solids | Weekly | 2.6 | 1.3 |
| 5. | 9 | Aluminum | Quarterly | 0.022 | Not applicable |
| 6. | 9 | Boron | Quarterly | 1.7 | Not applicable |
| 7. | 12 | Mercury | Quarterly | 0.000042 | Not applicable |
| 8. | 14 | Phenolics (4AAP) | Weekly | 0.0062 | 0.0023 |
| 9. | 15 | Sulphide | Quarterly | 0.010 | Not applicable |
| 10. | 16 | 1,1,2-Trichloroethane | Weekly | 0.021 | 0.0065 |
| 11. | 16 | 1,1-Dichloroethane | Weekly | 0.047 | 0.015 |
| 12. | 23 | 1,2,4-Trichlorobenzene | Weekly | 0.0030 | 0.0011 |
| 13. | 23 | 2,4,5-Trichlorotoluene | Weekly | 0.00043 | 0.00020 |
| 14. | 23 | Hexachlorobenzene | Quarterly | 0.00010 | Not applicable |
| 15. | 23 | Hexachlorobutadiene | Weekly | 0.00028 | 0.00020 |
| 16. | 23 | Hexachloroethane | Weekly | 0.00028 | 0.00020 |
| 17. | 23 | Octachlorostyrene | Weekly | 0.00021 | 0.00020 |
| 18. | 25 | Oil and grease | Weekly | 1.8 | 1.4 |
| 19. | 24 | 2,3,7,8-tetrachlorodibenzo-para-dioxin | Quarterly | Not applicable | Not applicable |
| 20. | 24 | 2,3,7,8-tetrachlorodibenzofuran | Quarterly | Not applicable | Not applicable |
| 21. | 24 | Total toxic equivalent of 2,3,7,8 substituted dioxin and furan congeners | Quarterly | Not applicable | Not applicable |

Table 6
Dow Chemical Canada Inc. (Sarnia) — Vidal Street

Designated Process Effluent Sampling Points:

1900, Biox Plant Effluent flowing into 4th Street Outfall to St. Clair River

2000, Boiler Feedwater Effluent flowing into 4th Street Outfall to St. Clair River

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Daily Plant Loading Limit in kilograms per day | Column 4 Monthly Average Plant Loading Limit in kilograms per day |
| --- | --- | --- | --- | --- | --- |
| 1. | 4a | Ammonia plus Ammonium | Weekly | 140 | 43 |
| 2. | 4a | Total Kjeldahl nitrogen | Daily | 78 | 22 |
| 3. | 5a | DOC | Daily | 190 | 73 |
| 4. | 6 | Total phosphorus | Weekly | 21 | 7.0 |
| 5. | 8 | Total suspended solids | Daily | 640 | 230 |
| 6. | 9 | Aluminum | Weekly | 25 | 8.9 |
| 7. | 14 | Phenolics (4AAP) | Weekly | 0.94 | 0.30 |
| 8. | 16 | 1,2-Dichloroethane | Weekly | 0.51 | 0.18 |
| 9. | 16 | 1,2-Dichloropropane | Weekly | 1.9 | 0.62 |
| 10. | 16 | Methylene chloride | Quarterly | 0.089 | Not applicable |
| 11. | 16 | Tetrachloroethylene | Weekly | 0.11 | 0.041 |
| 12. | 16 | Trichloroethylene | Quarterly | 0.055 | Not applicable |
| 13. | 16 | Vinyl chloride | Quarterly | 0.58 | Not applicable |
| 14. | 17 | Ethylbenzene | Quarterly | 0.055 | Not applicable |
| 15. | 19 | Bis (2-chloroisopropyl)ether | Quarterly | 0.076 | Not applicable |
| 16. | 23 | 1,2,4-Trichlorobenzene | Weekly | 0.017 | 0.0056 |
| 17. | 23 | 1,2,4,5-Tetrachlorobenzene | Weekly | 0.0020 | 0.00075 |
| 18. | 23 | 2,4,5-Trichlorotoluene | Weekly | 0.0037 | 0.0016 |
| 19. | 23 | Hexachlorobenzene | Quarterly | 0.0030 | Not applicable |
| 20. | 23 | Hexachloroethane | Weekly | 0.00052 | 0.00033 |
| 21. | 25 | Oil and grease | Weekly | 180 | 76 |
| 22. | 24 | 2,3,7,8-tetrachlorodibenzo-para-dioxin | Quarterly | Not applicable | Not applicable |
| 23. | 24 | 2,3,7,8-tetrachlorodibenzofuran | Quarterly | Not applicable | Not applicable |
| 24. | 24 | Total toxic equivalent of 2,3,7,8 substituted dioxin and furan congeners | Quarterly | Not applicable | Not applicable |

Table 7
Dupont Canada Inc. (Kingston)

Designated Process Effluent Sampling Point: 0600, Flake Effluent to Catch Tank

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Daily Plant Loading Limit in kilograms per day | Column 4 Monthly Average Plant Loading Limit in kilograms per day |
| --- | --- | --- | --- | --- | --- |
| 1. | 5a | DOC | Daily | 11 | 7.6 |
| 2. | 6 | Total phosphorus | Weekly | 1.3 | 0.42 |
| 3. | 8 | Total suspended solids | Daily | 43 | 13 |
| 4. | 12 | Mercury | Quarterly | 0.00036 | Not applicable |
| 5. | 14 | Phenolics (4AAP) | Weekly | 0.0072 | 0.0035 |
| 6. | 19 | Biphenyl | Quarterly | 0.014 | Not applicable |
| 7. | 19 | Diphenyl ether | Weekly | 0.071 | 0.029 |
| 8. | 25 | Oil and grease | Weekly | 7.5 | 2.9 |

Table 8
Dupont Canada Inc. (Maitland)

Designated Process Effluent Sampling Points:

0300, Wastewater Treatment Plant Effluent to Main Effluent Stream

0400, CFH Effluent to Cribbed Ditch

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Daily Plant Loading Limit in kilograms per day | Column 4 Monthly Average Plant Loading Limit in kilograms per day |
| --- | --- | --- | --- | --- | --- |
| 1. | 2 | Cyanide Total | Weekly | 5.5 | 2.1 |
| 2. | 4a | Ammonia plus Ammonium | Weekly | 140 | 48 |
| 3. | 4b | Nitrate + Nitrite | Weekly | 110 | 40 |
| 4. | 4a | Total Kjeldahl nitrogen | Daily | 320 | 93 |
| 5. | 5a | DOC | Daily | 710 | 360 |
| 6. | 6 | Total phosphorus | Weekly | 21 | 8.0 |
| 7. | 8 | Total suspended solids | Daily | 650 | 210 |
| 8. | 9 | Chromium | Weekly | 1.1 | 0.46 |
| 9. | 9 | Cobalt | Weekly | 0.71 | 0.23 |
| 10. | 9 | Copper | Weekly | 0.28 | 0.098 |
| 11. | 9 | Vanadium | Weekly | 0.58 | 0.29 |
| 12. | 10 | Arsenic | Weekly | 0.89 | 0.19 |
| 13. | 14 | Phenolics (4AAP) | Weekly | 0.29 | 0.14 |
| 14. | 16 | Tetrachloroethylene | Daily | 1.0 | 0.61 |
| 15. | 23 | Hexachlorobenzene | Quarterly | 0.0053 | Not applicable |
| 16. | 25 | Oil and grease | Weekly | 120 | 46 |
| 17. | 24 | 2,3,7,8-tetrachlorodibenzo-para-dioxin | Quarterly | Not applicable | Not applicable |
| 18. | 24 | 2,3,7,8-tetrachlorodibenzofuran | Quarterly | Not applicable | Not applicable |
| 19. | 24 | Total toxic equivalent of 2,3,7,8 substituted dioxin and furan congeners | Quarterly | Not applicable | Not applicable |

Table 9
Ethyl Canada Inc. (Sarnia)

Designated Process Effluent Sampling Point: 0200, Speciality Chemicals Effluent

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Daily Plant Loading Limit in kilograms per day | Column 4 Monthly Average Plant Loading Limit in kilograms per day |
| --- | --- | --- | --- | --- | --- |
| 1. | 4b | Nitrate + Nitrite | Weekly | 0.30 | 0.15 |
| 2. | 5a | DOC | Daily | 0.30 | 0.087 |
| 3. | 6 | Total phosphorus | Quarterly | 0.0050 | Not applicable |
| 4. | 8 | Total suspended solids | Daily | 0.46 | 0.10 |
| 5. | 9 | Aluminum | Daily | 0.35 | 0.17 |
| 6. | 9 | Chromium | Quarterly | 0.0043 | Not applicable |
| 7. | 9 | Lead | Daily | 0.018 | 0.0035 |
| 8. | 12 | Mercury | Quarterly | 0.000017 | Not applicable |
| 9. | 14 | Phenolics (4AAP) | Weekly | 0.00023 | 0.000078 |
| 10. | 16 | 1,2-Dichloroethane | Weekly | 0.0071 | 0.0028 |
| 11. | 16 | Ethylene dibromide | Quarterly | 0.0021 | Not applicable |
| 12. | 17 | Toluene | Weekly | 0.00047 | 0.00010 |
| 13. | 25 | Oil and grease | Weekly | 0.17 | 0.13 |

Table 10
GE Plastics Canada (Cobourg)

Designated Process Effluent Sampling Point: 0100, Final Filter Effluent to Lake

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Daily Plant Loading Limit in kilograms per day | Column 4 Monthly Average Plant Loading Limit in kilograms per day |
| --- | --- | --- | --- | --- | --- |
| 1. | 2 | Cyanide Total | Quarterly | 0.024 | Not applicable |
| 2. | 4a | Ammonia plus Ammonium | Weekly | 1.2 | 0.60 |
| 3. | 4b | Nitrate + Nitrite | Weekly | 5.5 | 2.2 |
| 4. | 4a | Total Kjeldahl nitrogen | Daily | 4.0 | 1.2 |
| 5. | 5a | DOC | Daily | 14 | 6.9 |
| 6. | 6 | Total phosphorus | Weekly | 0.48 | 0.24 |
| 7. | 8 | Total suspended solids | Daily | 24 | 12 |
| 8. | 9 | Aluminum | Quarterly | 0.74 | Not applicable |
| 9. | 10 | Antimony | Quarterly | 0.21 | Not applicable |
| 10. | 14 | Phenolics (4AAP) | Weekly | 0.055 | 0.022 |
| 11. | 25 | Oil and grease | Weekly | 28.2 | 9.9 |

Table 11
Geon Canada Inc. (Niagara Falls)

Designated Process Effluent Sampling Point: 0100, Final Effluent to River

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Daily Plant Loading Limit in kilograms per day | Column 4 Monthly Average Plant Loading Limit in kilograms per day |
| --- | --- | --- | --- | --- | --- |
| 1. | 2 | Cyanide Total | Quarterly | 0.55 | Not applicable |
| 2. | 4a | Ammonia plus Ammonium | Weekly | 21 | 12 |
| 3. | 4a | Total Kjeldahl nitrogen | Daily | 38 | 24 |
| 4. | 5a | DOC | Daily | 194 | 126 |
| 5. | 6 | Total phosphorus | Weekly | 4 | 2.9 |
| 6. | 8 | Total suspended solids | Daily | 91 | 50 |
| 7. | 9 | Aluminum | Quarterly | 9.5 | Not applicable |
| 8. | 14 | Phenolics (4AAP) | Weekly | 0.32 | 0.10 |
| 9. | 16 | Vinyl chloride | Weekly | 0.24 | 0.091 |
| 10. | 25 | Oil and grease | Weekly | 28 | 16 |
| 11. | 24 | 2,3,7,8-tetrachlorodibenzo-para-dioxin | Quarterly | Not applicable | Not applicable |
| 12. | 24 | 2,3,7,8-tetrachlorodibenzofuran | Quarterly | Not applicable | Not applicable |
| 13. | 24 | Total toxic equivalent of 2,3,7,8 substituted dioxin and furan congeners | Quarterly | Not applicable | Not applicable |

Table 12
Imperial Oil Chemicals Division (Sarnia)

Designated Process Effluent Sampling Point:

0200, Carbon Contactor

0300. Final Effluent to River

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Daily Plant Loading Limit in kilograms per day | Column 4 Monthly Average Plant Loading Limit in kilograms per day |
| --- | --- | --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly | 117 | 40 |
| 2. | 8 | Total suspended solids | Weekly | 187 | 60.7 |
| 3. | 16 | Vinyl chloride | Daily | 11 | 2.4 |
| 4. | 17 | Benzene | Weekly | 0.28 | 0.09 |
| 5. | 23 | Hexachlorobutadiene | Quarterly | 0.004 | Not applicable |
| 6. | 25 | Oil and grease | Weekly | 17.7 | 8 |

Table 13
Nova Chemicals Canada Ltd (Corunna)

Designated Process Effluent Sampling Point: 0200, Final Effluent to River

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Daily Plant Loading Limit in kilograms per day | Column 4 Monthly Average Plant Loading Limit in kilograms per day |
| --- | --- | --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly | 440 | 220 |
| 2. | 6 | Total phosphorus | Quarterly | 22 | Not applicable |
| 3. | 8 | Total suspended solids | Weekly | 1300 | 570 |
| 4. | 9 | Aluminum | Quarterly | 45 | Not applicable |
| 5. | 14 | Phenolics (4AAP) | Weekly | 0.75 | 0.34 |
| 6. | 17 | Toluene | Quarterly | 1.3 | Not applicable |
| 7. | 25 | Oil and grease | Weekly | 460 | 170 |

Table 14
Nova Chemicals Canada Ltd (Mooretown)

Designated Process Effluent Sampling Point: 0100, Final Effluent to River

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Daily Plant Loading Limit in kilograms per day | Column 4 Monthly Average Plant Loading Limit in kilograms per day |
| --- | --- | --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly | 19 | 11 |
| 2. | 6 | Total phosphorus | Weekly | 1.9 | 1.1 |
| 3. | 8 | Total suspended solids | Daily | 98 | 34 |
| 4. | 9 | Aluminum | Quarterly | 2.6 | Not applicable |
| 5. | 9 | Zinc | Quarterly | 0.28 | Not applicable |
| 6. | 14 | Phenolics (4AAP) | Weekly | 0.015 | 0.0070 |
| 7. | 25 | Oil and grease | Weekly | 6.6 | 3.3 |

Table 15
RohMax Canada (Morrisburg)

Designated Process Effluent Sampling Point: 0100, Final Outfall to River

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Daily Plant Loading Limit in kilograms per day | Column 4 Monthly Average Plant Loading Limit in kilograms per day |
| --- | --- | --- | --- | --- | --- |
| 1. | 5a | DOC | Daily | 16 | 5.0 |
| 2. | 6 | Total Phosphorus | Quarterly | 0.65 | Not applicable |
| 3. | 8 | Total suspended solids | Weekly | 14 | 6.5 |
| 4. | 14 | Phenolics (4AAP) | Weekly | 0.0056 | 0.0026 |
| 5. | 17 | Toluene | Weekly | 0.035 | 0.013 |
| 6. | 25 | Oil and grease | Weekly | 4.8 | 2.1 |

Table 16
Stepan Canada Inc. (Longford Mills)

Designated Process Effluent Sampling Point: 0200, Clarifier Effluent

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Daily Plant Loading Limit in kilograms per day | Column 4 Monthly Average Plant Loading Limit in kilograms per day |
| --- | --- | --- | --- | --- | --- |
| 1. | 4a | Ammonia plus Ammonium | Weekly | 1.2 | 0.42 |
| 2. | 4a | Total Kjeldahl nitrogen | Daily | 1.5 | 0.46 |
| 3. | 5a | DOC | Daily | 6.2 | 2.8 |
| 4. | 6 | Total phosphorus | Weekly | 0.16 | 0.061 |
| 5. | 8 | Total suspended solids | Daily | 6.3 | 2.1 |
| 6. | 9 | Aluminum | Weekly | 0.68 | 0.24 |
| 7. | 14 | Phenolics (4AAP) | Weekly | 0.0073 | 0.0028 |
| 8. | 16 | Carbon tetrachloride | Quarterly | 0.0020 | Not applicable |
| 9. | 16 | Chloroform | Quarterly | 0.0054 | Not applicable |
| 10. | 25 | Oil and grease | Weekly | 2.2 | 0.94 |
| 11. | 27 | PCBT | Quarterly | 0.040 | Not applicable |

SCHEDULE 3 
ANALYTICAL REQUIREMENTS AT PLANTS WITH MORE THAN ONE PROCESS EFFLUENT SAMPLING POINT

Table 1
Bayer Inc. (Sarnia)

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Whether the sample collected at sampling point 900 needs to be analyzed for the parameter in Column 1 | Column 3 Whether the sample collected at sampling point 1300 needs to be analyzed for the parameter in Column 1 | Column 4 Whether the sample collected at sampling point 1700 needs to be analyzed for the parameter in Column 1 | Column 5 Whether the sample collected at sampling point 1800 needs to be analyzed for the parameter in Column 1 |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | 4a | Ammonia plus Ammonium | No | No | No | Yes |
| 2. | 4b | Nitrate + Nitrite | No | No | No | Yes |
| 3. | 4a | Total Kjeldahl nitrogen | No | No | Yes | Yes |
| 4. | 5a | DOC | Yes | Yes | Yes | Yes |
| 5. | 6 | Total phosphorus | Yes | Yes | Yes | Yes |
| 6. | 8 | Total suspended solids | Yes | Yes | Yes | Yes |
| 7. | 9 | Aluminum | Yes | Yes | Yes | Yes |
| 8. | 9 | Cobalt | No | No | No | Yes |
| 9. | 9 | Zinc | Yes | Yes | Yes | Yes |
| 10. | 14 | Phenolics (4AAP) | Yes | Yes | Yes | Yes |
| 11. | 16 | Bromoform | No | No | No | Yes |
| 12. | 16 | Bromomethane | Yes | No | No | Yes |
| 13. | 16 | Chloroform | Yes | No | No | Yes |
| 14. | 16 | Chloromethane | Yes | Yes | No | Yes |
| 15. | 16 | Methylene chloride | Yes | No | No | Yes |
| 16. | 17 | Benzene | Yes | No | No | Yes |
| 17. | 25 | Oil and Grease | Yes | Yes | Yes | Yes |

Table 2 
Dow Chemical Canada Inc. (Sarnia) — Vidal Street

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Whether the sample collected at sampling point 1900 needs to be analyzed for the parameter in Column 1 | Column 3 Whether the sample collected at sampling point 2000 needs to be analyzed for the parameter in Column 1 |
| --- | --- | --- | --- | --- |
| 1. | 4a | Ammonia plus Ammonium | Yes | No |
| 2. | 4a | Total Kjeldahl nitrogen | Yes | No |
| 3. | 5a | DOC | Yes | Yes |
| 4. | 6 | Total phosphorus | Yes | Yes |
| 5. | 8 | Total suspended solids | Yes | Yes |
| 6. | 9 | Aluminum | Yes | Yes |
| 7. | 14 | Phenolics (4AAP) | Yes | Yes |
| 8. | 16 | 1,2-Dichloroethane | Yes | No |
| 9. | 16 | 1,2-Dichloropropane | Yes | No |
| 10. | 16 | Methylene chloride | Yes | No |
| 11. | 16 | Tetrachloroethylene | Yes | No |
| 12. | 16 | Trichloroethylene | Yes | No |
| 13. | 16 | Vinyl chloride | Yes | No |
| 14. | 17 | Ethylbenzene | Yes | No |
| 15. | 19 | Bis (2-chloroisopropyl)ether | Yes | No |
| 16. | 23 | 1,2,4-Trichlorobenzene | Yes | No |
| 17. | 23 | 1,2,4,5-Tetrachlorobenzene | Yes | No |
| 18. | 23 | 2,4,5-Trichlorotoluene | Yes | No |
| 19. | 23 | Hexachlorobenzene | Yes | No |
| 20. | 23 | Hexachloroethane | Yes | No |
| 21. | 25 | Oil and Grease | Yes | Yes |
| 22. | 24 | 2,3,7,8-tetrachlorodibenzo-para-dioxin | Yes | Yes |
| 23. | 24 | 2,3,7,8-tetrachlorodibenzofuran | Yes | Yes |
| 24. | 24 | Total toxic equivalent of 2,3,7,8 substituted dioxin and furan congeners | Yes | Yes |

Table 3
Dupont Canada Inc. (Maitland)

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Whether the sample collected at sampling point 0300 needs to be analyzed for the parameter in Column 1 | Column 3 Whether the sample collected at sampling point 0400 needs to be analyzed for the parameter in Column 1 |
| --- | --- | --- | --- | --- |
| 1. | 2 | Cyanide Total | Yes | No |
| 2. | 4a | Ammonia plus Ammonium | Yes | No |
| 3. | 4b | Nitrate + Nitrite | Yes | No |
| 4. | 4a | Total Kjeldahl nitrogen | Yes | No |
| 5. | 5a | DOC | Yes | Yes |
| 6. | 6 | Total phosphorus | Yes | Yes |
| 7. | 8 | Total suspended solids | Yes | Yes |
| 8. | 9 | Chromium | Yes | No |
| 9. | 9 | Cobalt | Yes | No |
| 10. | 9 | Copper | Yes | No |
| 11. | 9 | Vanadium | Yes | No |
| 12. | 10 | Arsenic | No | Yes |
| 13. | 14 | Phenolics (4AAP) | Yes | Yes |
| 14. | 16 | Tetrachloroethylene | No | Yes |
| 15. | 23 | Hexachlorobenzene | No | Yes |
| 16. | 25 | Oil and grease | Yes | Yes |
| 17. | 24 | 2,3,7,8-tetrachlorodibenzo-para-dioxin | Yes | No |
| 18. | 24 | 2,3,7,8-tetrachlorodibenzofuran | Yes | No |
| 19. | 24 | Total toxic equivalent of 2,3,7,8 substituted dioxin and furan congeners | Yes | No |

Table 4
Imperial Oil Chemicals Division (Sarnia)

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Whether the sample collected at sampling point 200 needs to be analyzed for the parameter in Column 1 | Column 3 Whether the sample collected at sampling point 300 needs to be analyzed for the parameter in Column 1 |
| --- | --- | --- | --- | --- |
| 1. | 5a | DOC | Yes | No |
| 2. | 8 | Total suspended solids | Yes | No |
| 3. | 16 | Vinyl chloride | No | Yes |
| 4. | 17 | Benzene | Yes | No |
| 5. | 23 | Hexachlorobutadiene | Yes | No |
| 6. | 25 | Oil and Grease | Yes | No |

SCHEDULE 4 
PROCESS EFFLUENT BATCH: DESIGNATED SAMPLING POINTS, LIMITS, MONITORING FREQUENCY

Table 1
Dow Chemical Canada Inc. (Sarnia) — La Salle Road

Designated Process Effluent Batch Sampling Points: 2300, La Salle Road Site - East Pond to Talfourd Creek

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Batch Loading Limit in kilograms per batch |
| --- | --- | --- | --- | --- |
| 1. | 5a | DOC | Once for each batch | 130 |
| 2. | 6 | Total phosphorus | Once for each batch | 3.8 |
| 3. | 8 | Total suspended solids | Once for each batch | 160 |
| 4. | 14 | Phenolics (4AAP) | Once for each batch | 0.090 |
| 5. | 25 | Oil and grease | Once for each batch | 12 |

Table 2 
Ethyl Canada Inc. (Sarnia)

Designated Process Effluent Batch Sampling Point: 0300, TEL Unit Effluent

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency | Column 3 Batch Loading Limit in kilograms per batch |
| --- | --- | --- | --- | --- |
| 1. | 5a | DOC | Once for each batch | 7.3 |
| 2. | 6 | Total phosphorus | Once for each batch | 0.28 |
| 3. | 8 | Total suspended solids | Once for each batch | 11 |
| 4. | 9 | Aluminum | Once for each batch | 7.4 |
| 5. | 9 | Boron | Once for each batch | 4.68 |
| 6. | 9 | Lead | Once for each batch | 1.60 |
| 7. | 9 | Zinc | Once for each batch | 0.18 |
| 8. | 12 | Mercury | Once for each batch | 0.0013 |
| 9. | 13 | Total Alkyl Lead | Once for each batch | 1.1 |
| 10. | 14 | Phenolics (4AAP) | Once for each batch | 0.015 |
| 11. | 15 | Sulphide | Once for each batch | 0.15 |
| 12. | 16 | 1,2-Dichloroethane | Once for each batch | 0.21 |
| 13. | 16 | Ethylene dibromide | Once for each batch | 0.11 |
| 14. | 16 | Methylene chloride | Once for each batch | 0.030 |
| 15. | 17 | Toluene | Once for each batch | 0.0046 |
| 16. | 19 | 1-Methylnaphthalene | Once for each batch | 0.027 |
| 17. | 19 | 2-Methylnaphthalene | Once for each batch | 0.026 |
| 18. | 19 | Acenaphthene | Once for each batch | 0.0090 |
| 19. | 19 | Biphenyl | Once for each batch | 0.0085 |
| 20. | 19 | Fluorene | Once for each batch | 0.0081 |
| 21. | 19 | Indole | Once for each batch | 0.67 |
| 22. | 19 | Naphthalene | Once for each batch | 0.013 |
| 23. | 19 | Phenanthrene | Once for each batch | 0.0064 |
| 24. | 25 | Oil and grease | Once for each batch | 8.2 |

SCHEDULE 5
MERGED EFFLUENT: DESIGNATED SAMPLING POINTS, MONITORING FREQUENCY

Table 1
Bayer Inc. (Sarnia)

Designated Merged Effluent Sampling Point: 0400, 66 inch Sewer to River

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 8 | Total suspended solids | Weekly |

Table 2
Celanese Canada Inc. (Millhaven)

Designated Merged Effluent Sampling Point: 0100, Centre Outfall to Lake

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 8 | Total suspended solids | Weekly |

Table 3
Dow Chemical Canada Inc. (Sarnia) — Vidal Street

Designated Merged Effluent Sampling Point: 0900, 4th Street Outfall to River

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 8 | Total suspended solids | Weekly |
| 3. | 14 | Phenolics (4AAP) | Weekly |

Table 4
Dupont Canada Inc. (Kingston)

Designated Merged Effluent Sampling Point: 1100, Catch Tank Effluent to Lake

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 8 | Total suspended solids | Weekly |
| 3. | 19 | Diphenyl ether | Weekly |

Table 5
Dupont Canada Inc. (Maitland)

Designated Merged Effluent Sampling Point: 1100, Site Effluent to River

| Item | Analytical Test Group | Column1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 8 | Total suspended solids | Weekly |
| 3. | 14 | Phenolics (4AAP) | Weekly |
| 4. | 25 | Oil and Grease | Weekly |

Table 6
Ethyl Canada Inc. (Sarnia)

Designated Merged Effluent Sampling Point: 0100, Final Effluent to River

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 8 | Total suspended solids | Weekly |
| 3. | 14 | Phenolics (4AAP) | Weekly |
| 4. | 25 | Oil and Grease | Weekly |

Table 7
Imperial Oil Chemicals Division (Sarnia)

Designated Merged Effluent Sampling Point: 0300, Final Effluent to River

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 8 | Total suspended solids | Weekly |
| 3. | 14 | Phenolics | Weekly |
| 4. | 17 | Benzene | Weekly |
| 5. | 17 | Toluene | Weekly |
| 6. | 23 | Hexachlorobutadiene | Weekly |
| 7. | 25 | Oil and Grease | Weekly |

SCHEDULE 6 
COOLING WATER EFFLUENT: DESIGNATED SAMPLING POINTS, MONITORING FREQUENCY

Table 1
BASF Canada Inc. (Arnprior)

Designated Cooling Water Effluent Sampling Point: 0100, Storm Discharge to River

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 6 | Total phosphorus | Weekly |
| 3. | 8 | Total suspended solids | Weekly |
| 4. | 14 | Phenolics (4AAP) | Weekly |

Table 2 
Bayer Inc. (Sarnia)

Designated Cooling Water Effluent Sampling Points:

0200, 72 inch Sewer to River

0500, 54 inch Sewer to River

1400, Turbine Cooling Water to River

2100, NBR Cooling Water to Cole Drain

2200, BE-2 Cooling Water to Cole Drain

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 8 | Total suspended solids | Weekly |

Table 3 
Celanese Canada Inc. (Millhaven)

Designated Cooling Water Effluent Sampling Point: 0200, West Outfall to Lake

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 8 | Total suspended solids | Weekly |

Table 4 
Dow Chemical Canada Inc. (Sarnia) — Vidal Street

Designated Cooling Water Effluent Sampling Points:

0600, 2nd Street Outfall to River

0700, 3rd Street Outfall to River

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 8 | Total suspended solids | Weekly |
| 3. | 14 | Phenolics (4AAP) | Weekly |

Table 5 
Dupont Canada Inc. (Kingston)

Designated Cooling Water Effluent Sampling Point: 0700, Service Sewer to Lake

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 8 | Total suspended solids | Weekly |
| 3. | 19 | Diphenyl ether | Weekly |

Table 6 
Dupont Canada Inc. (Whitby)

Designated Effluent Sampling Point: 1000, Cooling Water Ditch at Pellet Pond

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 8 | Total suspended solids | Weekly |

Table 7 
GE Plastics Canada (Cobourg)

Designated Cooling Water Effluent Sampling Point: 0400, Cooling Water to Lake

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 8 | Total suspended solids | Weekly |
| 3. | 14 | Phenolics (4AAP) | Weekly |
| 4. | 25 | Oil and Grease | Weekly |

Table 8 
Goodyear Canada Inc. (Bowmanville)

Designated Cooling Water Effluent Sampling Points: 0200 Final Outfall

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 8 | Total suspended solids | Weekly |

Table 9 
Morbern Inc. (Cornwall)

Designated Cooling Water Effluent Sampling Point: 0100, East Cooling Water to River

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 6 | Total phosphorus | Weekly |
| 3. | 8 | Total suspended solids | Weekly |
| 4. | 9 | Aluminum | Weekly |
| 5. | 9 | Zinc | Weekly |
| 6. | 14 | Phenolics (4AAP) | Weekly |
| 7. | 25 | Oil and grease | Weekly |

Table 10 
Stepan Canada Inc. (Longford Mills)

Designated Cooling Water Effluent Sampling Point: 0100, Cooling Water to Lake

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 6 | Total phosphorus | Weekly |
| 3. | 8 | Total suspended solids | Weekly |

Table 11 
Uniroyal Chemical Ltd. (Elmira)

Designated Cooling Water Effluent Sampling Points:

0200, Outfall #2 to Canagagigue Creek

0800, Shirt Factory Creek from Site

| Item | Analytical Test Group | Column 1 Parameter | Column 2 Monitoring Frequency |
| --- | --- | --- | --- |
| 1. | 5a | DOC | Weekly |
| 2. | 8 | Total suspended solids | Weekly |
| 3. | 14 | Phenolics (4AAP) | Weekly |
| 4. | 25 | Oil and Grease | Weekly |

SCHEDULE 7
ACUTE LETHALITY: SAMPLING POINTS

| Column 1 Plant Name | Column 2 Sampling Point — Number and Description |
| --- | --- |
| Amoco Canada Petroleum Company Limited (Sarnia) | 0100 , API Separator Effluent to River (Process Effluent) |
| BASF Canada Inc. (Arnprior) | 0100 , Storm Discharge to River (Cooling Water) |
| Bayer Inc. (Sarnia) | 0200 , 72 inch Sewer to River (Cooling Water) 0400 , 66 inch Sewer to River (Merged Effluent) 0500 , 54 inch Sewer to River (Cooling Water) 1300 , Butyl Holdup Pond to River (Process Effluent) 1400 , Turbine Cooling Water to River (Cooling Water) 1700 , Neutralization Sump to River (Process Effluent) 1800 , Biox Plant Effluent to River (Process Effluent) 2100 , NBR Cooling Water to Cole Drain (Cooling Water) 2200 , BE-2 Cooling Water to Cole Drain (Cooling Water) |
| Celanese Canada Inc. (Millhaven) | 0100 , Centre Outfall to Lake (Merged Effluent) 0200 , West Outfall to Lake (Cooling Water) |
| Chinook Group Limited (Sombra) | 0100 , Sump Effluent to River (Process Effluent) |
| Dow Chemical Canada Inc. (Sarnia) — Vidal Street | 0600 , 2nd Street Outfall to River (Cooling Water) 0700 , 3rd Street Outfall to River (Cooling Water) 0900 , 4th Street Outfall to River (Merged Effluent) |
| Dow Chemical Canada Inc. (Sarnia) — Scott Road | 2100 , Scott Road Treated Runoff to River (Process Effluent) |
| Dow Chemical Canada Inc. (Sarnia) — La Salle Road | 2300 , La Salle Road Site - East Pond to Talfourd Creek (Process Effluent Batch) |
| Dupont Canada Inc. (Kingston) | 0700 , Service Sewer to Lake (Cooling Water) 1100 , Catch Tank Effluent to Lake (Merged Effluent) |
| Dupont Canada Inc. (Maitland) | 1100 , Site Effluent to River (Merged Effluent) |
| Dupont Canada Inc. (Whitby) | 1000 , Cooling Water Ditch at Pellet Pond (Cooling Water) |
| Ethyl Canada Inc. (Sarnia) | 0100 , Final Effluent to River (Merged Effluent) |
| GE Plastics Canada (Cobourg) | 0100 , Clarifier Effluent to Lake (Process Effluent) 0400 , Cooling Water to Lake (Cooling Water) |
| Geon Canada Inc. (Niagara Falls) | 0100 , Final Effluent to River (Process Effluent) |
| Goodyear Canada Inc. (Bowmanville) | 0200 , Final Outfall (Cooling Water) |
| Imperial Oil Chemicals Division (Sarnia) | 0300 , Final Effluent to River (Merged Effluent) |
| Morbern Inc. (Cornwall) | 0100 , East Cooling Water to River (Cooling Water) |
| Novacor Chemicals Canada Ltd. (Corunna) | 0200 , Final Effluent to River (Process Effluent) |
| Novacor Chemicals Canada Ltd. (Mooretown) | 0100 , Final Effluent to River (Process Effluent) |
| RohMax Canada (Morrisburg) | 0100 , Final Outfall to River (Process Effluent) |
| Stepan Canada Inc. (Longford Mills) | 0100 , Cooling Water to Lake (Cooling Water) 0200 , Clarifier Effluent (Process Effluent) |
| Uniroyal Chemical Ltd. (Elmira) | 0200 , Outfall #2 to Canagagigue Creek (Cooling Water) 0800 , Shirt Factory Creek from Site (Cooling Water) |

SCHEDULE 8
ACUTE LETHALITY TESTING: SAMPLING POINTS

| Column 1 Plant Name | Column 2 Sampling Point — Number and Description |
| --- | --- |
| Bayer Inc. (Sarnia) | 0900 , Butyl II Effluent to 66 inch Sewer |
| Celanese Canada Inc. (Millhaven) | 0400 , Treatment Plant Effluent to Centre Outfall |
| Dow Chemical Canada Inc. (Sarnia) — Vidal Street | 1900 , Biox Plant Effluent to 4th Street Outfall |
| Dupont Canada Inc. (Kingston) | 0600 , Flake Effluent to Catch Tank |
| Dupont Canada Inc. (Maitland) | 0300 , Wastewater Treatment Plant Effluent to Main Effluent Stream 0400 , CFH Effluent to Cribbed Ditch |
| Ethyl Canada Inc. (Sarnia) | 0200 , Specialty Chemicals Effluent 0300 , TEL Unit Effluent |
| Imperial Oil Chemicals Division (Sarnia) | 0200 , Carbon Contactor |

SCHEDULE 9
CHRONIC TOXICITY TESTING: SAMPLING POINTS

| Column 1 Plant Name | Column 2 Sampling Point — Number and Description |
| --- | --- |
| Amoco Canada Petroleum Company Limited (Sarnia) | 0100 , API Separator Effluent to River (Process Effluent) |
| Bayer Inc. (Sarnia) | 0400 , 66 inch Sewer to River (Merged Effluent) 1300 , Butyl Holdup Pond to River (Process Effluent) 1700 , Neutralization Sump to River (Process Effluent) 1800 , Biox Plant Effluent to River (Process Effluent) |
| Celanese Canada Inc. (Millhaven) | 0100 , Centre Outfall to Lake (Merged Effluent) |
| Chinook Group Limited (Sombra) | 0100 , Sump Effluent to River (Process Effluent) |
| Dow Chemical Canada Inc. (Sarnia) — Vidal Street | 0900 , 4th Street Outfall to River (Merged Effluent) |
| Dow Chemical Canada Inc. (Sarnia) — Scott Road | 2100 , Scott Road Treated Runoff to River (Process Effluent) |
| Dow Chemical Canada Inc. (Sarnia) — La Salle Road | 2300 , La Salle Road Site - East Pond to Talfourd Creek (Process Effluent Batch) |
| Dupont Canada Inc. (Kingston) | 1100 , Catch Tank Effluent to Lake (Merged Effluent) |
| Dupont Canada Inc. (Maitland) | 1100 , Site Effluent to River (Merged Effluent) |
| Ethyl Canada Inc. (Sarnia) | 0100 , Final Effluent to River (Merged Effluent) |
| GE Plastics Canada (Cobourg) | 0100 , Clarifier Effluent to Lake (Process Effluent) |
| Geon Canada Inc. (Niagara Falls) | 0100 , Final Effluent to River (Process Effluent) |
| Imperial Oil Chemicals Division (Sarnia) | 0300 , Final Effluent to River (Process Effluent) |
| Nova Chemicals Canada Ltd. (Corunna) | 0200 , Final Effluent to River (Process Effluent) |
| Nova Chemicals Canada Ltd. (Mooretown) | 0100 , Final Effluent to River (Process Effluent) |
| RohMax Canada (Morrisburg) | 0100 , Final Outfall to River (Process Effluent) |
| Stepan Canada Inc. (Longford Mills) | 0200 , Clarifier Effluent (Process Effluent) |

SCHEDULE 10 
STORM WATER CONTROL STUDY PARAMETERS

Table 1
Cornwall Chemicals Ltd, (Cornwall)

| Item | Analytical Test Group | Column 1 Parameter |
| --- | --- | --- |
| 1. | 5a | Dissolved Organic Carbon |
| 2. | 6 | Total phosphorus |
| 3. | 8 | Total suspended solids |
| 4. | 9 | Aluminum |
| 5. | 9 | Nickel |
| 6. | 9 | Zinc |
| 7. | 12 | Mercury |
| 8. | 15 | Sulphide |
| 9. | 16 | Carbontetrachloride |
| 10. | 23 | Hexachloroethane |
| 11. | 25 | Oil and Grease |
| 12. | 24 | 2,3,7,8-tertachlorodibenzo-para-dioxin |
| 13. | 24 | 2,3,7,8-tetrachlorodibenzofuran |
| 14. | 24 | TEQ - total toxic equivalent of 2,3,7,8 substituted dioxin and furan congeners |

Table 2 
Nova Chemicals Canada Ltd. (Sarnia)

| Item | Analytical Test Group | Column 1 Parameter |
| --- | --- | --- |
| 1. | 5a | Dissolved Organic Carbon |
| 2. | 8 | Total suspended solids |
| 3. | 9 | Aluminum |
| 4. | 14 | Phenolics (4AAP) |
| 5. | 17 | Benzene |
| 6. | 17 | Ethylbenzene |
| 7. | 17 | Toluene |
| 8. | 25 | Oil and Grease |

Table 3 
OxyChem Durez (Fort Erie)

| Item | Analytical Test Group | Column 1 Parameter |
| --- | --- | --- |
| 1. | 3 | pH |
| 2. | 4a | Ammonia plus Ammonium |
| 3. | 5a | Dissolved Organic Carbon |
| 4. | 6 | Total phosphorus |
| 5. | 8 | Total suspended solids |
| 6. | 9 | Aluminum |
| 7. | 9 | Chromium |
| 8. | 9 | Zinc |
| 9. | 14 | Phenolics (4AAP) |
| 10. | 25 | Oil and Grease |

Table 4 
RohMax Canada (West Hill)

| Item | Analytical Test Group | Column 1 Parameter |
| --- | --- | --- |
| 1. | 4b | Nitrate plus Nitrite |
| 2. | 5a | Dissolved Organic Carbon |
| 3. | 8 | Total suspended solids |
| 4. | 9 | Aluminum |
| 5. | 9 | Boron |
| 6. | 9 | Molybdenum |
| 7. | 14 | Phenolics (4AAP) |
| 8. | 25 | Oil and Grease |

SCHEDULE 11
REFERENCE MATERIALS

| Column 1 Plant Name | Column 2 Materials |
| --- | --- |
| Amoco Canada Petroleum Company Limited | Propane Isobutane n-Butane Hydrocarbon Condensate |
| Bayer Inc. | Butadiene Rubbers Specialty Rubbers |
| Celanese Canada Inc. | Polyester Staple Fibre |
| Chinook Group Limited | Alkyl Amines Dimethyl Formamide Choline Chloride |
| Cornwall Chemicals Limited | Carbon Disulphide Carbon Tetrachloride |
| Dow Chemical Canada Inc. (Sarnia) — Vidal Street | Ethylbenzene Styrene Polystyrene Polyethylene Acrylic Latex Epoxy Resins Propylene Oxide Derivatives |
| Dupont Canada Inc. (Kingston) | Nylon 66 Yarn Nylon 66 Resin |
| Dupont Canada Inc. (Maitland) | Adipic Acid Hexamethylene Diamine Dichlorotrifluoroethane (HCFC-123) |
| Ethyl Canada Inc. | Tetraethyl Lead Aluminum Alkyls Diesel Ignition Improvers |
| GE Plastics Canada | Various resin plastics in pellet form including Acrylonitrile-Butadiene-Styrene (ABS), Polycarbonate, ABS/Polycarbonate, Polybutylene Terephthalate (PBT), Polybutylene Terephythalate (PBT)/Polycarbonate, Polyphenylene Oxide, (PPO)Polystyrene and Polyetherimide |
| Geon Canada Inc. | Polyvinyl Chloride (PVC) Resins |
| Imperial Oil Chemicals Division | Polyvinyl Chloride (PVC) Resins Polyethylene Resins Aromatics Feed |
| Nova Chemicals Canada Ltd. (Corunna) | Polyethylene |
| Nova Chemicals Canada Ltd. (Mooretown) | Polyethylene |
| RohMax Canada — Morrisburg | Petroleum Additives |
| Stepan Canada Inc. | Surfactants |

Commencement

20.  This Regulation comes into force on the day it is filed.

Back to top