Français

ontario regulation 376/17

made under the

Highway Traffic Act

Made: September 27, 2017
Filed: September 29, 2017
Published on e-Laws: September 29, 2017
Printed in The Ontario Gazette: October 14, 2017

Amending Reg. 587 of R.R.O. 1990

(EQUIPMENT)

1.  (1)  Subsection 5 (1) of Regulation 587 of the Revised Regulations of Ontario, 1990 is revoked and the following substituted:

(1)  The push rod stroke of the service brake chamber of a vehicle equipped with wheel brake air chambers shall not be more than the push rod stroke listed in Column 2 of Schedule 1, 2, 3, 4 or 5, as the case may be, for the type of chamber listed in Column 1 of the applicable Schedule, if the wheel brake has cam or disc type brakes.

(2)  Subsection 5 (2) of the Regulation is amended by striking out “of Schedule 1” at the end and substituting “of Schedule 1, 2, 3, 4 or 5”.

2.  Schedule 1 to the Regulation is revoked and the following substituted:

Schedule 1
Clamp Type Brake Chamber

| Column 1 Service Brake Chambers Type — Outside Diameter | Column 2 Push Rod Travel |
| --- | --- |
| 6 — 4½" (114.3 mm) | 1¼" (31.75 mm) |
| 9 — 5¼" (133.35 mm) | 13/8" (34.93 mm) |
| 12 — 511/16" (144.46 mm) | 13/8" (34.93 mm) |
| 12 Long Stroke — 511/16" (144.46 mm) | 1¾" (44.45 mm) |
| 16 — 63/8" (161.93 mm) | 1¾" (44.45 mm) |
| 16 Long Stroke — 63/8" (161.93 mm) | 2" (50.8 mm) |
| 20 — 625/32" (172.24 mm) | 1¾" (44.45 mm) |
| 20 Long Stroke — 625/32" (172.24 mm) | 2" (50.8 mm) |
| 24 — 77/32" (183.36mm) | 1¾" (44.45 mm) |
| 24 Long Stroke — 77/32" (183.36 mm) | 2" (50.8 mm) |
| 24 Long Stroke with Square Inlet Port or with Square Raised Embossment on Lid — 77/32" (183.36 mm) | 2½" (63.5 mm) |
| 30 — 8 3/32" (205.58 mm) | 2" (50.8 mm) |
| 30 Long Stroke with Square Inlet Port or with Square Raised Embossment on Lid — 8 3/32" (205.58 mm) | 2½" (63.5 mm) |
| 36 — 9" (228.6 mm) | 2¼" (57.15mm) |

Schedule 2
Bolt Type Chamber

| Column 1 Service Brake Chambers Type — Outside Diameter | Column 2 Push Rod Travel |
| --- | --- |
| A — 615/16" (176.21 mm) | 13/8" (34.93 mm) |
| B — 93/16" (233.36 mm) | 1¾" (44.45 mm) |
| C — 81/16" (204.79 mm) | 1¾" (44.45 mm) |
| D — 5¼" (133.35 mm) | 1¼" (31.75 mm) |
| E — 63/16" (157.16 mm) | 13/8" (34.93 mm) |
| F — 11" (279.4 mm) | 2¼" (57.15 mm) |
| G — 97/8" (250.83 mm) | 2" (50.8 mm) |

Schedule 3
Rotochamber Type Chamber

| Column 1 Service Brake Chambers Type — Outside Diameter | Column 2 Push Rod Travel |
| --- | --- |
| 9 — 49/32" (108.74 mm) | 1½" (38.1 mm) |
| 12 — 413/16" (122.23 mm) | 1½" (38.1 mm) |
| 16 — 513/32" (137.32 mm) | 2" (50.8 mm) |
| 20 — 515/16" (150.81 mm) | 2" (50.8 mm) |
| 24 — 613/32" (162.72 mm) | 2" (50.8 mm) |
| 30 — 71/16" (179.39 mm) | 2¼" (57.15 mm) |
| 36 — 75/8" (193.68 mm) | 2¾" (69.85 mm) |
| 50 — 87/8" (225.43 mm) | 3" (76.2 mm) |

Schedule 4
Tie Rod Piston Type Chamber

| Column 1 Service Brake Chambers Type — Outside Diameter | Column 2 Push Rod Travel |
| --- | --- |
| 30 Long Stroke with Square Inlet Port — 6½" (165.1 mm) | 2½" (63.5 mm) |

Schedule 5
DD-3 Type Chamber

| Column 1 Service Brake Chambers Type — Outside Diameter | Column 2 Push Rod Travel |
| --- | --- |
| 30 — 81/8" (206.37 mm) | 2¼" (57.15 mm) |

Commencement

3.  This Regulation comes into force on the later of the day section 7 of Schedule 12 to the Jobs for Today and Tomorrow Act (Budget Measures), 2016 comes into force and the day this Regulation is filed.

Français

Back to top