ontario regulation 228/19

made under the

Highway Traffic Act

Made: June 21, 2019
Filed: June 28, 2019
Published on e-Laws: June 28, 2019
Printed in The Ontario Gazette: July 13, 2019

Amending O. Reg. 413/05

(VEHICLE WEIGHTS AND DIMENSIONS - FOR SAFE, PRODUCTIVE AND INFRASTRUCTURE-FRIENDLY VEHICLES)

1.  (1)  Clause (c) of the definition of “quadruple axle” in subsection 2 (2) of Ontario Regulation 413/05 is revoked and the following substituted:

(c)	do not include a self-steering axle or a liftable axle, other than a smart lift axle,

(2)  Subsection 2 (2) of the Regulation is amended by adding the following definitions:

“smart lift axle” means a liftable axle contained within an axle unit or axle group that is equipped with smart lift axle technology;

“smart lift axle technology” means a system that meets the description and requirements set out in section 5.1;

(3)  The definition of “tandem axle” in subsection 2 (2) of the Regulation is revoked and the following substituted:

“tandem axle” means a dual axle as defined in section 114 of the Act that does not include a self-steering axle or a liftable axle, other than a smart lift axle, and that has the same number of tires at each wheel position;

(4)  The definition of “tridem axle” in subsection 2 (2) of the Regulation is revoked and the following substituted:

“tridem axle” means a triple axle as defined in section 114 of the Act that does not include a self-steering axle or a liftable axle, other than a smart lift axle, and that has the same number of tires at each wheel position;

2.  Item 14 of the Table to section 3 of the Regulation is revoked and the following substituted:

| 14 | 14 | Stinger-Steer Tractor Semi-trailer Auto or Boat Carrier |
| --- | --- | --- |

3.  Subsection 5 (2) of the Regulation is revoked and the following substituted:

(2)  A designated truck or a tractor in a designated combination may not be equipped with or have controls, whether remote or manual, that would allow the driver from the cab of the truck or tractor to lift, deploy or alter the weight on a self-steering axle or a forced-steer auxiliary pusher axle of the truck or of any drawn trailer unless,

(a)	the truck, truck-trailer combination or tractor-trailer combination is designed to carry raw forest products;

(b)	if the truck or tractor was built before January 1, 2020, the controls,

(i)	do not activate unless the emergency 4-way flashers are activated,

(ii)	contain a device that prevents lifting the axle or altering the axle weight when the truck or vehicle combination is travelling at a speed over 60 kilometres per hour, and

(iii)	in the case of a tractor-trailer combination, operate only on the most forward self-steering axle of the semi-trailer; or

(c)	if the truck or tractor was built on or after January 1, 2020, the control is an emergency lift axle override control that meets all of the following conditions:

(i)	It allows the driver from the cab of the truck or tractor to lift, alter or deploy the weight on a self-steering axle or forced-steer auxiliary pusher axle of the truck or of any drawn trailer.

(ii)	It is separate and independent from the 4-way flasher control.

(iii)	The words “Emergency Lift Axle Override” appear adjacent to the control.

(iv)	It activates the emergency 4-way flashers.

(v)	It only lifts, alters or deploys,

(A)	in the case of a tractor-trailer combination, the most forward self-steering axle of the semi-trailer, or

(B)	in the case of a truck, the self-steering axle or forced-steer auxiliary pusher axle.

(vi)	It begins the lifting or altering of weight immediately upon activation.

(vii)	It does not allow the axle to be lifted or its weight altered when the truck or vehicle combination is travelling at a speed over 60 kilometres per hour.

(viii)	It fully deploys the self-steering axle or forced-steer auxiliary pusher axle,

(A)	immediately upon the vehicle stopping,

(B)	immediately upon the tractor power being turned off, and

(C)	within three minutes after the lifting or altering of weight.

4.  The Regulation is amended by adding the following section:

Smart lift axles

5.1  (1)  Smart lift axle technology is a system on a semi-trailer in a designated tractor-trailer combination consisting of a load sensing control device and a lifting mechanism that,

(a)	automatically and without driver action lifts one, two or three of the smart lift axles of an axle unit or axle group, but,

(i)	only lifts the smart lift axles if the weight on the remaining axle unit or group will be at or below the weight limits permitted under the Act and this Regulation, and

(ii)	does not begin to lift any smart lift axle if any self-steering axle is deployed;

(b)	maintains the braking power of the remaining axles within the axle unit or axle group when the smart lift axles are lifted;

(c)	automatically deploys all axles within the axle unit or axle group when the semi-trailer is parked, the tractor power is off or the system malfunctions; and

(d)	does not cause the semi-trailer to fail to meet any standard under the Motor Vehicle Safety Regulations made under the Motor Vehicle Safety Act (Canada).

(2)  For the purpose of subclause (1) (a) (i), in determining whether the weight remaining on the axle unit or group meets the weight requirements under the Act or this Regulation, the applicable weight requirements are those for the axle unit or group with the number of axles deployed.

(3)  An axle unit or axle group that is equipped with smart lift axle technology must maintain, at a minimum, the fixed axles set out in the following table:

Smart Lift Technology — Fixed axle requirement

| Designated Tractor-trailer combination | Axle unit or group | Fixed axle requirement |
| --- | --- | --- |
| 1 | Tandem axle | Either axle |
| 1 | Tridem axle | Middle or rear axle |
| 2 | Tandem axle | Either axle |
| 3 | Tridem axle | Middle or rear axle |
| 4 | Tridem axle | Rear axle |
| 5 | Tridem axle | Middle or rear axle |
| 6 | Quadruple axle | Rear Axle |
| 7 | Quadruple axle | Any axle |
| 8 | Tandem axle | Either axle |
| 8 | Tridem axle | Middle or rear axle |
| 9 | Tandem axle | Either axle |
| 10 | Tridem axle | Middle or rear axle |
| 11 | Tandem axle | Rear axle |
| 12 | Tandem axle | Either axle |
| 12 | Tridem axle | Middle or rear axle |
| 13 | Tandem axle | Rear axle |

(4)  In the case of designated tractor-trailer combination 7, the smart lift axle technology must prevent the lifting of only the second or third axle or only both second and third axles of the quadruple axle.

5.  Subsection 15 (2) of the Regulation is revoked and the following substituted:

(2)  A long combination that is a designated tractor-trailer combination 4, 6 or 7 in every respect except that both self-steering axles are raised, and in the case of designated tractor-trailer combination 7 no smart lift axles are raised, shall comply with the dimensional limits in the corresponding Schedule and not with the dimensional limits specified in section 109 of the Act, if the dimensional limits in the Schedule are greater than those in section 109 of the Act and if the gross vehicle weight of the long combination does not exceed the amount permitted in Vehicle Weight Table 32.

6.  Subsection 37 (1) of the Regulation is amended by adding the following paragraph:

2.1	If the axle unit or group is equipped with a smart lift axle, a reference to a tandem, tridem or quadruple axle in an axle unit description in a Dimensional Limit Chart or Weight Limit Chart shall be read as a reference to an axle unit or axle group of the number of axles actually deployed.

7.  (1)  The Dimensional Limit Chart in Schedule 1 to the Regulation is amended by striking out,

| Tractor | (6) | Wheelbase | Max. 7.2m if tractor built after 2005 |
| --- | --- | --- | --- |
| Tractor | (6) | Wheelbase — (long combination) | Max. 7.2m |

and substituting the following:

| Tractor | (6) | Wheelbase | Max. 7.2m |
| --- | --- | --- | --- |

(2)  The Weight Limit Chart (Other Axle Maximums) in Schedule 1 to the Regulation is amended by striking out,

| c) by axle unit description | Other Tractor Axles and Trailer Axles: Single Axle (Single Tires): 9,000 kg Single Axle (Dual Tires): 10,000 kg Tandem Axle by axle spread: 1.2 < 1.8m: 18,000 kg 1.8m or more (Single Tires): 18,000 kg 1.8m or more (Dual Tires):19,100 kg Tridem Axle – by axle spread: 2.4 < 3.0m: 21,300 kg 3.0 < 3.6m: 24,000 kg 3.6 to 3.7m: 26,000 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | Other Tractor Axles and Trailer Axles: Single Axle: 10,000 kg Tandem Axle by axle spread: 1.2 < 1.8m: 18,000 kg 1.8m or more (Single Tires): 18,000 kg 1.8m or more (Dual Tires): 19,100 kg Tridem Axle – by axle spread: 2.4 < 3.0m: 21,300 kg 3.0 < 3.6m: 24,000 kg 3.6 to 3.7m: 26,000 kg |
| --- | --- |

8.  (1)  The Dimensional Limit Chart in Schedule 2 to the Regulation is revoked and the following substituted:

DIMENSIONAL LIMIT CHART

| Vehicle | Ref. | Feature | Dimensional Limit |
| --- | --- | --- | --- |
| Overall | (1) | Overall Length | Max. 23.0m |
| Overall | (2) | Width | Max. 2.6m |
| Overall | (3) | Height | Max. 4.15m |
| Tractor | (4) | Inter-axle Spacing | Min. 3.0 m |
| Tractor | (5) | Tandem Axle Spread | 1.2 to 1.85m |
| Tractor | (6) | Wheelbase | Max. 6.8m |
| Semi-Trailer | (8) | Length | Max. 16.2m |
| Semi-Trailer | (9) | End-Dump Semi-Trailer Bed Length | Max. 14.65m |
| Semi-Trailer | (10) | Swing Radius | Max. 2.0m |
| Semi-Trailer | (11) | Wheelbase — if tractor wheelbase is 6.2 m or less | 6.25 to 12.5m |
| Semi-Trailer | (11) | Wheel base — if tractor wheel base is > 6.2 m to 6.3 m | 6.25 to 12.47m |
| Semi-Trailer | (11) | Wheel base — if tractor wheel base is > 6.3 m to 6.4 m | 6.25 to 12.40m |
| Semi-Trailer | (11) | Wheel base — if tractor wheel base is > 6.4 m to 6.5 m | 6.25 to 12.33m |
| Semi-Trailer | (11) | Wheel base — if tractor wheel base is > 6.5 m to 6.6 m | 6.25 to 12.27m |
| Semi-Trailer | (11) | Wheel base — if tractor wheel base is > 6.6 m to 6.7 m | 6.25 to 12.20m |
| Semi-Trailer | (11) | Wheel base — if tractor wheel base is > 6.7 m to 6.8 m | 6.25 to 12.13m |
| Semi-Trailer | (12) | Effective Rear Overhang | Max. 35% of wheelbase |
| Semi-Trailer | (13) | Inter-vehicle-unit Distance | Min. 4.0m if trailer built after 2005 |
| Semi-Trailer | (15) | Inter-axle Spacing | > 2.5 to 3.0m |
| Semi-Trailer | (16) | Tandem Spread | 1.2 to 2.8m |
| Semi-Trailer | (19) | Track Width — tandem — trailer with single tires built before 2010 | 2.3 to 2.6m |
| Semi-Trailer | (19) | Track Width — tandem — trailer with single tires built after 2009 | 2.45 to 2.6m |
| Semi-Trailer | (19) | Track Width — tandem — all other trailers | 2.5 to 2.6m |

(2)  The Weight Limit Chart (Other Axle Maximums) in Schedule 2 to the Regulation is amended by striking out,

| c) by axle unit description | Tractor Tandem Drive Axle by axle spread: 1.2 < 1.8m: 18,000 kg 1.8m or more (Single Tires): 18,000 kg 1.8m or more (Dual Tires): 19,100 kg Trailer Axles: i. If tandem axle spread is 1.2 < 1.8m or there are single tires on any wheel of trailer: 1. self-steer axle (load equalized): 9,000 kg 2. self-steer axle (not load equalized): 7,500 kg 3. tandem axle (load equalized): 18,000 kg 4. tandem axle (not load equalized): 15,000 kg ii. If tandem axle spread is 1.8 to 2.8m and there are no single tires on any wheel of trailer: 1. self-steer axle (load equalized): 9,550 kg 2. self-steer axle (not load equalized): 8,000 kg 3. tandem axle (load equalized): 19,100 kg 4. tandem axle (not load equalized): 16,000 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | Tractor Tandem Drive Axle by axle spread: 1.2 < 1.8m: 18,000 kg 1.8m or more (Single Tires): 18,000 kg 1.8m or more (Dual Tires): 19,100 kg Trailer Axles: i. If tandem axle spread is 1.2 < 1.8m: 1. self-steer axle (load equalized): 9,000 kg 2. self-steer axle (not load equalized): 7,500 kg 3. tandem axle (load equalized): 18,000 kg 4. tandem axle (not load equalized): 15,000 kg ii. If tandem axle spread is 1.8 to 2.8m: 1. self-steer axle (load equalized): 9,550 kg 2. self-steer axle (not load equalized): 8,000 kg 3. tandem axle (load equalized): 19,100 kg 4. tandem axle (not load equalized): 16,000 kg |
| --- | --- |

9.  The Dimensional Limit Chart in Schedule 3 to the Regulation is revoked and the following substituted:

DIMENSIONAL LIMIT CHART

| Vehicle | Ref. | Feature | Dimensional Limit |
| --- | --- | --- | --- |
| Overall | (1) | Overall Length | Max. 23.0m |
| Overall | (2) | Width | Max. 2.6m |
| Overall | (3) | Height | Max. 4.15m |
| Tractor | (4) | Inter-axle Spacing | Min. 3.0m |
| Tractor | (5) | Tandem Axle Spread | 1.2 to 1.85m |
| Tractor | (6) | Wheelbase | Max. 6.8 |
| Semi-Trailer | (8) | Length | Max. 16.2m |
| Semi-Trailer | (9) | End-Dump Semi-Trailer Bed Length | Max. 14.65m |
| Semi-Trailer | (10) | Swing Radius | Max. 2.0m |
| Semi-Trailer | (11) | Wheelbase — if tractor wheel base is 6.2 m or less | 6.25 to 12.50m |
| Semi-Trailer | (11) | Wheel base — if tractor wheel base is > 6.2 m to 6.3 m | 6.25 to 12.47m |
| Semi-Trailer | (11) | Wheel base — if tractor wheel base is > 6.3 m to 6.4 m | 6.25 to 12.40m |
| Semi-Trailer | (11) | Wheel base — if tractor wheel base is > 6.4 m to 6.5 m | 6.25 to 12.33m |
| Semi-Trailer | (11) | Wheel base — if tractor wheel base is > 6.5 m to 6.6 m | 6.25 to 12.27m |
| Semi-Trailer | (11) | Wheel base — if tractor wheel base is > 6.6 m to 6.7 m | 6.25 to 12.20m |
| Semi-Trailer | (11) | Wheel base — if tractor wheel base is > 6.7 m to 6.8 m | 6.25 to 12.13m |
| Semi-Trailer | (12) | Effective Rear Overhang | Max. 35% of wheelbase |
| Semi-Trailer | (13) | Inter-vehicle-unit Distance — if tridem spread is 3.0 < 3.6m | Min. 6.0m |
| Semi-Trailer | (13) | Inter-vehicle-unit Distance — if tridem spread is 3.6 to 3.7m | Min. 5.5m |
| Semi-Trailer | (15) | Inter-axle Spacing | > 2.5 to 3.0m |
| Semi-Trailer | (16) | Tridem Spread | 3.0 to 3.7m |
| Semi-Trailer | (19) | Track Width — tridem — trailer with single tires built before 2010 | 2.3 to 2.6m |
| Semi-Trailer | (19) | Track Width — tridem — trailer with single tires built after 2009 | 2.45 to 2.6m |
| Semi-Trailer | (19) | Track Width — tridem — all other trailers | 2.5 to 2.6m |

10.  The Dimensional Limit Chart in Schedule 4 to the Regulation is amended by striking out,

| Tractor | (6) | Wheelbase | Max. 6.2m if tractor built after 2005 |
| --- | --- | --- | --- |
| Tractor | (6) | Wheelbase — (long combination) | Max. 6.2m |

and substituting the following:

| Tractor | (6) | Wheelbase | Max. 6.8m |
| --- | --- | --- | --- |

11.  The Dimensional Limit Chart in Schedule 5 to the Regulation is revoked and the following substituted:

DIMENSIONAL LIMIT CHART

| Vehicle | Ref. | Feature | Dimensional Limit |
| --- | --- | --- | --- |
| Overall | (1) | Overall Length | Max. 23.0m |
| Overall | (2) | Width | Max. 2.6m |
| Overall | (3) | Height | Max. 4.15m |
| Tractor | (4) | Inter-axle Spacing | Min. 3.0 m |
| Tractor | (5) | Tandem Axle Spread | 1.3 to 1.85m |
| Tractor | (6) | Wheelbase | Max. 6.8m |
| Semi-Trailer | (8) | Length | Max. 16.2m |
| Semi-Trailer | (9) | End-Dump Semi-Trailer Bed Length | Max. 14.65m |
| Semi-Trailer | (10) | Swing Radius | Max. 2.0m |
| Semi-Trailer | (11) | Wheelbase – if tractor wheelbase is 6.2 m or less | 11.5 to 12.5m |
| Semi-Trailer | (11) | Wheelbase – if tractor wheelbase is > 6.2 m to 6.3 m | 11.5 to 12.47m |
| Semi-Trailer | (11) | Wheelbase – if tractor wheelbase is > 6.3 m to 6.4 m | 11.5 to 12.40 m |
| Semi-Trailer | (11) | Wheelbase – if tractor wheelbase is > 6.4 m to 6.5 m | 11.5 to 12. 33 m |
| Semi-Trailer | (11) | Wheelbase – if tractor wheelbase is > 6.5 m to 6.6 m | 11.5 to 12.27 m |
| Semi-Trailer | (11) | Wheelbase – if tractor wheelbase is > 6.6 m to 6.7 m | 11.5 to 12.20 m |
| Semi-Trailer | (11) | Wheelbase – if tractor wheelbase is > 6.7 m to 6.8 m | 11.5 to 12.13 m |
| Semi-Trailer | (12) | Effective Rear Overhang | Max. 35% of wheelbase |
| Semi-Trailer | (13) | Inter-vehicle-unit Distance | Min. 3.2m |
| Semi-Trailer | (14) | Inter-axle Spacing | 1.5 to 2.8m |
| Semi-Trailer | (13) + (14) | Inter-vehicle unit Distance + Inter-axle Spacing | Min. 6.0m |
| Semi-Trailer | (15) | Inter-axle Spacing | >2.5 to 2.8m |
| Semi-Trailer | (16) | Tridem Spread | 3.0 to 3.1m |
| Semi-Trailer | (19) | Track Width — tridem axle — trailer with single tires built before 2010 | 2.3 to 2.6m |
| Semi-Trailer | (19) | Track Width — tridem axle — trailer with single tires built after 2009 | 2.45 to 2.6m |
| Semi-Trailer | (19) | Track Width — tridem axle — all other trailers | 2.5 to 2.6m |

12.  The Dimensional Limit Chart in Schedule 6 to the Regulation is amended by striking out,

| Tractor | (6) | Wheelbase | Max. 6.2m if tractor built after 2005 |
| --- | --- | --- | --- |
| Tractor | (6) | Wheelbase — (long combination) | Max. 6.2m |

and substituting the following:

| Tractor | (6) | Wheelbase | Max. 6.8m |
| --- | --- | --- | --- |

13.  (1)  Schedule 7 to the Regulation is amended by adding the following immediately before the Dimensional Limit Chart:

Alternatives

A combination that meets every requirement to be Designated Tractor-Trailer Combination 7, except that both self-steer axles and one or more smart lift axles of the semi-trailer are not deployed, is Designated Tractor-Trailer Combination 1.

(2)  The Dimensional Limit Chart in Schedule 7 to the Regulation is amended by striking out,

| Tractor | (6) | Wheelbase | Max. 6.2m if tractor built after 2005 |
| --- | --- | --- | --- |
| Tractor | (6) | Wheelbase — (long combination) | Max. 6.2m |

and substituting the following:

| Tractor | (6) | Wheelbase | Max. 6.8m |
| --- | --- | --- | --- |

14.  The Weight Limit Chart (Other Axle Maximums) in Schedule 8 to the Regulation is amended by striking out,

| c) by axle unit description | Tractor Drive Axle – by tridem axle spread: 2.4 < 2.7m: 21,300 kg 2.7 to 2.8m: 22,000 kg Trailer Axles: Single Axle (Single Tires): 9,000 kg Single Axle (Dual Tires): 10,000 kg Tandem axle – by axle spread: 1.2 < 1.8m: 18,000 kg 1.8m or more (Single Tires): 18,000 kg 1.8m or more (Dual Tires): 19,100 kg Tridem Axle – by axle spread: 2.4 < 3.0m: 21,300 kg 3.0 < 3.6m: 24,000 kg 3.6 to 3.7m: 26,000 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | Tractor Drive Axle – by tridem axle spread: 2.4 < 2.7m: 21,300 kg 2.7 to 2.8m: 22,000 kg Trailer Axles: Single Axle: 10,000 kg Tandem axle – by axle spread: 1.2 < 1.8m: 18,000 kg 1.8m or more (Single Tires): 18,000 kg 1.8m or more (Dual Tires): 19,100 kg Tridem Axle – by axle spread: 2.4 < 3.0m: 21,300 kg 3.0 < 3.6m: 24,000 kg 3.6 to 3.7m: 26,000 kg |
| --- | --- |

15.  (1)  The Dimensional Limit Chart in Schedule 9 to the Regulation is amended by striking out,

| Semi-Trailer | (8) | Length (long combination) | Max. 16.2m |
| --- | --- | --- | --- |

and substituting the following:

| Semi-Trailer | (8) | Length | Max. 16.2m |
| --- | --- | --- | --- |

(2)  The Weight Limit Chart (Other Axle Maximums) in Schedule 9 to the Regulation is amended by striking out,

| c) by axle unit description | Tractor Drive Axle – by tridem axle spread: 2.4 < 2.7m: 21,300 kg 2.7 to 2.8m: 22,000 kg Trailer Axles: i. If tandem axle spread is 1.2 < 1.8m or there are single tires on any wheel of trailer: 1. self-steer axle (load equalized): 9,000 kg 2. self-steer axle (not load equalized): 7,500 kg 3. tandem axle (load equalized): 18,000 kg 4. tandem axle (not load equalized): 15,000 kg ii. If tandem axle spread is 1.8 to 2.8m and there are no single tires on any wheel of trailer: 1. self-steer axle (load equalized): 9,550 kg 2. self-steer axle (not load equalized): 8,000 kg 3. tandem axle (load equalized): 19,100 kg 4. tandem axle (not load equalized): 16,000 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | Tractor Drive Axle – by tridem axle spread: 2.4 < 2.7m: 21,300 kg 2.7 to 2.8m: 22,000 kg Trailer Axles: i. If tandem axle spread is 1.2 < 1.8m: 1. self-steer axle (load equalized): 9,000 kg 2. self-steer axle (not load equalized): 7,500 kg 3. tandem axle (load equalized): 18,000 kg 4. tandem axle (not load equalized): 15,000 kg ii. If tandem axle spread is 1.8 to 2.8m: 1. self-steer axle (load equalized): 9,550 kg 2. self-steer axle (not load equalized): 8,000 kg 3. tandem axle (load equalized): 19,100 kg 4. tandem axle (not load equalized): 16,000 kg |
| --- | --- |

16.  (1)  The Dimensional Limit Chart in Schedule 11 to the Regulation is amended by striking out,

| Tractor | (6) | Wheelbase | Max. 6.2m if tractor built after 2005 |
| --- | --- | --- | --- |
| Tractor | (6) | Wheelbase — (long combination) | Max. 6.2m |

and substituting the following:

| Tractor | (6) | Wheelbase | Max. 6.2m |
| --- | --- | --- | --- |

(2)  The Weight Limit Chart (Other Axle Maximums) in Schedule 11 to the Regulation is amended by striking out,

| c) by axle unit description | Other Tractor Axles and Trailer Axles: Single Axle (Single Tires): 9,000 kg Single Axle (Dual Tires): 10,000 kg Tandem Axle – by axle spread: 1.2 < 1.8m: 18,000 kg 1.8 to 1.85m (Single Tires): 18,000 kg 1.8 to 1.85m (Dual Tires): 19,100 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | Other Tractor Axles and Trailer Axles: Single Axle: 10,000 kg Tandem Axle – by axle spread: 1.2 < 1.8m: 18,000 kg 1.8 to 1.85m (Single Tires): 18,000 kg 1.8 to 1.85m (Dual Tires): 19,100 kg |
| --- | --- |

17.  (1)  The Dimensional Limit Chart in Schedule 12 to the Regulation is amended by striking out,

| Tractor | (6) | Wheelbase | Max. 6.8m if tractor built after 2005 |
| --- | --- | --- | --- |
| Tractor | (6) | Wheelbase — (long combination) | Max. 6.8m |

.     .     .     .     .

| Lead Semi-Trailer | (13) | Inter-vehicle-unit Distance — single to tandem or tridem | Min. 3.0m |
| --- | --- | --- | --- |

and substituting the following:

| Tractor | (6) | Wheelbase | Max. 6.8m |
| --- | --- | --- | --- |

.     .     .     .     .

| Lead Semi-Trailer | (13) | Inter-vehicle-unit Distance — single to single, tandem or tridem | Min. 3.0m |
| --- | --- | --- | --- |

(2)  The Weight Limit Chart (Other Axle Maximums) in Schedule 12 to the Regulation is amended by striking out,

| c) by axle unit description | Other Tractor and Trailer Axles: Single Axle (Single Tires): 9,000 kg Single Axle (Dual Tires): 10,000 kg Tandem Axle – by axle spread: 1.2 < 1.8m: 18,000 kg 1.8 to 1.85m (Single Tires): 18,000 kg 1.8 to 1.85m (Dual Tires): 19,100 kg Tridem Axle – by axle spread 2.4 < 3.0m: 21,300 kg 3.0 to 3.1: 24,000 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | Other Tractor and Trailer Axles: Single Axle: 10,000 kg Tandem Axle – by axle spread: 1.2 < 1.8m: 18,000 kg 1.8 to 1.85m (Single Tires): 18,000 kg 1.8 to 1.85m (Dual Tires): 19,100 kg Tridem Axle – by axle spread 2.4 < 3.0m: 21,300 kg 3.0 to 3.1: 24,000 kg |
| --- | --- |

18.  (1)  The Dimensional Limit Chart in Schedule 13 to the Regulation is amended by striking out,

| Tractor | (6) | Wheelbase | Max. 6.2m if tractor built after 2005 |
| --- | --- | --- | --- |
| Tractor | (6) | Wheelbase — (long combination) | Max. 6.2m |

and substituting the following:

| Tractor | (6) | Wheelbase | Max. 6.2m |
| --- | --- | --- | --- |

(2)  The Weight Limit Chart (Other Axle Maximums) in Schedule 13 to the Regulation is amended by striking out,

| c) by axle unit description | Other Tractor and Trailer Axles: Single Axle (Single Tires): 9,000 kg Single Axle (Dual Tires): 10,000 kg Tandem Axle – by axle spread 1.2 < 1.8m: 18,000 kg 1.8 to 1.85m (Single Tires): 18,000 kg 1.8 to 1.85m (Dual Tires): 19,100 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | Other Tractor and Trailer Axles: Single Axle: 10,000 kg Tandem Axle – by axle spread 1.2 < 1.8m: 18,000 kg 1.8 to 1.85m (Single Tires): 18,000 kg 1.8 to 1.85m (Dual Tires): 19,100 kg |
| --- | --- |

19.  (1)  The title of Schedule 14 to the Regulation is amended by adding “or boat” after “auto”.

(2)  Schedule 14 to the Regulation is amended by striking out the first paragraph under the heading “Exceptions” and substituting the following:

A non-enclosed auto or boat carrier where only the loaded motor vehicles or boats and support plates, if any, on which the motor vehicles or boats rest extend beyond the Overall Length (1), Height (3) and Tractor Effective Rear Overhang (12) limits set out in the Dimensional Limit Chart below is exempt from those limits. The following limits apply to these exempted auto or boat carriers:

(3)  The Weight Limit Chart (Other Axle Maximums) in Schedule 14 to the Regulation is amended by striking out,

| c) by axle unit description | Other Tractor Axles and Trailer Axles: Single Axle (Single Tires): 9,000 kg Single Axle (Dual Tires): 10,000 kg Tandem Axle: 18,000 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | Other Tractor Axles and Trailer Axles: Single Axle: 10,000 kg Tandem Axle: 18,000 kg |
| --- | --- |

20.  The Weight Limit Chart (Other Axle Maximums) in Schedule 16 to the Regulation is amended by striking out,

| c) by axle unit description | Single Drive Axle (Single Tires): 9,000 kg Single Drive Axle (Dual Tires): 10,000 kg Single Drive Axle (Dual Tires) on Urban Transit Bus not equipped with a tag axle: 11,400 kg Tag Axle (by Two Axle Group spacing) 1.2 < 1.3m : 6,300 kg 1.3 < 1.4m: 6,700 kg 1.4 < 1.5m: 7,000 kg 1.5 < 1.85m: 7,400 kg Tandem Drive Axle: 18,000 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | Single Drive Axle: 10,000 kg Single Drive Axle (Dual Tires) on Urban Transit Bus not equipped with a tag axle: 11,400 kg Tag Axle (by Two Axle Group spacing) 1.2 < 1.3m : 6,300 kg 1.3 < 1.4m: 6,700 kg 1.4 < 1.5m: 7,000 kg 1.5 < 1.85m: 7,400 kg Tandem Drive Axle: 18,000 kg |
| --- | --- |

21.  The Weight Limit Chart (Other Axle Maximums) in Schedule 17 to the Regulation is amended by striking out,

| c) by axle unit description | Single Drive Axle (Single Tires): 9,000 kg Single Drive Axle (Dual Tires): 10,000 kg Tag Axle (by Two Axle Group spacing) 1.2 < 1.3m: 6,300 kg 1.3 < 1.4m: 6,700 kg 1.4 < 1.5m: 7,000 kg 1.5 < 1.85m: 7,400 kg Tandem Drive Axle: 18,000 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | Single Drive Axle: 10,000 kg Tag Axle (by Two Axle Group spacing) 1.2 < 1.3m: 6,300 kg 1.3 < 1.4m: 6,700 kg 1.4 < 1.5m: 7,000 kg 1.5 < 1.85m: 7,400 kg Tandem Drive Axle: 18,000 kg |
| --- | --- |

22.  The Weight Limit Chart (Other Axle Maximums) in Schedule 18 to the Regulation is amended by striking out,

| c) by axle unit description | Single Axle (Single Tires): 9,000 kg Single Axle (Dual Tires): 10,000 kg Single Drive Axle (Dual Tires) on Urban Transit Bus: 11,400 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | Single Axle: 10,000 kg Single Drive Axle (Dual Tires) on Urban Transit Bus: 11,400 kg |
| --- | --- |

23.  The Weight Limit Chart (Other Axle Maximums) in Schedule 19 to the Regulation is amended by striking out,

| c) by axle unit description | Drive Axle: Single Axle (Single Tires): 9,000 kg Single Axle (Dual Tires): 10,000 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | Drive Axle: Single Axle: 10,000 kg |
| --- | --- |

24.  The Weight Limit Chart (Other Axle Maximums) in Schedule 23 to the Regulation is amended by striking out,

| c) by axle unit description | i. If tandem axle spread is 1.2 < 1.8m or there are single tires on any wheel: 1. self-steer axle (load equalized): 9,000 kg 2. self-steer axle (not load equalized): 7,500 kg 3. tandem drive axle (load equalized): 18,000 kg 4. tandem drive axle (not load equalized): 15,000 kg ii. If tandem axle spread is 1.8 to 1.88m and there are no single tires on any wheel: 1. self-steer axle (load equalized): 9,550 kg 2. self-steer axle (not load equalized): 8,000 kg 3. tandem drive axle (load equalized): 19,100 kg 4. tandem drive axle (not load equalized): 16,000 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | i. If tandem axle spread is 1.2 < 1.8m: 1. self-steer axle (load equalized): 9,000 kg 2. self-steer axle (not load equalized): 7,500 kg 3. tandem drive axle (load equalized): 18,000 kg 4. tandem drive axle (not load equalized): 15,000 kg ii. If tandem axle spread is 1.8 to 1.88m: 1. self-steer axle (load equalized): 9,550 kg 2. self-steer axle (not load equalized): 8,000 kg 3. tandem drive axle (load equalized): 19,100 kg 4. tandem drive axle (not load equalized): 16,000 kg |
| --- | --- |

25.  The Weight Limit Chart (Trailer Axle Maximums) in Schedule 26 to the Regulation is amended by striking out,

| c) by axle unit description | Single Axle (Single Tires): 9,000 kg Single Axle (Dual Tires): 10,000 kg Tandem Axle by axle spread: 1.2 < 1.8m: 18,000 kg 1.8 to 1.85m (Single Tires): 18,000 kg 1.8 to 1.85m (Dual Tires): 19,100 kg Tridem Axle by axle spread: 2.4 < 3.0m: 21,300 kg 3.0 < 3.6m: 24,000 kg 3.6 to 3.7m: 26,000 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | Single Axle: 10,000 kg Tandem Axle by axle spread: 1.2 < 1.8m: 18,000 kg 1.8 to 1.85m (Single Tires): 18,000 kg 1.8 to 1.85m (Dual Tires): 19,100 kg Tridem Axle by axle spread: 2.4 < 3.0m: 21,300 kg 3.0 < 3.6m: 24,000 kg 3.6 to 3.7m: 26,000 kg |
| --- | --- |

26.  The Weight Limit Chart (Trailer Axle Maximums) in Schedule 27 to the Regulation is amended by striking out,

| c) by axle unit description | i. If tandem axle spread is 1.2 < 1.8m or there are single tires on any wheel: 1. self-steer axle (load equalized): 9,000 kg 2. self-steer axle (not load equalized): 7,500 kg 3. tandem axle (load equalized): 18,000 kg 4. tandem axle (not load equalized): 15,000 kg ii. If tandem axle spread is 1.8 to 1.85m and there are no single tires on any wheel: 1. self-steer axle (load equalized): 9,550 kg 2. self-steer axle (not load equalized): 8,000 kg 3. tandem axle (load equalized): 19,100 kg 4. tandem axle (not load equalized): 16,000 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | i. If tandem axle spread is 1.2 < 1.8m: 1. self-steer axle (load equalized): 9,000 kg 2. self-steer axle (not load equalized): 7,500 kg 3. tandem axle (load equalized): 18,000 kg 4. tandem axle (not load equalized): 15,000 kg ii. If tandem axle spread is 1.8 to 1.85m: 1. self-steer axle (load equalized): 9,550 kg 2. self-steer axle (not load equalized): 8,000 kg 3. tandem axle (load equalized): 19,100 kg 4. tandem axle (not load equalized): 16,000 kg |
| --- | --- |

27.  The Weight Limit Chart (Trailer Axle Maximums) in Schedule 28 to the Regulation is amended by striking out,

| c) by axle unit description | Single Axle (Single tires): 9,000 kg Single Axle (Dual tires): 9,100 kg Tandem Axle: 18,000 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | Single Axle: 9,100 kg Tandem Axle: 18,000 kg |
| --- | --- |

28.  The Weight Limit Chart (Trailer Axle Maximums) in Schedule 29 to the Regulation is amended by striking out,

| c) by axle unit description | Self-steer axle (single tires): 9,000 kg Self-steer axle (dual tires): 9,550 kg Tandem Axle – by axle spread: 1.2 < 1.8 m: 18,000 kg 1.8 to 1.85m (Single Tires): 18,000 kg 1.8 to 1.85m (Dual Tires): 19,100 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | Self-steer axle: 10,000 kg Tandem Axle – by axle spread: 1.2 < 1.8 m: 18,000 kg 1.8 to 1.85m (Single Tires): 18,000 kg 1.8 to 1.85m (Dual Tires): 19,100 kg |
| --- | --- |

29.  (1)  Schedule 31 to the Regulation is amended by striking out “Overall Length (1) – 25m” under the heading “Exceptions” and substituting “Overall Length (1) – 27.5m”

(2)  The Weight Limit Chart (Other Axle Maximums) in Schedule 31 to the Regulation is amended by striking out,

| c) by axle unit description | Lead Tractor or Truck and drawn Tractors or Trucks: Single Axle (Single Tires): 9,000 kg Single Axle (Dual Tires): 10,000 kg Tandem Axle – by axle spread: 1.2 < 1.8m: 18,000 kg 1.8 m or more (Single Tires): 18,000 kg 1.8 m or more (Dual Tires): 19,100 kg Tridem Axle – by axle spread: 2.4 < 2.7m: 21,300 kg 2.7 m or more: 22,000 kg |
| --- | --- |

and substituting the following:

| c) by axle unit description | Lead Tractor or Truck and drawn Tractors or Trucks: Single Axle: 10,000 kg Tandem Axle – by axle spread: 1.2 < 1.8m: 18,000 kg 1.8 m or more (Single Tires): 18,000 kg 1.8 m or more (Dual Tires): 19,100 kg Tridem Axle – by axle spread: 2.4 < 2.7m: 21,300 kg 2.7 m or more: 22,000 kg |
| --- | --- |

Commencement

30.  This Regulation comes into force on the later of July 1, 2019 and the day it is filed.

Back to top