Français

ontario regulation 327/19

made under the

Occupational Health and Safety Act

Made: September 13, 2019
Filed: October 2, 2019
Published on e-Laws: October 2, 2019
Printed in The Ontario Gazette: October 19, 2019

Amending O. Reg. 213/91

(CONSTRUCTION PROJECTS)

1.  (1)  Subsection 75 (1) of Ontario Regulation 213/91 is amended by striking out “that will be at least two storeys high when it is finished”.

(2)  Subsection 75 (2) of the Regulation is amended by striking out “permanent or temporary stairs shall be installed up to” at the end of the portion before clause (a) and substituting “permanent or temporary stairs shall be installed from the lowest level, including the basement, up to”.

(3)  Subsection 75 (3) of the Regulation is amended by striking out “or” at the end of clause (a), by adding “or” at the end of clause (b) and by adding the following clause:

(c)	a part of a building or structure in which formwork or falsework is erected to a suspended slab.

2.  Subsection 181 (1) of the Regulation is amended by striking out “revised 2014” at the end and substituting “revised 2019”.

Commencement

3.  This Regulation comes into force on the later of January 1, 2020 and the day it is filed.

Français

Back to top