Safe Drinking Water Act, 2002
Loi de 2002 sur la salubrité de l’eau potable

ONTARIO REGULATION 128/04

certification of drinking water system Operators and water quality analysts

Historical version for the period December 6, 2010 to December 31, 2010.

Last amendment:  O. Reg. 466/10.

This Regulation is made in English only.

Skip Table of Contents

CONTENTS

| Definitions |  |
| --- | --- |
| 1. | Definitions |
| Classification of Municipal Residential Systems |  |
| 2. | Subsystems and classes of municipal residential systems |
| 3. | Certificate of classification |
| 4. | Existing Certificates of Classification |
| Types of Limited Systems |  |
| 5. | Subsystems of limited systems |
| Certification of Operators |  |
| 6. | Classes of operators’ certificates |
| 7. | Municipal residential operators’ certificates — Classes I, II, III and IV |
| 7.1 | Municipal residential operator’s certificate, out-of-province applicant |
| 8. | Limited subsystem operators’ certificates |
| 8.1 | Limited subsystem operator’s certificate, out-of-province applicant |
| 9. | Operator-in-training’s certificates |
| 10. | Conditional operators’ certificates |
| 11. | Reissuance of certificate after expiry |
| 12. | Transferability of certificates |
| 13. | Revocation or suspension of certificate |
| 14. | Replacement certificates |
| 15. | Certificate to be displayed |
| Certification of Water Quality Analysts |  |
| 16. | Water quality analyst’s certificates |
| 16.1 | Water quality analyst’s certificate, out-of-province applicant |
| 17. | Conditional water quality analyst’s certificates |
| 18. | Reissuance of certificate after expiry |
| 19. | Revocation or suspension of certificate |
| 20. | Replacement certificates |
| 21. | Certificate to be displayed |
| Operating Standards for Municipal Residential Subsystems and Limited Subsystems |  |
| 22. | Owner or operating authority responsibility |
| 23. | Overall responsible operator |
| 24. | Strikes and lock-outs |
| 25. | Operator-in-charge |
| 26. | Duties of operator-in-charge |
| 27. | Record-keeping re operation of subsystem |
| 28. | Operations and maintenance manuals |
| Operator and Water Quality Analyst Training |  |
| 29. | Operator training requirements |
| 30. | Transition, operator training |
| 31. | Water quality analyst training requirements |
| 32. | Transition, water quality analyst training |
| Schedule 1 | Municipal residential subsystem classification |
| Schedule 2 | Qualifications for operators’ certificates |
| Schedule 3 | Qualifications for water quality analysts’ certificates |
| Schedule 4 | Qualifications for renewal of certificates |

Definitions

Definitions

1.  (1)  In this Regulation,

Note:  On January 1, 2011, subsection (1) is amended by adding the following definition:

“authorizing certificate” has the same meaning as in subsection 2 (1) of the Ontario Labour Mobility Act, 2009;

See:  O. Reg. 466/10, ss. 1 (1), 3.

“distribution and supply subsystem” means a type of municipal residential system that is a groundwater system that distributes and treats water, where the treatment is limited to disinfection only, but does not include a drinking water system where the system is deemed to be a drinking water system that obtains water from a raw water supply that is surface water under section 2 of Ontario Regulation 170/03 (Drinking Water Systems);

“distribution subsystem” means a type of municipal residential system that is used to supply or distribute water, but does not include that part of the system that collects, produces or treats water;

“licensed engineering practitioner” means a person who holds a licence, limited licence or temporary licence under the Professional Engineers Act;

“limited groundwater subsystem” means a type of limited system where the raw water is groundwater, but does not include,

(a)	a limited system to which Ontario Regulation 170/03 (Drinking Water Systems) applies, if the system is deemed, under section 2 of that regulation, to be a drinking water system that obtains water from a raw water supply that is surface water;

“limited subsystem” means a limited groundwater subsystem or a limited surface water subsystem;

“limited surface water subsystem” means a type of limited system where,

(a)	the raw water supply is surface water, or

(b)	Ontario Regulation 170/03 (Drinking Water Systems) applies to the limited system and the limited system is deemed, under section 2 of that regulation, to be a drinking water system that obtains water from a raw water supply that is surface water;

“limited system” means a drinking water system that is,

(a)	a small municipal residential system, as defined in Ontario Regulation 170/03 (Drinking Water Systems), if the raw water supply is groundwater, unless the groundwater is deemed to be surface water under section 2 of that regulation,

(b)	a non-municipal year-round residential system as defined in Ontario Regulation 170/03 (Drinking Water Systems), or

(c)	a large municipal non-residential system or large non-municipal non-residential system, as defined in Ontario Regulation 170/03 (Drinking Water Systems), if,

(i)	Ontario Regulation 170/03 applies to the system;

“municipal residential subsystem” means a distribution subsystem, distribution and supply subsystem or water treatment subsystem;

“municipal residential system” means a drinking water system that is,

(a)	a large municipal residential system, within the meaning of section 1 of Ontario Regulation 170/03 (Drinking Water Systems), or

(b)	a small municipal residential system, within the meaning of section 1 of Ontario Regulation 170/03 (Drinking Water Systems) where the raw water supply is surface water or where the raw water supply is deemed to be surface water under section 2 of that regulation;

Note:  On January 1, 2011, subsection (1) is amended by adding the following definition:

“occupation” has the same meaning as in subsection 2 (1) of the Ontario Labour Mobility Act, 2009;

See:  O. Reg. 466/10, ss. 1 (1), 3.

“operator” means a person who conducts operational checks of or who adjusts, tests or evaluates a process that controls the effectiveness or efficiency of a subsystem and includes a person who adjusts or directs the flow, pressure or quality of the water within the subsystem, if the person works in a distribution subsystem or a distribution and supply subsystem;

“operator-in-charge” means an operator or licensed engineering practitioner who is designated as an operator-in-charge of a subsystem under section 25;

Note:  On January 1, 2011, subsection (1) is amended by adding the following definition:

“out-of-province regulatory authority” has the same meaning as in subsection 2 (1) of the Ontario Labour Mobility Act, 2009;

See:  O. Reg. 466/10, ss. 1 (1), 3.

“overall responsible operator” means an operator designated as overall responsible operator of a subsystem under section 23;

Note:  On January 1, 2011, subsection (1) is amended by adding the following definitions:

“regulated occupation” has the same meaning as in subsection 2 (1) of the Ontario Labour Mobility Act, 2009;

“restricted certificate”, in relation to an occupation, means a certificate, licence, registration, or other form of official recognition that is granted by a regulatory authority to an individual who is not qualified to practise the occupation but is permitted with conditions to practise the occupation;

See:  O. Reg. 466/10, ss. 1 (1), 3.

“subsystem” means a distribution subsystem, distribution and supply subsystem, water treatment subsystem, limited groundwater subsystem or limited surface water subsystem;

“water quality analyst” means a person who holds a water quality analyst’s certificate issued under section 16 or who holds a conditional water quality analyst’s certificate issued under section 17;

Note:  On January 1, 2011, the definition of “water quality analyst” is amended by striking out “issued under section 16” and substituting “issued under section 16 or 16.1”.  See:  O. Reg. 466/10, ss. 1 (2), 3.

“water treatment subsystem” means a type of a municipal residential system that collects, produces or treats water but does not include that part of the system that is a distribution subsystem or distribution and supply subsystem.  O. Reg. 128/04, s. 1 (1); O. Reg. 256/05, s. 1; O. Reg. 323/08, s. 1; O. Reg. 415/09, s. 1.

(2)  For the purpose of subsections 12 (2) and (3) of the Act,

“a valid operator’s licence” means a valid drinking water system operator’s licence.  O. Reg. 128/04, s. 1 (2).

Classification of Municipal Residential Systems

Subsystems and classes of municipal residential systems

2.  (1)  For the purposes of this Regulation, municipal residential systems are divided into the following types of subsystems:

1.	Distribution.

2.	Distribution and supply.

3.	Water Treatment.  O. Reg. 128/04, s. 2 (1).

(2)  Each type of subsystem is divided into Class I, Class II, Class III and Class IV subsystems.  O. Reg. 128/04, s. 2 (2).

Certificate of classification

3.  (1)  The owner of a municipal residential subsystem shall file an application with the Director for the determination of the type and class of the subsystem.  O. Reg. 128/04, s. 3 (1).

(2)  The Director shall determine the type and class of the subsystem in accordance with the Tables set out in Schedule 1 and shall issue to the owner a certificate of classification for the subsystem, if the required fee is paid.  O. Reg. 128/04, s. 3 (2).

(3)  If a subsystem is to be replaced or altered, the owner of the subsystem shall apply for a redetermination of the type and class of the subsystem when approval of the alteration is applied for under subsection 32 (1) or (2) of the Act.  O. Reg. 128/04, s. 3 (3).

(4)  The Director may require the owner of a subsystem that has been typed and classified under this section to apply for redetermination of its type and class if,

(a)	section 2 or Schedule 1 is amended; or

(b)	the Director is of the opinion that the subsystem or any type of subsystem no longer meets the criteria under which it has been typed and classified.  O. Reg. 128/04, s. 3 (4).

(5)  The owner or operating authority of a subsystem shall ensure that the certificate of classification of the subsystem is conspicuously displayed at the operator’s workplace or at the premises from which the subsystem is managed.  O. Reg. 128/04, s. 3 (5).

Existing Certificates of Classification

4.  (1)  A certificate of classification issued to the owner of a water distribution facility under section 4 of Ontario Regulation 435/93 for that water distribution facility under the Ontario Water Resources Act that is a valid certificate on August 1, 2004 is deemed to be a certificate of classification for a distribution and supply subsystem under this Regulation until the earlier of August 1, 2005 and the day that there is a redetermination of the type and class of the subsystem under section 3 of this Regulation.  O. Reg. 128/04, s. 4 (1).

(2)  A certificate of classification issued to the owner of a water treatment facility under section 4 of Ontario Regulation 435/93 for that water treatment facility under the Ontario Water Resources Act that is a valid certificate on August 1, 2004 is deemed to be a certificate of classification for a water treatment subsystem under this Regulation until the earlier of August 1, 2005 and the day that there is a redetermination of the type and class of the subsystem under section 3 of this Regulation.  O. Reg. 128/04, s. 4 (2).

Types of Limited Systems

Subsystems of limited systems

5.  For the purposes of this Regulation, limited systems are divided into the following types of drinking water subsystems:

1.	Limited groundwater subsystems.

2.	Limited surface water subsystems.  O. Reg. 128/04, s. 5.

Certification of Operators

Classes of operators’ certificates

6.  (1)  For each type of municipal residential subsystem, there are four classes of operators’ certificates, designated as Class I, Class II, Class III and Class IV.  O. Reg. 128/04, s. 6 (1).

(2)  There is also a class of operators’ certificates for limited groundwater subsystems and for limited surface-water subsystems.  O. Reg. 128/04, s. 6 (2).

(3)  There is also a class of operators’ certificates for operators-in-training for each type of municipal residential subsystem.  O. Reg. 128/04, s. 6 (3).

Municipal residential operators’ certificates — Classes I, II, III and IV

7.  (1)  An individual may apply to the Director for the issuance of an operator’s certificate described in subsection 6 (1).  O. Reg. 128/04, s. 7 (1).

(2)  Subject to the requirements in this section, the Director shall issue the certificate if the applicant meets the qualifications set out in Schedule 2 for that type and class of certificate and the required fee is paid.  O. Reg. 128/04, s. 7 (2).

(3)  The Director may refuse to issue a certificate if,

(a)	any of the circumstances described in subsection 13 (1) apply;

(b)	the applicant is the holder of any water operator’s licence or certificate that has been revoked or suspended or is the holder of any other certificate that is suspended or that the Director is authorized under subsection 13 (1) or section 19 to revoke or suspend; or

(c)	the applicant is the holder of a wastewater operator’s licence issued under Ontario Regulation 129/04 that is suspended or the Director is authorized to revoke or suspend.  O. Reg. 128/04, s. 7 (3).

(4)  An operator’s certificate expires three years after it is issued or, as provided in subsection (7), on an earlier date specified on the certificate but an individual may apply to the Director before it expires to have the certificate renewed.  O. Reg. 128/04, s. 7 (4).

(5)  Subject to the requirements in this section, the Director shall renew a certificate if the applicant meets the qualifications set out in section 2 of Schedule 4 for that type and class of certificate and the required fee is paid.  O. Reg. 128/04, s. 7 (5).

(6)  Despite subsection (5), the Director may renew a certificate even if the applicant failed to complete the training requirements set out in section 29, if the Director is satisfied that the applicant will do so before the expiry of the renewed certificate.  O. Reg. 128/04, s. 7 (6).

(7)  A certificate renewed under subsection (6) expires on the date set out on the certificate, which may be any date that is less than six months after it is renewed.  O. Reg. 128/04, s. 7 (7).

(8)  The Director may refuse to renew a certificate if,

(a)	any of the circumstances described in subsection 13 (1) apply;

(b)	the applicant is the holder of any other certificate that has been revoked or suspended or that the Director is authorized under subsection 13 (1) or section 19 to revoke or suspend;

(c)	the certificate to be renewed was revoked or suspended under subsection 13 (1); or

(d)	the applicant is the holder of a wastewater operator’s licence issued under Ontario Regulation 129/04 that was revoked or is suspended or that the Director is authorized to revoke or suspend.  O. Reg. 128/04, s. 7 (8).

(9)  A valid distribution operator’s licence that is continued as an operator’s certificate pursuant to subsection 12 (2) or (3) of the Act is deemed to be a distribution and supply operator’s certificate of the same class as the class of licence held under Ontario Regulation 435/93 under the Ontario Water Resources Act until the certificate is suspended, revoked or expires.  O. Reg. 128/04, s. 7 (9).

(10)  A valid water treatment operator’s licence that is continued as an operator’s certificate pursuant to subsection 12 (2) or (3) of the Act is deemed to be a municipal residential operator’s certificate of the same type and class as the type and class of licence held under Ontario Regulation 435/93 under the Ontario Water Resources Act until the certificate is suspended, revoked or expires.  O. Reg. 128/04, s. 7 (10).

(11)  An individual may apply under subsection (4) to the Director to have his or her operator’s licence that is continued as a certificate under subsection 12 (2) of the Act renewed and subsections (5) and (8) apply to the renewal.  O. Reg. 128/04, s. 7 (11).

(12)  An individual may apply to the Director to have his or her operator’s licence that is continued as a certificate pursuant to subsection 12 (3) of the Act renewed and the Director shall renew the certificate if,

(a)	the applicant pays the required fee;

(b)	the applicable requirements set out in section 1 of Schedule 4 are met; and

(c)	there is no basis under subsection (8) for the Director to refuse to renew the certificate.  O. Reg. 128/04, s. 7 (12).

(13)  When a certificate that is renewed under subsection (12) expires, if the individual has obtained a mark that the Director considers satisfactory in an examination approved by the Director, the individual may apply under subsection (4) to have the certificate renewed and subsections (5) and (8) apply to the renewal.  O. Reg. 128/04, s. 7 (13).

Note:  On January 1, 2011, the Regulation is amended by adding the following section:

Municipal residential operator’s certificate, out-of-province applicant

7.1  (1)  A person who holds an authorizing certificate issued by an out-of-province regulatory authority in respect of an occupation may apply to the Director for the issuance of an operator’s certificate described in subsection 6 (1) in respect of the same occupation.  O. Reg. 466/10, s. 2.

(2)  Subject to subsections (3) and (4), the Director shall issue an operator’s certificate described in subsection 6 (1) to the applicant if,

(a)	the applicant pays the required fee;

(b)	the applicant provides the Director with a copy of the applicant’s authorizing certificate;

(c)	the operator’s certificate is, in the opinion of the Director, in respect of the same occupation as the applicant’s authorizing certificate;

(d)	the applicant provides the Director with confirmation in writing from the out-of-province regulatory authority that the applicant’s authorizing certificate is in good standing; and

(e)	the applicant provides the Director with evidence satisfactory to the Director that the applicant has practised the regulated occupation within the three-year period preceding the day on which the applicant submits his or her application under subsection (1).  O. Reg. 466/10, s. 2.

(3)  An applicant who applies for a Class I operator’s certificate shall be issued the certificate under subsection (2) only if the applicant complies with the training requirements set out in section 29.  O. Reg. 466/10, s. 2.

(4)  The Director may refuse to issue a certificate if the out-of-province regulatory authority has revoked or suspended or circumstances exist that authorize the authority to revoke or suspend an authorizing certificate or a restricted certificate that was issued to the applicant in respect of one or more of the following occupations:

1.	Wastewater operator.

2.	Water operator.

3.	Water quality analyst.  O. Reg. 466/10, s. 2.

(5)  Subsections 7 (4) to (13) apply to an operator’s certificate issued by the Director under this section.  O. Reg. 466/10, s. 2.

See:  O. Reg. 466/10, ss. 2, 3.

Limited subsystem operators’ certificates

8.  (1)  An individual may apply to the Director for the issuance of an operator’s certificate described in subsection 6 (2).  O. Reg. 128/04, s. 8 (1).

(2)  Subject to the requirements in this section, the Director shall issue the certificate if the applicant meets the qualifications set out in Schedule 2 for that type and class of certificate and the required fee is paid.  O. Reg. 128/04, s. 8 (2).

(3)  Despite subsection (2), the Director may issue a certificate to an applicant for a limited subsystem operator’s certificate who has not successfully completed Grade 12 in Ontario or does not have education qualifications that the Director considers equivalent if the Director would have been authorized to issue the certificate to the applicant under this Regulation as it read immediately before the day subsection 1 (8) of Schedule D to the Health System Improvements Act, 2007 came into force.  O. Reg. 323/08, s. 2.

(4)  The Director may refuse to issue a certificate if,

(a)	any of the circumstances described in subsection 13 (1) apply;

(b)	the applicant is the holder of any water operator’s licence or certificate that has been revoked or suspended or is the holder of any other certificate that is suspended or that the Director is authorized under subsection 13 (1) or section 19 to revoke or suspend; or

(c)	the applicant is the holder of a wastewater operator’s licence issued under Ontario Regulation 129/04 that is suspended or the Director is authorized to revoke or suspend.  O. Reg. 128/04, s. 8 (4).

(5)  A limited subsystem operator’s certificate expires three years after it is issued or, as provided in subsection (7), on an earlier date specified on the certificate, but may be renewed before it expires if the requirements in section 2 of Schedule 4 are met and the applicant pays the required fee.  O. Reg. 128/04, s. 8 (5).

(6)  Despite subsection (5), the Director may renew a certificate even if the applicant failed to complete the training requirements set out in section 29, if the Director is satisfied that the applicant will do so before the expiry of the renewed certificate.  O. Reg. 128/04, s. 8 (6).

(7)  A certificate renewed under subsection (6) expires on the date set out on the certificate, which may be any date that is less than six months after it is renewed.  O. Reg. 128/04, s. 8 (7).

(8)  The Director may refuse to renew a certificate if,

(a)	any of the circumstances described in subsection 13 (1) apply;

(b)	the applicant is the holder of any other certificate that has been revoked or suspended or that the Director is authorized under subsection 13 (1) or section 19 to revoke or suspend;

(c)	the certificate to be renewed was revoked or suspended under subsection 13 (1); or

(d)	the applicant is the holder of a wastewater operator’s licence issued under Ontario Regulation 129/04 that was revoked or is suspended or that the Director is authorized to revoke or suspend.  O. Reg. 128/04, s. 8 (8).

Note:  On January 1, 2011, the Regulation is amended by adding the following section:

Limited subsystem operator’s certificate, out-of-province applicant

8.1  (1)  A person who holds an authorizing certificate issued by an out-of-province regulatory authority in respect of an occupation may apply to the Director for the issuance of an operator’s certificate described in subsection 6 (2) in respect of the same occupation.  O. Reg. 466/10, s. 2.

(2)  Subject to subsection (3), the Director shall issue an operator’s certificate described in subsection 6 (2) to the applicant if,

(a)	the applicant pays the required fee;

(b)	the applicant provides the Director with a copy of the applicant’s authorizing certificate;

(c)	the operator’s certificate is, in the opinion of the Director, in respect of the same occupation as the applicant’s authorizing certificate;

(d)	the applicant provides the Director with confirmation in writing from the out-of-province regulatory authority that the applicant’s authorizing certificate is in good standing; and

(e)	the applicant provides the Director with evidence satisfactory to the Director that he or she has practised the regulated occupation within the five-year period preceding the day on which the applicant submits his or her application under subsection (1).  O. Reg. 466/10, s. 2.

(3)  The Director may refuse to issue a certificate if the out-of-province regulatory authority has revoked or suspended or circumstances exist that authorize the authority to revoke or suspend an authorizing certificate or a restricted certificate that was issued to the applicant in respect of one or more of the following occupations:

1.	Wastewater operator.

2.	Water operator.

3.	Water quality analyst.  O. Reg. 466/10, s. 2.

(4)  Subsections 8 (5) to (8) apply with necessary modifications to an operator’s certificate issued under this section.  O. Reg. 466/10, s. 2.

See:  O. Reg. 466/10, ss. 2, 3.

Operator-in-training’s certificates

9.  (1)  An individual may apply to the Director for the issuance of an operator-in-training’s certificate.  O. Reg. 128/04, s. 9 (1).

(2)  Subject to the requirements in this section, the Director shall issue the certificate if the applicant meets the qualifications set out in subsection 1 (1) of Schedule 2 for that type of certificate and the required fee has been paid.  O. Reg. 128/04, s. 9 (2).

(3)  Despite subsection (2), the Director may issue a certificate even if the applicant does not meet the qualifications set out in paragraph 3 of subsection 1 (1) of Schedule 2 when the applicant applies.  O. Reg. 128/04, s. 9 (3).

(4)  The Director may refuse to issue a certificate if,

(a)	any of the circumstances described in subsection 13 (1) apply;

(b)	the applicant is the holder of any other certificate that has been revoked or suspended or that the Director is authorized under subsection 13 (1) or section 19 to revoke or suspend; or

(c)	the applicant has previously held an operator-in-training’s certificate and the applicant either failed to successfully complete the course of study approved by the Director related to the functions performed by operators-in-training or failed to obtain a mark in the course that the Director considers satisfactory.  O. Reg. 128/04, s. 9 (4).

(5)  Subject to subsections (6), (7), (8) and (9), an operator-in-training’s certificate expires 16 months after it is issued.  O. Reg. 128/04, s. 9 (5).

(6)  If at the time of his or her application for an operator-in-training’s certificate the applicant has already satisfied the requirements set out in paragraph 3 of subsection 1 (1) of Schedule 2, the certificate expires three years after it is issued.  O. Reg. 128/04, s. 9 (6).

(7)  An operator-in-training’s certificate issued to a person described in subsection 1 (2) of Schedule 2 expires 12 months after it was issued.  O. Reg. 128/04, s. 9 (7).

(8)  The Director may extend the expiry date of an operator-in-training’s certificate,

(a)	to a date that is three years after it was first issued, if the holder of an operator-in-training’s certificate satisfies the requirements of paragraph 3 of subsection 1 (1) of Schedule 2 within 16 months after obtaining his or her operator-in-training’s certificate; or

(b)	to a date that is six months after the expiry date, if the Director is satisfied that the holder will satisfy the requirements of paragraph 3 of subsection 1 (1) of Schedule 2 before the extended expiry date.  O. Reg. 128/04, s. 9 (8).

(9)  The Director may extend the expiry date of an operator-in-training’s certificate to a date that is three years after it was first issued, if the certificate has been extended under clause (8) (b) and the individual meets the requirements referenced in clause (8) (b) within the time period set out in that clause.  O. Reg. 128/04, s. 9 (9).

(10)  The Director may renew the certificate of a person described in subsection 1 (2) of Schedule 2 but shall not renew it more than four times.  O. Reg. 128/04, s. 9 (10).

(11)  If an operator-in-training has met the requirements of subsection 1 (1) of Schedule 2 but has been unable to obtain at least one year of experience as an operator-in-training in a municipal residential subsystem before the certificate expires, the Director may renew the certificate of the operator-in-training for one three-year period.  O. Reg. 128/04, s. 9 (11).

(12)  An individual who holds an operator-in-training licence issued under Ontario Regulation 435/93 under the Ontario Water Resources Act that expires on or after August 1, 2004 and before August 1, 2006 may renew his or her operator-in-training’s certificate under this Regulation and the certificate shall be renewed if the individual pays the required fee and meets the requirements of Ontario Regulation 435/93 as it read on July 31, 2004.  O. Reg. 128/04, s. 9 (12).

(13)  A certificate renewed under subsection (12) expires on August 1, 2006.  O. Reg. 128/04, s. 9 (13).

(14)  An individual who holds an operator-in-training licence issued under Ontario Regulation 435/93 under the Ontario Water Resources Act that expires on or after August 1, 2006 and before August 1, 2007 may renew his or her operator-in-training’s certificate under this Regulation and the certificate shall be renewed if the individual pays the required fee and meets the requirements of Ontario Regulation 435/93 as it read on July 31, 2004.  O. Reg. 128/04, s. 9 (14).

(15)  A certificate renewed under subsection (14) expires on August 1, 2007.  O. Reg. 128/04, s. 9 (15).

(16)  Except as provided under subsections (10), (11), (12) and (14), an operator-in-training’s certificate issued under this Regulation shall not be renewed.  O. Reg. 128/04, s. 9 (16).

Conditional operators’ certificates

10.  (1)  An individual may apply to the Director for the issuance of a conditional operator’s certificate for each type and class of operator’s certificate that may be issued under section 7 or 8.  O. Reg. 128/04, s. 10 (1).

(2)  An individual whose operator’s licence was continued as a certificate pursuant to subsection 12 (3) of the Act may not apply for a conditional operator’s certificate under subsection (1) unless the individual has met the requirements of subsection 1 (3) or (4) of Schedule 4.  O. Reg. 128/04, s. 10 (2).

(3)  The Director may issue the conditional certificate if,

(a)	the Director is satisfied that,

(i)	the owner or operating authority of a subsystem employs or has offered to employ the applicant, or

(ii)	the owner or operator of a small drinking water system within the meaning of the Health Protection and Promotion Act employs or has offered to employ the applicant;

(b)	the Director is satisfied that the owner, operating authority or operator referred to in clause (a) cannot readily obtain the services of an operator who holds the type and class of certificate otherwise required under this Regulation or under Ontario Regulation 318/08 (Transitional — Small Drinking Water Systems) or Ontario Regulation 319/08 (Small Drinking Water Systems) made under the Health Protection and Promotion Act;

(b.1)	the owner, operating authority or operator referred to in clause (a) gives the applicant and the Director an undertaking in writing to co-operate in facilitating the applicant’s compliance with any conditions imposed under subsection (4); and

(c)	the required fee is paid.  O. Reg. 128/04, s. 10 (3); O. Reg. 323/08, s. 3 (1).

(4)  The Director may issue a conditional certificate subject to conditions.  O. Reg. 128/04, s. 10 (4).

(5)  A conditional certificate is valid only in respect of,

(a)	the subsystem referred to in subclause (3) (a) (i); or

(b)	the small drinking water system referred to in subclause (3) (a) (ii).  O. Reg. 323/08, s. 3 (2).

(6)  The Director may refuse to issue a conditional certificate if,

(a)	any of the circumstances described in subsection 13 (1) apply;

(b)	the applicant is the holder of any water operator’s licence or certificate that has been revoked or suspended or is the holder of any other certificate that is suspended or that the Director is authorized under subsection 13 (1) or section 19 to revoke or suspend; or

(c)	the applicant is the holder of a wastewater operator’s licence issued under Ontario Regulation 129/04 that is suspended or the Director is authorized to revoke or suspend.  O. Reg. 128/04, s. 10 (6).

(7)  A conditional certificate expires three years after it is issued or on such earlier date as may be specified on the certificate, but may be renewed before it expires in accordance with the requirements set out in section 3 of Schedule 4 if the applicant pays the required fee.  O. Reg. 128/04, s. 10 (7).

(8)  The Director may refuse to renew the certificate if,

(a)	any of the circumstances described in subsection 13 (1) apply;

(b)	the applicant is the holder of any other certificate that has been revoked or suspended or that the Director is authorized under subsection 13 (1) or section 19 to revoke or suspend;

(c)	the certificate to be renewed was revoked or suspended under subsection 13 (1); or

(d)	the applicant is the holder of a wastewater operator’s licence issued under Ontario Regulation 129/04 that was revoked or is suspended or that the Director is authorized to revoke or suspend.  O. Reg. 128/04, s. 10 (8).

Reissuance of certificate after expiry

11.  (1)  Despite subsections 7 (4) and 8 (5), if a person fails to apply for renewal of an operator’s certificate described in subsection 6 (1) or (2) before the certificate has expired, the person may apply for the reissuance of the certificate and the Director shall reissue the certificate if the person meets the following requirements:

1.	If the person’s certificate expired more than one year before the application for reissuance, the person must,

i.	meet the qualifications set out in Schedule 2 for that type and class of certificate, except the Director may waive the requirement that the applicant have a class and type of operator’s certificate that is one below the class and type for which the applicant is applying if the Director is satisfied that the applicant previously held a valid certificate of the required type and class,

ii.	complete the approved mandatory training course referred to in subsection 29 (6), and

iii.	pay the required fees.

2.	If the person’s certificate expired one year or less before the application for reissuance, the person must,

i.	have completed the training requirements set out in section 29 since the certificate was last issued or renewed,

ii.	have at least three months experience in the previous 36 months working as an operator in a subsystem or having duties which the Director considers related to the duties of an operator in a subsystem, and

iii.	pay the required fees.  O. Reg. 128/04, s. 11 (1).

(2)  Subparagraph 1 ii of subsection (1) applies despite paragraph 2 of subsection 29 (3).  O. Reg. 128/04, s. 11 (2).

Transferability of certificates

12.  (1)  A person who holds a Class I, Class II, Class III or Class IV water treatment subsystem operator’s certificate is deemed to also hold a Class I distribution subsystem operator’s certificate and a Class I distribution and supply subsystem operator’s certificate.  O. Reg. 128/04, s. 12 (1).

(2)  A person who holds a Class I, Class II, Class III or Class IV distribution and supply subsystem operator’s certificate is deemed to also hold a distribution subsystem operator’s certificate of the same class.  O. Reg. 128/04, s. 12 (2).

(3)  A person who holds a Class I, Class II, Class III or Class IV water treatment subsystem operator’s certificate is deemed to also hold a limited groundwater subsystem operator’s certificate and a limited surface water subsystem operator’s certificate.  O. Reg. 128/04, s. 12 (3).

(4)  A person who holds a limited surface water subsystem operator’s certificate is deemed to also hold a limited groundwater subsystem operator’s certificate.  O. Reg. 128/04, s. 12 (4).

(5)  A person who holds a Class I, Class II, Class III or Class IV distribution and supply subsystem operator’s certificate is deemed to also hold a limited groundwater subsystem operator’s certificate.  O. Reg. 128/04, s. 12 (5).

Revocation or suspension of certificate

13.  (1)  The Director may revoke or suspend an operator’s certificate, an operator-in-training’s certificate or a conditional operator’s certificate, if one or more of the following circumstances exist:

1.	The application was fraudulent or contained inaccurate information.

2.	The person has been discharged from employment in a drinking water system for gross negligence or for incompetence in the performance of his or her duties, unless the person has not yet exhausted the rights of appeal available under a collective agreement.

3.	The person has worked as an operator for any length of time without being certified for the type or class of operator he or she worked as, or has held himself or herself out to an owner, operating authority, the Director or any Ministry employee as holding a type or class of operator’s certificate that he or she does not hold.

4.	The person has previously had an operator’s certificate or a water quality analyst’s certificate or an operator’s licence or a wastewater operator’s licence issued under Ontario Regulation 435/93 under the Ontario Water Resources Act or a wastewater operator’s licence issued under Ontario Regulation 129/04 revoked or suspended for any reason and the Director has reasonable grounds to believe that the person is not competent to be an operator.

5.	The person has contravened section 26 or 27 and the contravention,

i.	resulted in the discharge of a pollutant into the natural environment,

ii.	had an adverse effect on the health or safety of an individual, or

iii.	had an adverse effect on a process in the subsystem or the system of which the subsystem is a part.

6.	The person has failed,

i.	to exercise the level of care, diligence and skill in respect of a drinking water system that a reasonably prudent operator would be expected to exercise in a similar situation, or

ii.	to act honestly, competently and with integrity, with a view to ensuring the protection and safety of the users of a drinking water system.

7.	The person has failed to meet or has contravened any condition that is set out in his or her certificate.  O. Reg. 128/04, s. 13 (1); O. Reg. 323/08, s. 4.

(2)  When a person’s certificate is revoked or suspended, the Director may issue a certificate of another type and class to the person if the person meets the qualifications set out in Schedule 2 for that type and class of certificate.  O. Reg. 128/04, s. 13 (2).

Replacement certificates

14.  (1)  The Director shall issue a replacement operator’s certificate, operator-in-training’s certificate or a conditional operator’s certificate, if the required fee is paid and,

(a)	the operator indicates that his or her certificate has been lost or destroyed; or

(b)	the operator indicates that his or her name has changed and returns the original certificate to the Director.  O. Reg. 128/04, s. 14 (1).

(2)  The Director may refuse to issue a replacement certificate if the operator is the holder of a certificate that is revoked or suspended or that the Director is authorized under subsection 13 (1) or section 19 to revoke or suspend.  O. Reg. 128/04, s. 14 (2).

Certificate to be displayed

15.  The owner or operating authority of a subsystem shall ensure that a copy of the certificate of every certified operator who is employed in the subsystem is conspicuously displayed at the operator’s workplace or at the premises from which the subsystem is managed.  O. Reg. 128/04, s. 15.

Certification of Water Quality Analysts

Water quality analyst’s certificates

16.  (1)  An individual may apply to the Director for the issuance of a water quality analyst’s certificate.  O. Reg. 128/04, s. 16 (1).

(2)  Subject to the requirements in this section, the Director shall issue the certificate if the applicant meets the qualifications set out in Schedule 3 and the required fee has been paid.  O. Reg. 128/04, s. 16 (2).

(3)  The Director may refuse to issue a certificate if,

(a)	any of the circumstances described in section 19 apply;

(b)	the applicant is the holder of any other certificate that has been revoked or suspended or that the Director is authorized under subsection 13 (1) or section 19 to revoke or suspend; or

(c)	the applicant is the holder of a wastewater operator’s licence issued under Ontario Regulation 129/04 that is suspended or the Director is authorized to revoke or suspend.  O. Reg. 128/04, s. 16 (3).

(4)  A water quality analyst’s certificate expires three years after it is issued or, as provided in subsection (7), on an earlier date specified on the certificate but an individual may apply to the Director before it expires to have the certificate renewed.  O. Reg. 128/04, s. 16 (4).

(5)  Subject to the requirements in this section, the Director shall renew a certificate if the applicant meets the qualifications set out in section 4 of Schedule 4 and the required fee is paid.  O. Reg. 128/04, s. 16 (5).

(6)  Despite subsection (4), the Director may renew a certificate even if the applicant failed to complete the training requirements set out in section 31, if the Director is satisfied that the applicant will do so before the expiry of the renewed certificate.  O. Reg. 128/04, s. 16 (6).

(7)  A certificate renewed under subsection (5) expires on the date set out on the certificate, which may be any date that is less than six months after it is renewed.  O. Reg. 128/04, s. 16 (7).

(8)  The Director may refuse to renew a certificate if,

(a)	any of the circumstances described in section 19 apply;

(b)	the applicant is the holder of a certificate that is revoked or suspended or that the Director is authorized under subsection 13 (1) to revoke or suspend;

(c)	the applicant is the holder of a wastewater operator’s licence issued under Ontario Regulation 129/04 that is suspended or the Director is authorized to revoke or suspend; or

(d)	the certificate to be renewed is revoked or suspended under section 19.  O. Reg. 128/04, s. 16 (8).

(9)  An individual who was licensed by the Ministry as a water quality analyst on July 31, 2004 is deemed to hold a water quality analyst’s certificate under this Regulation until the earlier of the day the individual is issued a water quality analyst’s certificate under this section and August 1, 2007.  O. Reg. 128/04, s. 16 (9).

Note:  On January 1, 2011, the Regulation is amended by adding the following section:

Water quality analyst’s certificate, out-of-province applicant

16.1  (1)  A person who holds an authorizing certificate issued by an out-of-province regulatory authority in respect of an occupation may apply to the Director for the issuance of a water quality analyst’s certificate.  O. Reg. 466/10, s. 2.

(2)  Subject to subsection (3), the Director shall issue the water quality analyst’s certificate to the applicant if,

(a)	the applicant pays the required fee;

(b)	the applicant provides the Director with a copy of the applicant’s authorizing certificate;

(c)	the water quality analyst’s certificate is, in the opinion of the Director, in respect of the same occupation as the applicant’s authorizing certificate;

(d)	the applicant provides the Director with confirmation in writing from the out-of-province regulatory authority that the applicant’s authorizing certificate is in good standing; and

(e)	the applicant provides the Director with evidence satisfactory to the Director that he or she has practised the regulated occupation within the five-year period preceding the day on which the applicant submits his or her application under subsection (1).  O. Reg. 466/10, s. 2.

(3)  The Director may refuse to issue a certificate if the out-of-province regulatory authority has revoked or suspended or circumstances exist that authorize the authority to revoke or suspend an authorizing certificate or a restricted certificate that was issued to the applicant for one or more of the following occupations:

1.	Wastewater operator.

2.	Water operator.

3.	Water quality analyst.  O. Reg. 466/10, s. 2.

(4)  Subsections 16 (4) to (9) apply to a water quality analyst’s certificate issued under this section.  O. Reg. 466/10, s. 2.

See:  O. Reg. 466/10, ss. 2, 3.

Conditional water quality analyst’s certificates

17.  (1)  An individual may apply to the Director for the issuance of a conditional water quality analyst’s certificate.  O. Reg. 128/04, s. 17 (1).

(2)  The Director may issue the conditional certificate if,

(a)	the owner or operating authority of the subsystem that employs or has offered to employ the individual who has applied under subsection (1) satisfies the Director that the owner or operating authority cannot readily obtain the services of a certified water quality analyst;

(b)	the owner or operating authority referred to in clause (a) gives the applicant and the Director an undertaking in writing to co-operate in facilitating the applicant’s compliance with any conditions imposed under subsection (3); and

(c)	the required fee is paid.  O. Reg. 128/04, s. 17 (2).

(3)  The Director may issue a conditional certificate subject to conditions.  O. Reg. 128/04, s. 17 (3).

(4)  A conditional certificate is valid in respect of the subsystem referred to in clause (2) (a) and in respect of no other subsystem.  O. Reg. 128/04, s. 17 (4).

(5)  The Director may refuse to issue a conditional certificate if,

(a)	any of the circumstances described in section 19 apply;

(b)	the applicant is the holder of any water operator’s licence or certificate that has been revoked or suspended or is the holder of any other certificate that is suspended or that the Director is authorized under subsection 13 (1) or section 19 to revoke or suspend; or

(c)	the applicant is the holder of a wastewater operator’s licence issued under Ontario Regulation 129/04 that is suspended or the Director is authorized to revoke or suspend.  O. Reg. 128/04, s. 17 (5).

(6)  A conditional certificate expires three years after it is issued or on such earlier date as may be specified on the certificate, but may be renewed before it expires in accordance with section 5 of Schedule 4 if the applicant pays the required fee.  O. Reg. 128/04, s. 17 (6).

(7)  The Director may refuse to renew a conditional certificate if,

(a)	any of the circumstances described in section 19 apply;

(b)	the applicant is the holder of any other certificate that has been revoked or suspended or that the Director is authorized under subsection 13 (1) to revoke or suspend;

(c)	the certificate to be renewed is revoked or suspended under section 19; or

(d)	the applicant is the holder of a wastewater operator’s licence issued under Ontario Regulation 129/04 that was revoked or is suspended or that the Director is authorized to revoke or suspend.  O. Reg. 128/04, s. 17 (7).

Reissuance of certificate after expiry

18.  Despite subsection 16 (4), if a person fails to apply for renewal of a water quality analyst’s certificate before the certificate has expired, the person may apply for the reissuance of the certificate and the Director shall reissue the certificate if the person meets the following requirements:

1.	If the person’s certificate expired more than one year before the application for reissuance, the person must,

i.	meet the qualifications set out in Schedule 3,

ii.	complete the approved mandatory training course referred to in subsection 31 (6), and

iii.	pay the required fees.

2.	If the person’s certificate expired one year or less before the application for reissuance, the person must,

i.	have completed the training requirements set out in section 31 since the certificate was last issued or renewed,

ii.	have at least three months experience in the previous 36 months working as an operator in a subsystem or having duties which the Director considers related to the duties of an operator in a subsystem, and

iii.	pay the required fees.  O. Reg. 128/04, s. 18.

Revocation or suspension of certificate

19.  The Director may revoke or suspend a water quality analyst’s certificate if one or more of the following circumstances exist:

1.	The application was fraudulent or contained inaccurate information.

2.	The person has been discharged from employment in a subsystem for gross negligence or for incompetence in the performance of his or her duties, unless the person has not yet exhausted the rights of appeal available under a collective agreement.

3.	The person has worked as a water quality analyst for any length of time without being certified as such, or has held himself or herself out to an owner, operating authority, the Director or any Ministry employee as holding a water quality analyst’s certificate that he or she does not hold.

4.	The person has previously had a water quality analyst’s certificate revoked or suspended for any reason and the Director has reasonable grounds to believe that the person is not competent to carry out tests at a drinking water system for the parameters listed in subsection 2 (1) of Ontario Regulation 248/03 under the Act.

5.	The person has failed,

i.	to exercise the level of care, diligence and skill in respect of a municipal residential system or limited system that a reasonably prudent water quality analyst would be expected to exercise in a similar situation, or

ii.	to act honestly, competently and with integrity, with a view to ensuring the protection and safety of the users of the municipal residential system or limited system.

6.	The person has failed to meet or has contravened any condition that is set out in his or her certificate.  O. Reg. 128/04, s. 19.

Replacement certificates

20.  (1)  The Director shall issue a replacement water quality analyst’s certificate to a water quality analyst if the required fee is paid and,

(a)	the water quality analyst indicates that his or her certificate has been lost or destroyed; or

(b)	the water quality analyst indicates that his or her name has changed and returns the original certificate to the Director.  O. Reg. 128/04, s. 20 (1).

(2)  The Director may refuse to issue a replacement certificate if the water quality analyst is the holder of a certificate that is revoked or suspended or that the Director is authorized under section 19 to revoke or suspend.  O. Reg. 128/04, s. 20 (2).

Certificate to be displayed

21.  The owner or operating authority of a subsystem shall ensure that a copy of the certificate of every certified water quality analyst who is employed in the subsystem is conspicuously displayed at the analyst’s workplace or at the premises from which the subsystem is managed.  O. Reg. 128/04, s. 21.

Operating Standards for Municipal Residential Subsystems and Limited Subsystems

Owner or operating authority responsibility

22.  The owner or operating authority of a subsystem shall ensure that every operator employed in the subsystem holds,

(a)	a certificate applicable to that type of subsystem; or

(b)	a certificate applicable to that subsystem, in the case of an operator who holds a conditional certificate issued or renewed under section 10.  O. Reg. 128/04, s. 22.

Overall responsible operator

23.  (1)  The owner or operating authority of a municipal residential subsystem shall designate as overall responsible operator of the subsystem an operator who holds a certificate for that type of subsystem and that is of the same class as or higher than the class of that subsystem.  (For example, the overall responsible operator of a Class III water treatment subsystem must be an operator who holds a Class III or Class IV water treatment subsystem operator’s certificate.)  O. Reg. 128/04, s. 23 (1).

(2)  The owner or operating authority of a limited subsystem shall designate as overall responsible operator of the subsystem an operator who holds a limited subsystem operator’s certificate for that type of subsystem.  O. Reg. 128/04, s. 23 (2).

(3)  If the overall responsible operator designated under subsection (1) or (2) is a person whose valid operator’s licence is continued as an operator’s certificate pursuant to subsection 12 (3) of the Act, the owner or operating authority shall revoke the person’s designation as overall responsible operator unless the person,

(a)	takes an examination approved by the Director by May 14, 2005; and

(b)	obtains a mark the Director considers satisfactory on the examination.  O. Reg. 128/04, s. 23 (3).

(4)  If the overall responsible operator designated under subsection (1) or (2) is absent or unable to act, the owner or operating authority or, if the owner or operating authority authorizes it, the overall responsible operator may designate an operator who holds a certificate that is applicable to that type of subsystem and, if applicable, that is not more than one class lower than the class of the subsystem to act in the place of the overall responsible operator.  (For example, if the overall responsible operator is absent or unable to act, responsibility for the overall operation of a Class IV distribution subsystem may be delegated to an operator who holds a Class III distribution subsystem operator’s certificate.)  O. Reg. 128/04, s. 23 (4).

(5)  Subsection (4) does not permit delegating an overall responsible operator’s duties,

(a)	to an operator who holds an operator-in-training’s certificate; or

(b)	to an operator whose designation as overall responsible operator was revoked under subsection (3).  O. Reg. 128/04, s. 23 (5).

(6)  Subsection (4) shall not be relied on by the owner or operating authority of a municipal residential subsystem for more than 150 days in any 12-month period.  O. Reg. 128/04, s. 23 (6).

(7)  The Director may direct that subsection (6) not apply to a municipal residential subsystem for a time period specified by the Director, if the Director is satisfied that the owner or operating authority of the subsystem cannot reasonably comply with subsection (1) and the direction will not result in a drinking water hazard or a significant risk to the natural environment.  O. Reg. 128/04, s. 23 (7).

Strikes and lock-outs

24.  (1)  In the event of a strike or lock-out involving operators employed in a subsystem, the Director may direct that sections 22 and 23 not apply to the subsystem for the duration of the strike or lock-out, if the Director is satisfied that the subsystem will be operated without a significant risk to human health or the natural environment.  O. Reg. 128/04, s. 24 (1).

(2)  In the event of a strike or lock-out involving operators employed in a subsystem, the Director may exempt the owner or operating authority of the subsystem from complying with subsection 12 (1) of the Act for the duration of the strike or lock-out, if the Director is satisfied that the subsystem will be operated without a significant risk to human health or the natural environment.  O. Reg. 128/04, s. 24 (2).

Operator-in-charge

25.  (1)  The owner or operating authority of a subsystem or a person authorized by the owner or operating authority shall designate one or more operators as operators-in-charge of the subsystem.  O. Reg. 128/04, s. 25 (1).

(2)  The owner or operating authority or a person authorized by the owner or operating authority shall ensure that records are maintained of the amount of time each operator works as an operator-in-charge.  O. Reg. 128/04, s. 25 (2).

(3)  The owner or operating authority may designate a licensed engineering practitioner who does not have an operator’s certificate as an operator-in-charge.  O. Reg. 128/04, s. 25 (3); O. Reg. 415/09, s. 2 (1).

(4)  An owner or operating authority shall not use the power to designate a licensed engineering practitioner or a series of licensed engineering practitioners as an operator-in-charge under subsection (3) for more than 180 days in total in any 24-month period.  O. Reg. 128/04, s. 25 (4); O. Reg. 415/09, s. 2 (2).

(5)  A person who holds an operator-in-training’s certificate shall not be designated as an operator-in-charge.  O. Reg. 128/04, s. 25 (5).

Duties of operator-in-charge

26.  (1)  An operator-in-charge is authorized to,

(a)	set operational parameters for the subsystem or for a process that controls the effectiveness or efficiency of the subsystem; and

(b)	direct or instruct other operators in the subsystem to set such operational parameters.  O. Reg. 128/04, s. 26 (1).

(2)  An operator-in-charge shall,

(a)	take all steps reasonably necessary to operate the processes within his or her responsibility in a safe and efficient manner in accordance with the relevant operations manuals;

(b)	ensure that the processes within his or her responsibility are measured, monitored, sampled and tested in a manner that permits them to be adjusted when necessary;

(c)	ensure that records are maintained of all adjustments made to the processes within his or her responsibility; and

(d)	ensure that all equipment used in the processes within his or her responsibility is properly monitored, inspected, tested and evaluated and that records of equipment operating status are prepared and available at the end of every operating shift.  O. Reg. 128/04, s. 26 (2).

Record-keeping re operation of subsystem

27.  (1)  The owner or operating authority of a subsystem shall ensure that logs or other record-keeping mechanisms are provided to record information concerning the operation of the subsystem.  O. Reg. 128/04, s. 27 (1).

(2)  Entries in the logs or other record-keeping mechanisms shall be made chronologically.  O. Reg. 128/04, s. 27 (2).

(3)  No person shall make an entry in a log or other record-keeping mechanism unless the person is an overall responsible operator, an operator-in-charge or is authorized to make an entry by the owner, the operating authority, the overall responsible operator or an operator-in-charge.  O. Reg. 128/04, s. 27 (3).

(4)  A person who makes an entry in a log or other record-keeping mechanism shall do so in a manner that permits the person to be unambiguously identified as the maker of the entry.  O. Reg. 128/04, s. 27 (4).

(5)  An operator-in-charge or a person authorized by an operator-in-charge shall record the following information in the logs or other record-keeping mechanisms in respect of each operating shift:

1.	The date, the time of day the shift began and ended and the number or designation of the shift.

2.	The names of all operators on duty during the shift.

3.	Any departures from normal operating procedures that occurred during the shift and the time they occurred.

4.	Any special instructions that were given during the shift to depart from normal operating procedures and the person who gave the instructions.

5.	Any unusual or abnormal conditions that were observed in the subsystem during the shift, any action that was taken and any conclusions drawn from the observations.

6.	Any equipment that was taken out of service or ceased to operate during the shift and any action taken to maintain or repair equipment during the shift.  O. Reg. 128/04, s. 27 (5).

(6)  The owner or operating authority shall ensure that logs and other record-keeping mechanisms are accessible at the subsystem,

(a)	for at least five years after the last entry in it was made, in the case of a log or record-keeping mechanism that is kept in a book or document form or kept on a similarly fixed basis; or

(b)	for at least five years after each entry in it was made, in the case of a log or record-keeping mechanism that is kept on a loose-leaf or electronic basis or kept on a similarly continuous basis.  O. Reg. 128/04, s. 27 (6).

(7)  The owner or operating authority shall submit to the Director copies or summaries of the records kept under this section when requested to do so by the Director.  O. Reg. 128/04, s. 27 (7).

Operations and maintenance manuals

28.  The owner or operating authority of a subsystem shall ensure that operators and maintenance personnel in the subsystem have ready access to the comprehensive operations and maintenance manuals that contain plans, drawings and process descriptions sufficient for the safe and efficient operation of the subsystem.  O. Reg. 128/04, s. 28.

Operator and Water Quality Analyst Training

Operator training requirements

29.  (1)  Subject to subsection (2), the owner or operating authority of a subsystem shall take reasonable steps to ensure that every operator employed in the subsystem completes the annual number of hours of training set out in the Table to this section in each of the three years for which the operator’s certificate is valid.  O. Reg. 128/04, s. 29 (1).

(2)  The annual number of hours of training set out in the Table to this section may be averaged over the three years during which an operator’s certificate is valid but shall not be reduced or prorated for an operator who is employed on a part-time basis.  O. Reg. 128/04, s. 29 (2).

(3)  Operators shall comply with the following rules in respect of the training:

1.	If an operator works in more than one type and class of subsystem, the operator must complete the number of hours of training required for the highest type and class of subsystem where the operator works.

2.	If an operator is not employed in a subsystem on the day that he or she renews his or her certificate, the operator must complete the number of hours of training for the highest type and class of subsystem for which the operator was certified.  O. Reg. 128/04, s. 29 (3).

(4)  The continuing education that is used to meet the training requirements must be approved by the Director using criteria which includes the following:

1.	The training course must have documented learning objectives.

2.	The training course must be planned and be provided by a qualified training provider.

3.	The training course must include a means to verify that the participants have learned the material covered in the course.

4.	The training course must cover subject matter that is directly related to the duties typically performed by an operator.  O. Reg. 128/04, s. 29 (4).

(5)  The on-the-job practical training that is used to meet the training requirements must meet criteria that includes the following:

1.	The training must have documented learning objectives.

2.	The training must be provided by a trainer with expertise in the subject matter that is being covered.

3.	The training must be in respect of subject matter that is directly related to the duties typically performed by an operator.  O. Reg. 128/04, s. 29 (5).

(6)  The annual number of hours for training requirements set out in the Table to this section is the minimum number of hours for training and is comprised of a minimum number of hours of continuing education, including a mandatory training course approved by the Director, and on-the-job practical training.  O. Reg. 128/04, s. 29 (6).

(7)  The owner or operating authority shall ensure that records are maintained for at least five years of all on-the-job practical training completed by the operators employed in the subsystem, including the names of the operators who attend training, the dates of the training, the method used for training, the instructor, the duration of each training session and the subjects covered.  O. Reg. 128/04, s. 29 (7).

(8)  Every operator whose operator’s licence was deemed by subsection 12 (2) of the Act to be an operator’s certificate and whose certificate expires after August 1, 2007 shall submit to the Director on or before August 1, 2007 evidence of the training completed by him or her.  O. Reg. 128/04, s. 29 (8).

TABLE
Annual TRAINING FOR OPERATORS

| Type and Class of Subsystem Where the Operator is Employed | Training Requirements | Minimum Total Hours |
| --- | --- | --- |
| Limited Groundwater or Limited Surface Water | 7 hours or more of continuing education, with the remaining hours to at least the minimum total as on-the-job practical training | 20 |
| Class I Water Treatment or Class I Distribution or Class I Distribution and Supply | 7 hours or more of continuing education, with the remaining hours to at least the minimum total as on-the-job practical training | 30 |
| Class II Water Treatment or Class II Distribution or Class II Distribution and Supply | 12 hours or more of continuing education, with the remaining hours to at least the minimum total as on-the-job practical training | 35 |
| Class III Water Treatment or Class III Distribution or Class III Distribution and Supply | 14 hours or more of continuing education, with the remaining hours to at least the minimum total as on-the-job practical training | 40 |
| Class IV Water Treatment or Class IV Distribution or Class IV Distribution and Supply | 14 hours or more of continuing education, with the remaining hours to at least the minimum total as on-the-job practical training | 50 |

O. Reg. 128/04, s. 29, Table.

Transition, operator training

30.  (1)  Despite subsection 29 (1), the owner or operating authority of a subsystem shall take reasonable steps to ensure that every operator employed in the subsystem completes the following training:

1.	By July 31, 2005, at least 40 hours of training as required by section 17 of Ontario Regulation 435/93 under the Ontario Water Resources Act as it read on July 31, 2004.

2.	In each of the two years that begin on August 1, 2005 and end on July 31, 2007, the annual number of hours of training required under section 29 in each year.  O. Reg. 128/04, s. 30 (1).

(2)  Despite subsection (1), the training required under paragraph 1 of that subsection is required solely for an operator who was employed in a subsystem during the year that begins on August 1, 2004 and ends on July 31, 2005.  O. Reg. 128/04, s. 30 (2).

Water quality analyst training requirements

31.  (1)  Subject to subsection (2), the owner or operating authority of a subsystem shall take reasonable steps to ensure that every water quality analyst employed in the subsystem completes the annual number of hours of training set out in the Table to this section in each of the three years for which the water quality analyst’s certificate is valid.  O. Reg. 128/04, s. 31 (1).

(2)  The annual number of hours of training set out in the Table to this section may be averaged over the three years during which a water quality analyst’s certificate is valid but shall not be reduced or prorated for a water quality analyst who is employed on a part-time basis.  O. Reg. 128/04, s. 31 (2).

(3)  Despite subsection (1), if a water quality analyst also holds an operator’s certificate, the owner or operating authority of the subsystem shall take reasonable steps to ensure that the water quality analyst completes the number of hours of training required for the highest type and class of subsystem where the person works as an operator rather than the annual number of hours set out in the Table to this section.  O. Reg. 128/04, s. 31 (3).

(4)  The continuing education that is used to meet the training requirements must be approved by the Director using criteria which includes the following:

1.	The training course must have documented learning objectives.

2.	The training course must be planned and be provided by a qualified training provider.

3.	The training course must include a means to verify that the participants have learned the material covered in the course.

4.	The training course must cover subject matter that is directly related to the duties typically performed by a water quality analyst.  O. Reg. 128/04, s. 31 (4).

(5)  The on-the-job practical training that is used to meet the training requirements must meet criteria which includes the following:

1.	The training must have documented learning objectives.

2.	The training must be provided by a trainer with expertise in the subject matter that is being covered.

3.	The training must be in respect of subject matter that is directly related to the duties typically performed by a water quality analyst.  O. Reg. 128/04, s. 31 (5).

(6)  The annual number of hours for training requirements set out in the Table to this section is the minimum number of hours for training and is comprised of a minimum number of hours of continuing education, including a mandatory training course approved by the Director, and on-the-job practical training.  O. Reg. 128/04, s. 31 (6).

(7)  The owner or operating authority shall ensure that records are maintained for at least five years of the on-the-job practical training completed by the water quality analysts employed in the subsystem, including the names of the water quality analysts who attend training, the dates of the training, the method used for training, the instructor, the duration of each training session and the subjects covered.  O. Reg. 128/04, s. 31 (7).

TABLE
annual TRAINING FOR WATER QUALITY ANALYSTS

| Training Requirements | Minimum Total Hours |
| --- | --- |
| 7 hours or more of continuing education, with the remaining hours to at least the minimum total as on-the-job practical training | 20 |

O. Reg. 128/04, s. 31, Table.

Transition, water quality analyst training

32.  Despite subsection 31 (1), the owner or operating authority of a subsystem shall take reasonable steps to ensure that every water quality analyst employed in the subsystem completes in each of the two years that begin on August 1, 2005 and end on July 31, 2007, the annual number of hours of training required under section 31 in each year.  O. Reg. 128/04, s. 32.

33.  Omitted (provides for coming into force of provisions of this Regulation).  O. Reg. 128/04, s. 33.

Schedule 1
Municipal residential SubSystem CLASSIFICATION

1.  A distribution and supply subsystem is classified in accordance with Table 1 of this Schedule, based on the number of points applicable to the subsystem under the point structure described in Table 2 of this Schedule.

2.  A distribution subsystem is classified in accordance with Table 1 of this Schedule, based on the number of points applicable to the subsystem under the point structure described in Table 3 of this Schedule.

3.  A water treatment subsystem, including a package treatment subsystem, is classified in accordance with Table 1 of this Schedule, based on the number of points applicable to the subsystem under the point structure described in Table 4 of this Schedule.

4.  (1)  For the purpose of section 3 of this Schedule, the number of points applicable to a package treatment subsystem under the point structure described in Table 4 of this Schedule may be reduced by the Director if he or she is satisfied that the operational needs of the package treatment subsystem are less than the operational needs of other water treatment subsystems.

(2)  In subsection (1),

“package water treatment subsystem” means a water treatment subsystem where the treatment processes of the subsystem meet the following criteria:

1.	The subsystem has been manufactured as a complete unit.

2.	The subsystem has been preassembled and delivered in not more than four modules to the site where it is used.

3.	The subsystem has a design flow of 1,300 cubic metres per day or less.

TABLE 1
CLASSES OF municipal residential subsystems

| Class | Number of Points |
| --- | --- |
| Class I | 30 or less |
| Class II | 31 to 55 |
| Class III | 56 to 75 |
| Class IV | 76 or more |

table 2
distribution and supply subsystems

|  | Subsystem Characteristics | Number of Points |
| --- | --- | --- |
| 1. | Size | 1 point for every 4,500 cubic metres of average daily design flow, 1 point for every 4,500 cubic metres of average daily flow in peak month, or 1 point for every 100 kilometres of water main, whichever is greatest, to a maximum of 10 points |
| 2. | Water Treatment by Subsystem |  |
|  | – Subsystem treats water | 5 |
|  | – Treatment provided by subsystem |  |
|  | – Chlorination or a comparable process | 5 |
|  | – pH adjustment | 4 |
|  | – Stability or corrosion control by chemical addition | 4 |
| 3. | Operating Systems |  |
|  | – Pressures in subsystem are less than 150 psi | 2 |
|  | – Pressures in subsystem are 150 psi or more | 3 |
|  | – Pressure zones |  |
|  | – Subsystem has single pressure zone | 2 |
|  | – Subsystem has multiple pressure zones with no pressure control | 5 |
|  | – Subsystem has multiple pressure zones with pressure control | 10 |
|  | – Pumps |  |
|  | – Subsystem has single speed, manually operated pumps | 10 |
|  | – Subsystem has single speed, automatically operated pumps | 3 |
|  | – Subsystem has variable speed pumps | 4 |
|  | – Subsystem has more than one variable speed pump to one pressure zone | 2 |
|  | – Subsystem has surge tanks | 2 |
|  | – Storage |  |
|  | – Subsystem has above ground water storage | 5 |
|  | – Subsystem has in ground water storage | 6 |
|  | – Specialized valves |  |
|  | – Subsystem has pressure reducing valves | 3 |
|  | – Subsystem has pressure relief valves | 3 |
|  | – Subsystem has pneumatic control valves | 3 |
|  | – Subsystem has motorized valves | 3 |
|  | – Subsystem has elevation valves | 3 |
| 4. | Subsystem Specifications |  |
|  | – Piping |  |
|  | – Size | 1 point for every 10% of subsystem’s piping that is less than 600 millimetres in diameter, to a maximum of 5 points, plus 1 point for every 10% of subsystem’s piping that is 600 millimetres or more in diameter |
|  | – Water main Materials |  |
|  | – Cement and iron, ductile iron, polyvinyl chloride, asbestos cement or polyethylene | 2 |
|  | – Concrete pressure pipe | 3 |
|  | – Steel | 3 |
|  | – Other | 3 |
|  | – Non-Residential Meters on Subsystem |  |
|  | – Flow meters with register only | 2 |
|  | – Flow and pressure meters with register and chart | 2 |
|  | – Flow and pressure meters in a supervisory control and data acquisition (SCADA) system | 3 |
|  | – Instrumentation |  |
|  | – Subsystem has manual controls | 3 |
|  | – Subsystem has semi-automatic controls | 2 |
|  | – Subsystem has fully automatic controls | 2 |
|  | – Subsystem has supervisory control and data acquisition (SCADA) system | 3 |
| 5. | Laboratory Control by Subsystem Personnel |  |
|  | (a) Bacteriological and Biological | The greatest of the following amounts that apply to the subsystem: |
|  | – Subsystem personnel perform bacteriological tests for non-regulatory purposes | 4 |
|  | – Subsystem personnel do biological identification | 7 |
|  | – Lab work is performed by an accredited laboratory | 0 |
|  | (b) Chemical and Physical | The greatest of the following amounts that apply to the subsystem: |
|  | – Subsystem personnel use colorimetric methods to do simple determinations such as pH or chlorine residual | 3 |
|  | – Subsystem personnel do alkalinity or hardness determinations, titrations, jar tests, or similar tests | 5 |
|  | – Subsystem personnel do determinations for operational parameters (e.g. aluminium) or perform advanced chemical testing beyond regulatory requirements | 7 |
|  | – Subsystem personnel perform procedures using very sophisticated instrumentation, such as atomic absorption or gas chromatography | 10 |

TABLE 3
DISTRIBUTION SUBSYSTEMS

|  | Subsystem Characteristics | Number of Points |
| --- | --- | --- |
| 1. | Size | 1 point for every 4,500 cubic metres of average daily design flow, 1 point for every 4,500 cubic metres of average daily flow in peak month, or 1 point for every 100 kilometres of water main, whichever is greatest, to a maximum of 10 points |
| 2. | Chemical Addition |  |
|  | – Re-chlorination within distribution system | 5 |
|  | – pH adjustment | 4 |
|  | – Stability or corrosion control by chemical addition | 4 |
| 3. | Operating Systems |  |
|  | – Pressures in subsystem are less than 150 psi | 2 |
|  | – Pressures in subsystem are 150 psi or more | 3 |
|  | – Pressure zones |  |
|  | – Subsystem has single pressure zone | 2 |
|  | – Subsystem has multiple pressure zones with no pressure control | 5 |
|  | – Subsystem has multiple pressure zones with pressure control | 10 |
|  | – Pumps |  |
|  | – Subsystem has single speed, manually operated pumps | 10 |
|  | – Subsystem has single speed, automatically operated pumps | 3 |
|  | – Subsystem has variable speed pumps | 4 |
|  | – Subsystem has more than one variable speed pump to one pressure zone | 2 |
|  | – Subsystem has surge tanks | 2 |
|  | – Storage |  |
|  | – Subsystem has above ground water storage | 5 |
|  | – Subsystem has in ground water storage | 6 |
|  | – Specialized valves |  |
|  | – Subsystem has pressure reducing valves | 3 |
|  | – Subsystem has pressure relief valves | 3 |
|  | – Subsystem has pneumatic control valves | 3 |
|  | – Subsystem has motorized valves | 3 |
|  | – Subsystem has elevation valves | 3 |
| 4. | Subsystem Specifications |  |
|  | – Piping |  |
|  | – Size | 1 point for every 10% of subsystem’s piping that is less than 600 millimetres in diameter, to a maximum of 5 points, plus 1 point for every 10% of subsystem’s piping that is 600 millimetres or more in diameter |
|  | – Water main Materials |  |
|  | – Cement and iron, ductile iron, polyvinyl chloride, asbestos cement or polyethylene | 2 |
|  | – Concrete pressure pipe | 3 |
|  | – Steel | 3 |
|  | – Other | 3 |
|  | – Non-Residential Meters on Subsystem |  |
|  | – Flow meters with register only | 2 |
|  | – Flow and pressure meters with register and chart | 2 |
|  | – Flow and pressure meters in a supervisory control and data acquisition (SCADA) system | 3 |
|  | – Instrumentation |  |
|  | – Subsystem has manual controls | 3 |
|  | – Subsystem has semi-automatic controls | 2 |
|  | – Subsystem has fully automatic controls | 2 |
|  | – Subsystem has supervisory control and data acquisition (SCADA) system | 3 |
| 5. | Laboratory Control by Subsystem Personnel |  |
|  | (a) Bacteriological and Biological | The greatest of the following amounts that apply to the subsystem: |
|  | – Subsystem personnel perform bacteriological tests for non-regulatory purposes | 4 |
|  | – Subsystem personnel do biological identification | 7 |
|  | – Lab work is performed by an accredited laboratory | 0 |
|  | (b) Chemical and Physical | The greatest of the following amounts that apply to the subsystem: |
|  | – Subsystem personnel use colorimetric methods to do simple determinations such as pH or chlorine residual | 3 |
|  | – Subsystem personnel do alkalinity or hardness determinations, titrations, jar tests, or similar tests | 5 |
|  | – Subsystem personnel do determinations for operational parameters (e.g. aluminium) or perform advanced chemical testing beyond regulatory requirements | 7 |
|  | – Subsystem personnel perform procedures using very sophisticated instrumentation, such as atomic absorption or gas chromatography | 10 |

TABLE 4
Water TREATMENT SUBSYSTEMS

|  | Subsystem Characteristics | Number of Points |
| --- | --- | --- |
| 1. | Population Served | 1 point for every 10,000 people, to a maximum of 10 points |
| 2. | Flow (average daily design flow or average daily flow in peak month, whichever is greater) | 1 point for every 4,500 cubic metres, to a maximum of 10 points |
| 3. | Water Supply Source |  |
|  | – Groundwater | 3 |
|  | – Surface water | 5 |
| 4. | Raw Water Quality |  |
|  | – Low variation in raw water quality and raw water quality is not subject to serious industrial waste pollution | 0 |
|  | – Moderate variation in raw water quality and raw water quality is not subject to serious industrial waste pollution | 2 |
|  | – High variation in raw water quality and raw water quality is not subject to serious industrial waste pollution | 5 |
|  | – Raw water quality is subject to serious industrial waste pollution | 10 |
| 5. | Processes |  |
|  | – Subsystem uses aeration, other than packed tower aeration | 2 |
|  | – Subsystem uses packed tower aeration | 6 |
|  | – Subsystem uses pH adjustment | 4 |
|  | – Subsystem uses stability or corrosion control | 4 |
|  | – Subsystem uses taste and odour control | 8 |
|  | – Subsystem uses colour control | 4 |
|  | – Subsystem uses iron or manganese removal | 10 |
|  | – Subsystem uses ion exchange softening | 10 |
|  | – Subsystem uses chemical precipitation softening | 20 |
|  | – Subsystem uses coagulant addition | 4 |
|  | – Subsystem uses flocculation | 6 |
|  | – Subsystem uses sedimentation | 5 |
|  | – Subsystem uses up flow clarification | 14 |
|  | – Subsystem uses filtration | 10 |
|  | – Subsystem uses fluoridation | 5 |
|  | – Subsystem uses disinfection | 5 |
|  | – Subsystem uses chlorine dioxide, chloramines or ozonation for disinfection | 5 |
|  | – Subsystem uses other special processes | 15 |
|  | – Subsystem has internal treatment of plant sludge | 6 |
| 6. | Sludge/Backwash Water Disposal |  |
|  | – No disposal to raw water sources | 0 |
|  | – Any disposal to raw water source | 2 |
|  | – Any disposal to subsystem raw water | 5 |
| 7. | Laboratory Control by Subsystem Personnel |  |
|  | (a) Bacteriological and Biological | The greatest of the following amounts that apply to the subsystem: |
|  | – Subsystem personnel perform bacteriological tests for non-regulatory purposes | 4 |
|  | – Subsystem personnel do biological identification | 7 |
|  | – Lab work is performed by an accredited laboratory | 0 |
|  | (b) Chemical and Physical | The greatest of the following amounts that apply to the subsystem: |
|  | – Subsystem personnel use colorimetric methods to do simple determinations such as pH or chlorine residual | 3 |
|  | – Subsystem personnel do alkalinity or hardness determinations, titrations, jar tests or similar tests | 5 |
|  | – Subsystem personnel do determinations for operational parameters (e.g. aluminium) or perform advanced chemical testing beyond regulatory requirements | 7 |
|  | – Subsystem personnel perform procedures using very sophisticated instrumentation, such as atomic absorption or gas chromatography | 10 |

O. Reg. 128/04, Sched. 1.

Schedule 2
QUALIFICATIONS FOR OPERATORS’ CERTIFICATES

Operators-in-Training

1.  (1)  The qualifications for an operator-in-training’s certificate for a type of municipal residential subsystem are:

1.	The person must have successfully completed Grade 12 in Ontario or have education qualifications that the Director considers equivalent.

2.	The person must have obtained a mark that the Director considers satisfactory in an examination approved by the Director relating to the functions performed by operators-in-training.

3.	The person must have successfully completed a course of study approved by the Director relating to the functions performed by operators-in-training and must have obtained a mark in that course that the Director considers satisfactory.

(2)  Paragraph 3 of subsection (1) does not apply to a person who works as an operator-in-training for not more than four consecutive months in any seven-month period but the person must successfully complete a mandatory training course approved by the Director each time the person renews his or her certificate under subsection 9 (10) of the regulation.

Limited Groundwater Subsystem Operators

2.  The qualifications for a limited groundwater subsystem operator’s certificate are:

1.	The person must have successfully completed Grade 12 in Ontario or have education qualifications or training that the Director considers equivalent.

2.	The person must have completed a course of study approved by the Director and must have obtained a mark that the Director considers satisfactory in the course.

3.	The person must have obtained a mark that the Director considers satisfactory in an examination approved by the Director relating to the functions performed by operators in a limited groundwater subsystem.

Limited Surface Water Subsystem Operators

3.  The qualifications for a limited surface water subsystem operator’s certificate are:

1.	The person must have successfully completed Grade 12 in Ontario or have education qualifications or training that the Director considers equivalent.

2.	The person must have completed a course of study approved by the Director and must have obtained a mark that the Director considers satisfactory in the course.

3.	The person must have obtained a mark that the Director considers satisfactory in an examination approved by the Director relating to the functions performed by operators in a limited surface water subsystem.

Class I Operators

4.  The qualifications for a Class I operator’s certificate for a type of municipal residential subsystem are:

1.	The person must have successfully completed Grade 12 in Ontario or have education qualifications that the Director considers equivalent.

2.	The person must have obtained a mark that the Director considers satisfactory in an examination approved by the Director relating to the functions performed by operators with Class I certificates for a municipal residential subsystem.

3.	The person must have completed the course of study approved by the Director for an operator-in-training, unless the person held an operator-in-training licence on August 1, 2005.

4.	The person must have at least one year of experience as an operator-in-training in that type of municipal residential subsystem after being certified as an operator-in-training.

5.	The person must have experience that the Director considers equivalent to the experience set out in paragraph 4 instead of those qualifications.

Class II Operators

5.  The qualifications for a Class II operator’s certificate for a type of municipal residential subsystem are:

1.	The person must have successfully completed Grade 12 in Ontario or have education qualifications that the Director considers equivalent.

2.	The person must have obtained a mark that the Director considers satisfactory in an examination approved by the Director relating to the functions performed by operators with Class II certificates for that type of subsystem.

3.	The person must have a Class I operator’s certificate for that type of municipal residential subsystem.

4.	The person must have at least three years of experience as an operator in that type of subsystem.

5.	The person must have experience that the Director considers equivalent to the qualifications set out in paragraph 4 instead of those qualifications.

Class III Operators

6.  (1)  The qualifications for a Class III operator’s certificate for a type of municipal residential subsystem are:

1.	The person must,

i.	have successfully completed Grade 12 in Ontario or have education qualifications that the Director considers equivalent, and

ii.	have successfully completed at least two years of additional education or training that, in the opinion of the Director, is relevant to the functions performed by operators of municipal residential subsystems.

2.	The person must have obtained a mark that the Director considers satisfactory in an examination approved by the Director relating to the functions performed by operators with Class III certificates for that type of subsystem.

3.	The person must have a Class II operator’s certificate for that type of municipal residential subsystem.

4.	The person must have at least four years of experience as an operator in that type of subsystem, including at least two years as an operator-in-charge in that type of Class III or Class IV subsystem.

5.	The person must have experience that the Director considers equivalent to the qualifications set out in paragraph 4 instead of those qualifications.

(2)  For the purpose of meeting the education or training qualifications in subparagraph 1 ii of subsection (1), an applicant for a certificate may substitute up to one year of experience as an operator-in-charge in a Class II, Class III or Class IV subsystem for the equivalent length of education, but experience as an operator-in-charge used for this purpose shall not be used to meet the experience qualification in paragraph 4 of subsection (1).

Class IV Operators

7.  (1)  The qualifications for a Class IV operator’s certificate for a type of municipal residential subsystem are:

1.	The person must,

i.	have successfully completed Grade 12 in Ontario or have education qualifications that the Director considers equivalent, and

ii.	have successfully completed at least four years of additional education or training that, in the opinion of the Director, is relevant to the functions performed by operators of that type of subsystem.

2.	The person must have obtained a mark that the Director considers satisfactory in an examination approved by the Director relating to the functions performed by operators with Class IV certificates for that type of subsystem.

3.	The person must have a Class III operator’s certificate for that type of municipal residential subsystem.

4.	The person must have at least four years of experience as an operator in that type of subsystem, including at least two years as an operator-in-charge in that type of Class III or Class IV subsystem.

5.	The person must have experience that the Director considers equivalent to the qualifications set out in paragraph 4 instead of those qualifications.

(2)  For the purpose of meeting the education or training qualifications in subparagraph 1 ii of subsection (1), an applicant for a certificate may substitute up to two years of experience as an operator-in-charge in that type of Class III or Class IV subsystem for the equivalent length of education, but experience as an operator-in-charge used for this purpose shall not be used to meet the experience qualification in paragraph 4 of subsection (1).

Supplemental Rules — Education and Training

8.  For the purpose of determining whether a person meets the education and training qualifications established by this Schedule, the length of education or training obtained through continuing education shall be determined on the basis that 450 hours of participation in continuing education is equivalent to one year of education or training.

Supplemental Rules — Experience

9.  (1)  The following rules apply for the purpose of determining whether a person meets the experience qualifications established by this Schedule for a Class II, Class III or Class IV certificate:

1.	An applicant for a certificate may substitute education or training that meets the requirements of the education and training qualifications established by this Schedule, other than elementary or secondary education, for up to half of the experience as an operator required by this Schedule, but education or training used for this purpose shall not be used to meet the education and training qualifications established by this Schedule.

2.	The Director may permit an applicant for a certificate to substitute experience as an operator in a different type of system or experience in a system other than as an operator or other relevant qualifications for the experience required by this Schedule, if the Director is of the opinion that the experience or qualifications are relevant to the certificate being applied for.

(2)  Experience as an operator-in-training shall not be considered for the purpose of determining whether a person meets the qualification of at least two years of experience as an operator-in-charge established by this Schedule for a Class III or Class IV certificate.

(3)  Experience obtained while a person is not certified for the work he or she is doing shall not be considered in determining whether the person meets any experience qualifications established by this Schedule for any class of certificate.

Supplemental Rules — Examinations

10.  Where this Schedule provides that obtaining a mark that the Director considers satisfactory in an examination approved by the Director is a qualification for a class of certificate, the Director may approve different examinations for different categories of applicants for that class of certificate.

O. Reg. 128/04, Sched. 2.

Schedule 3
QUALIFICATIONS FOR WATER QUALITY ANALYSTS’ CERTIFICATES

1.  The qualifications for a water quality analyst’s certificate are:

1.	The person must have successfully completed Grade 12 in Ontario or have education qualifications that the Director considers equivalent.

2.	The person must have obtained a mark that the Director considers satisfactory in an examination approved by the Director relating to water analysis in drinking water systems.

3.	The person must have at least one year of experience working in a drinking water system performing tests on water or working in a facility that in the Director’s opinion is related to the experience of working in a drinking water system, or

4.	The person must have completed the training approved by the Director related to the testing of water.

5.	The person must have experience that the Director considers equivalent to the qualifications set out in paragraph 3 instead of those qualifications.

O. Reg. 128/04, Sched. 3.

Schedule 4
QUALIFICATIONS FOR renewal of CERTIFICATES

Renewal of Certificates Continued under Subsection 12 (3) of the Act

1.  (1)  An operator’s licence that was deemed to be a certificate under subsection 12 (3) of the Act that expires before May 14, 2006 may be renewed as a certificate under the regulation, if the applicant has at least three months experience in the previous 36 months of working as an operator in a subsystem or of having duties that the Director considers related to those of an operator of a subsystem.

(2)  A certificate that is renewed under subsection (1) shall expire on May 14, 2006.

(3)  Despite subsection (2), if an applicant to whom subsection (1) applies obtains a mark that the Director considers satisfactory in an examination approved by the Director, the certificate shall be renewed for three years.

(4)  If an operator’s licence that was deemed to be a certificate under subsection 12 (3) of the Act has an expiry date on or after May 14, 2006 but which expires on May 14, 2006 because of the operation of subsection 12 (3) of the Act, the certificate may be renewed as a certificate under the regulation if the applicant obtains a mark that the Director considers satisfactory in an examination approved by the Director.

(5)  A certificate that is renewed under subsection (4) shall expire on the original expiry date of the licence that is continued as a certificate pursuant to subsection 12 (3) of the Act.

Renewal of Operator’s Certificates, General

2.  For an operator’s certificate to be renewed, since the issue of the certificate or since it was last renewed, whichever is applicable, the applicant for the renewal of the operator’s certificate,

(a)	must have completed the training requirements set out in section 29 of the regulation; and

(b)	must have at least three months experience in the previous 36 months of working as an operator in a subsystem or of having duties that the Director considers related to those of an operator of a subsystem.

Renewal of a Conditional Operator’s Certificate

3.  For a conditional operator’s certificate to be renewed, since the issue of the conditional certificate or since it was last renewed, whichever is applicable, the applicant for the renewal of the certificate,

(a)	must have completed the training requirements set out in section 29 of the regulation;

(b)	must have at least three months experience in the previous 36 months of working as an operator in a subsystem or of having duties that the Director considers related to those of an operator of a subsystem; and

(c)	must have met the conditions set out in the conditional certificate.

Renewal of a Water Quality Analyst’s Certificate

4.  For a water quality analyst’s certificate to be renewed, since the issuance of the certificate or since it was last renewed, whichever is applicable, the applicant for the renewal of the certificate,

(a)	must have completed the training requirements set out in section 31 of the regulation; and

(b)	must have at least three months experience in the previous 36 months of working as a water quality analyst in a subsystem or of having duties that the Director considers related to those of a water quality analyst in a subsystem.

Renewal of a Conditional Water Quality Analyst’s Certificate

5.  For a conditional water quality analyst’s certificate to be renewed, since the issue of the conditional certificate or since it was last renewed, whichever is applicable, the applicant for the renewal of the certificate,

(a)	must have completed the training requirements set out in section 31 of the regulation;

(b)	must have at least three months experience in the previous 36 months of working as a water quality analyst in a subsystem or of having duties that the Director considers related to those of a water quality analyst in a subsystem; and

(c)	must have met the conditions set out in the conditional certificate.

O. Reg. 128/04, Sched. 4.

Back to top