City of Toronto Act, 2006
Loi de 2006 sur la cité de Toronto

ONTARIO REGULATION 612/06

Minimum maintenance standards for highways in the city of toronto

Historical version for the period February 18, 2010 to January 24, 2013.

Last amendment:  O. Reg. 22/10.

This Regulation is made in English only.

Definitions

1.  (1)  In this Regulation,

“cm” means centimetres;

“day” means a 24-hour period;

“motor vehicle” has the same meaning as in subsection 1 (1) of the Highway Traffic Act, except that it does not include a motor assisted bicycle;

“non-paved surface” means a surface that is not a paved surface;

“Ontario Traffic Manual” means the Ontario Traffic Manual published by the Ministry of Transportation, as amended from time to time;

“paved surface” means a surface with a wearing layer or layers of asphalt, concrete or asphalt emulsion;

“roadway” has the same meaning as in subsection 1 (1) of the Highway Traffic Act;

“shoulder” means the portion of a highway that provides lateral support to the roadway and that may accommodate stopped motor vehicles and emergency use;

“surface” means the top of a roadway or shoulder.  O. Reg. 612/06, s. 1 (1); O. Reg. 22/10, s. 1 (1).

(2)  For the purposes of this Regulation, every highway or part of a highway under the City’s jurisdiction is classified in the Table to this section as a Class 1, Class 2, Class 3, Class 4, Class 5 or Class 6 highway, based on the speed limit applicable to it and the average annual daily traffic on it.  O. Reg. 612/06, s. 1 (2).

(3)  For the purposes of subsection (2) and the Table to this section, the average annual daily traffic on a highway or part of a highway under the City’s jurisdiction shall be determined,

(a)	by counting and averaging the daily two-way traffic on the highway or part of the highway; or

(b)	by estimating the average daily two-way traffic on the highway or part of the highway.  O. Reg. 612/06, s. 1 (3); O. Reg. 22/10, s. 1 (2).

(4)  For the purposes of this Regulation, the City is deemed to be aware of a fact if, in the absence of actual knowledge of the fact, circumstances are such that the City ought reasonably to be aware of the fact.  O. Reg. 22/10, s. 1 (3).

TABLE
CLASSIFICATION OF HIGHWAYS

| Average Annual Daily Traffic (number of motor vehicles) | Posted or Statutory Speed Limit (kilometres per hour) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 91 - 100 | 81 - 90 | 71 - 80 | 61 - 70 | 51 - 60 | 41 - 50 | 1 - 40 |
| 15,000 or more | 1 | 1 | 1 | 2 | 2 | 2 | 2 |
| 12,000 - 14,999 | 1 | 1 | 1 | 2 | 2 | 3 | 3 |
| 10,000 - 11,999 | 1 | 1 | 2 | 2 | 3 | 3 | 3 |
| 8,000 - 9,999 | 1 | 1 | 2 | 3 | 3 | 3 | 3 |
| 6,000 - 7,999 | 1 | 2 | 2 | 3 | 3 | 3 | 3 |
| 5,000 - 5,999 | 1 | 2 | 2 | 3 | 3 | 3 | 3 |
| 4,000 - 4,999 | 1 | 2 | 3 | 3 | 3 | 3 | 4 |
| 3,000 - 3,999 | 1 | 2 | 3 | 3 | 3 | 4 | 4 |
| 2,000 - 2,999 | 1 | 2 | 3 | 3 | 4 | 4 | 4 |
| 1,000 - 1,999 | 1 | 3 | 3 | 3 | 4 | 4 | 5 |
| 500 - 999 | 1 | 3 | 4 | 4 | 4 | 4 | 5 |
| 200 - 499 | 1 | 3 | 4 | 4 | 5 | 5 | 5 |
| 50 - 199 | 1 | 3 | 4 | 5 | 5 | 5 | 5 |
| 0 - 49 | 1 | 3 | 6 | 6 | 6 | 6 | 6 |

O. Reg. 612/06, s. 1, Table.

Application

2.  (1)  This Regulation sets out the minimum standards of repair for highways under the City’s jurisdiction for the purpose of clause 42 (3) (c) of the Act.  O. Reg. 612/06, s. 2 (1).

(2)  Revoked:  O. Reg. 22/10, s. 2.

(3)  This Regulation does not apply to Class 6 highways.  O. Reg. 612/06, s. 2 (3).

Minimum Standards

Patrolling

3.  (1)  The minimum standard for the frequency of patrolling of highways to check for conditions described in this Regulation is set out in the Table to this section.  O. Reg. 22/10, s. 3 (1).

(2)  During the season when the City performs winter highway maintenance, the minimum standard for patrolling highways is, in addition to that set out in subsection (1), to patrol highways that the City selects as representative of its highways, as necessary, to check for conditions described in sections 4 and 5.  O. Reg. 22/10, s. 3 (1).

(3)  Patrolling a highway consists of observing the highway, either by driving on or by electronically monitoring the highway, and may be performed by persons responsible for patrolling highways or by persons responsible for or performing highway maintenance activities.  O. Reg. 22/10, s. 3 (1).

(4)  This section does not apply in respect of the conditions described in section 10, subsections 11 (0.1) and 12 (1) and section 16.1.  O. Reg. 22/10, s. 3 (1).

table
PATROLLING FREQUENCY

| Class of Highway | Patrolling Frequency |
| --- | --- |
| 1 | 3 times every 7 days |
| 2 | 2 times every 7 days |
| 3 | once every 7 days |
| 4 | once every 14 days |
| 5 | once every 30 days |

O. Reg. 612/06, s. 3, Table; O. Reg. 22/10, s. 3 (2).

Snow accumulation

4.  (1)  The minimum standard for clearing snow accumulation is,

(a)	after becoming aware of the fact that the snow accumulation on a roadway is greater than the depth set out in the Table to this section, to deploy snow-clearing resources as soon as practicable; and

(b)	after the snow accumulation has ended, to clear the snow to a depth less than or equal to the depth set out in the Table within the time set out in the Table,

(i)	to provide a minimum lane width of the lesser of three metres for each lane or the actual lane width, or

(ii)	on a Class 4 or Class 5 highway with two lanes, to provide a total width of at least five metres.  O. Reg. 22/10, s. 4.

(2)  This section,

(a)	does not apply to that portion of the roadway designated for parking; and

(b)	only applies to a municipality during the season when the municipality performs winter highway maintenance.  O. Reg. 22/10, s. 4.

(3)  In this section,

“snow accumulation” means the natural accumulation of any of the following that, alone or together, covers more than half a lane width of a roadway:

1.	New fallen snow.

2.	Wind-blown snow.

3.	Slush.  O. Reg. 22/10, s. 4.

TABLE
SNOW ACCUMULATION

| Class of Highway | Depth | Time |
| --- | --- | --- |
| 1 | 2.5 cm | 4 hours |
| 2 | 5 cm | 6 hours |
| 3 | 8 cm | 12 hours |
| 4 | 8 cm | 16 hours |
| 5 | 10 cm | 24 hours |

O. Reg. 612/06, s. 4, Table.

Icy roadways

5.  (1)  The minimum standard for treating icy roadways after becoming aware of the fact that a roadway is icy is to treat the icy roadway within the time set out in the Table to this section.  O. Reg. 22/10, s. 5.

(2)  This section only applies during the season when the City performs winter highway maintenance.  O. Reg. 612/06, s. 5 (2).

TABLE
ICY ROADWAYS

| Class of Highway | Time |
| --- | --- |
| 1 | 3 hours |
| 2 | 4 hours |
| 3 | 8 hours |
| 4 | 12 hours |
| 5 | 16 hours |

O. Reg. 612/06, s. 5, Table.

Potholes

6.  (1)  If a pothole exceeds both the surface area and depth set out in Table 1, 2 or 3 to this section, as the case may be, the minimum standard is to repair the pothole within the time set out in Table 1, 2 or 3, as appropriate, after becoming aware of the fact.  O. Reg. 612/06, s. 6 (1).

(2)  A pothole is deemed to be repaired if its surface area or depth is less than or equal to that set out in Table 1, 2 or 3, as appropriate.  O. Reg. 612/06, s. 6 (2).

TABLE 1
POTHOLES ON PAVED SURFACE OF ROADWAY

| Class of Highway | Surface Area | Depth | Time |
| --- | --- | --- | --- |
| 1 | 600 cm² | 8 cm | 4 days |
| 2 | 800 cm² | 8 cm | 4 days |
| 3 | 1000 cm² | 8 cm | 7 days |
| 4 | 1000 cm² | 8 cm | 14 days |
| 5 | 1000 cm² | 8 cm | 30 days |

O. Reg. 612/06, s. 6, Table 1.

TABLE 2
POTHOLES ON NON-PAVED SURFACE OF ROADWAY

| Class of Highway | Surface Area | Depth | Time |
| --- | --- | --- | --- |
| 3 | 1500 cm² | 8 cm | 7 days |
| 4 | 1500 cm² | 10 cm | 14 days |
| 5 | 1500 cm² | 12 cm | 30 days |

O. Reg. 612/06, s. 6, Table 2.

TABLE 3
POTHOLES ON PAVED OR NON-PAVED SURFACE OF SHOULDER

| Class of Highway | Surface Area | Depth | Time |
| --- | --- | --- | --- |
| 1 | 1500 cm² | 8 cm | 7 days |
| 2 | 1500 cm² | 8 cm | 7 days |
| 3 | 1500 cm² | 8 cm | 14 days |
| 4 | 1500 cm² | 10 cm | 30 days |
| 5 | 1500 cm² | 12 cm | 60 days |

O. Reg. 612/06, s. 6, Table 3.

Shoulder drop-offs

7.  (1)  If a shoulder drop-off is deeper, for a continuous distance of 20 metres or more, than the depth set out in the Table to this section, the minimum standard is to repair the shoulder drop-off within the time set out in the Table after becoming aware of the fact.  O. Reg. 612/06, s. 7 (1).

(2)  A shoulder drop-off is deemed to be repaired if its depth is less than or equal to that set out in the Table.  O. Reg. 612/06, s. 7 (2).

(3)  In this section,

“shoulder drop-off” means the vertical differential, where the paved surface of the roadway is higher than the surface of the shoulder, between the paved surface of the roadway and the paved or non-paved surface of the shoulder.  O. Reg. 612/06, s. 7 (3).

TABLE
SHOULDER DROP-OFFS

| Class of Highway | Depth | Time |
| --- | --- | --- |
| 1 | 8 cm | 4 days |
| 2 | 8 cm | 4 days |
| 3 | 8 cm | 7 days |
| 4 | 8 cm | 14 days |
| 5 | 8 cm | 30 days |

O. Reg. 612/06, s. 7, Table.

Cracks

8.  (1)  If a crack on the paved surface of a roadway is greater, for a continuous distance of three metres or more, than both the width and depth set out in the Table to this section, the minimum standard is to repair the crack within the time set out in the Table after becoming aware of the fact.  O. Reg. 612/06, s. 8 (1).

(2)  A crack is deemed to be repaired if its width or depth is less than or equal to that set out in the Table.  O. Reg. 612/06, s. 8 (2).

TABLE
CRACKS

| Class of Highway | Width | Depth | Time |
| --- | --- | --- | --- |
| 1 | 5 cm | 5 cm | 30 days |
| 2 | 5 cm | 5 cm | 30 days |
| 3 | 5 cm | 5 cm | 60 days |
| 4 | 5 cm | 5 cm | 180 days |
| 5 | 5 cm | 5 cm | 180 days |

O. Reg. 612/06, s. 8, Table.

Debris

9.  (1)  If there is debris on a roadway, the minimum standard is to deploy resources, as soon as practicable after becoming aware of the fact, to remove the debris.  O. Reg. 612/06, s. 9 (1).

(2)  In this section,

“debris” means any material or object on a roadway,

(a)	that is not an integral part of the roadway or has not been intentionally placed on the roadway by the City, and

(b)	that is reasonably likely to cause damage to a motor vehicle or to injure a person in a motor vehicle.  O. Reg. 612/06, s. 9 (2).

Luminaires

10.  (0.1)  The minimum standard for the frequency of inspecting all luminaires to check to see that they are functioning is once per year.  O. Reg. 22/10, s. 6.

(1)  For conventional illumination, if three or more consecutive luminaires on a highway are not functioning, the minimum standard is to repair the luminaires within the time set out in the Table to this section after becoming aware of the fact.  O. Reg. 612/06, s. 10 (1).

(2)  For conventional illumination and high mast illumination, if 30 per cent or more of the luminaires on any kilometre of highway are not functioning, the minimum standard is to repair the luminaires within the time set out in the Table to this section after becoming aware of the fact.  O. Reg. 612/06, s. 10 (2).

(3)  Despite subsection (2), for high mast illumination, if all of the luminaires on consecutive poles are not functioning, the minimum standard is to deploy resources as soon as practicable after becoming aware of the fact to repair the luminaires.  O. Reg. 612/06, s. 10 (3).

(4)  Despite subsections (1), (2) and (3), for conventional illumination and high mast illumination, if more than 50 per cent of the luminaires on any kilometre of a Class 1 highway with a speed limit of 90 kilometres per hour or more are not functioning, the minimum standard is to deploy resources as soon as practicable after becoming aware of the fact to repair the luminaires.  O. Reg. 612/06, s. 10 (4).

(5)  Luminaires are deemed to be repaired,

(a)	for the purpose of subsection (1), if the number of non-functioning consecutive luminaires does not exceed two;

(b)	for the purpose of subsection (2), if more than 70 per cent of luminaires on any kilometre of highway are functioning;

(c)	for the purpose of subsection (3), if one or more of the luminaires on consecutive poles are functioning;

(d)	for the purpose of subsection (4), if more than 50 per cent of luminaires on any kilometre of highway are functioning.  O. Reg. 612/06, s. 10 (5).

(6)  Subsections (1), (2) and (3) only apply to,

(a)	Class 1 and Class 2 highways; and

(b)	Class 3, Class 4 and Class 5 highways with a posted speed of 80 kilometres per hour or more.  O. Reg. 612/06, s. 10 (6).

(7)  In this section,

“conventional illumination” means lighting, other than high mast illumination, where there are one or more luminaires per pole;

“high mast illumination” means lighting where there are three or more luminaires per pole and the height of the pole exceeds 20 metres;

“luminaire” means a complete lighting unit consisting of,

(a)	a lamp, and

(b)	parts designed to distribute the light, to position or protect the lamp and to connect the lamp to the power supply.  O. Reg. 612/06, s. 10 (7).

TABLE
LUMINAIRES

| Class of Highway | Time |
| --- | --- |
| 1 | 7 days |
| 2 | 7 days |
| 3 | 14 days |
| 4 | 14 days |
| 5 | 14 days |

O. Reg. 612/06, s. 10, Table.

Signs

11.  (0.1)  The minimum standard for the frequency of inspecting signs of a type listed in subsection (2) to check to see that they meet the retro-reflectivity requirements of the Ontario Traffic Manual is once per year.  O. Reg. 22/10, s. 7 (1).

(1)  If any sign of a type listed in subsection (2) is illegible, improperly oriented, obscured or missing, the minimum standard is to deploy resources as soon as practicable after becoming aware of the fact to repair or replace the sign.  O. Reg. 612/06, s. 11 (1); O. Reg. 22/10, s. 7 (2).

(2)  This section applies to the following types of signs:

1.	Checkerboard.

2.	Curve sign with advisory speed tab.

3.	Do not enter.

3.1	Load Restricted Bridge.

3.2	Low Bridge.

3.3	Low Bridge Ahead.

4.	One Way.

5.	School Zone Speed Limit.

6.	Stop.

7.	Stop Ahead.

8.	Stop Ahead, New.

9.	Traffic Signal Ahead, New.

10.	Two-Way Traffic Ahead.

11.	Wrong Way.

12.	Yield.

13.	Yield Ahead.

14.	Yield Ahead, New.  O. Reg. 612/06, s. 11 (2); O. Reg. 22/10, s. 7 (3).

Regulatory or warning signs

12.  (1)  The minimum standard for the frequency of inspecting regulatory signs or warning signs to check to see that they meet the retro-reflectivity requirements of the Ontario Traffic Manual is once per year.  O. Reg. 22/10, s. 8.

(2)  If a regulatory sign or warning sign is illegible, improperly oriented, obscured or missing, the minimum standard is to repair or replace the sign within the time set out in the Table to this section after becoming aware of the fact.  O. Reg. 22/10, s. 8.

(3)  In this section,

“regulatory sign” and “warning sign” have the same meanings as in the Ontario Traffic Manual, except that they do not include a sign listed in subsection 11 (2) of this Regulation.  O. Reg. 22/10, s. 8.

TABLE
REGULATORY AND WARNING SIGNS

| Class of Highway | Time |
| --- | --- |
| 1 | 7 days |
| 2 | 14 days |
| 3 | 21 days |
| 4 | 30 days |
| 5 | 30 days |

O. Reg. 612/06, s. 12, Table.

Traffic control signal systems

13.  (1)  If a traffic control signal system is defective in any way described in subsection (2), the minimum standard is to deploy resources as soon as practicable after becoming aware of the defect to repair the defect or replace the defective component of the traffic control signal system.  O. Reg. 612/06, s. 13 (1).

(2)  This section applies if a traffic control signal system is defective in any of the following ways:

1.	One or more displays show conflicting signal indications.

2.	The angle of a traffic control signal or pedestrian control indication has been changed in such a way that the traffic or pedestrian facing it does not have clear visibility of the information conveyed or that it conveys confusing information to traffic or pedestrians facing other directions.

3.	A phase required to allow a pedestrian or vehicle to safely travel through an intersection fails to occur.

4.	There are phase or cycle timing errors interfering with the ability of a pedestrian or vehicle to safely travel through an intersection.

5.	There is a power failure in the traffic control signal system.

6.	The traffic control signal system cabinet has been displaced from its proper position.

7.	There is a failure of any of the traffic control signal support structures.

8.	A signal lamp or a pedestrian control indication is not functioning.

9.	Signals are flashing when flashing mode is not a part of the normal signal operation.  O. Reg. 612/06, s. 13 (2).

(3)  Despite subsection (1) and paragraph 8 of subsection (2), if the posted speed of all approaches to the intersection or location of the non-functioning signal lamp or pedestrian control indication is less than 80 kilometres per hour and the signal that is not functioning is a green or a pedestrian “walk” signal, the minimum standard is to repair or replace the defective component by the end of the next business day.  O. Reg. 612/06, s. 13 (3).

(4)  In this section and section 14,

“cycle” means a complete sequence of traffic control indications at a location;

“display” means the illuminated and non-illuminated signals facing the traffic;

“indication” has the same meaning as in the Highway Traffic Act;

“phase” means a part of a cycle from the time where one or more traffic directions receive a green indication to the time where one or more different traffic directions receive a green indication;

“power failure” means a reduction in power or a loss in power preventing the traffic control signal system from operating as intended;

“traffic control signal” has the same meaning as in the Highway Traffic Act;

“traffic control signal system” has the same meaning as in the Highway Traffic Act.  O. Reg. 612/06, s. 13 (4).

Traffic control signal system sub-systems

14.  (1)  The minimum standard is to inspect, test and maintain the following traffic control signal system sub-systems every 12 months:

1.	The display sub-system, consisting of traffic signal and pedestrian crossing heads,  physical support structures and support cables.

2.	The traffic control sub-system, including the traffic control signal cabinet and internal devices such as timer, detection devices and associated hardware, but excluding conflict monitors.

3.	The external detection sub-system, consisting of detection sensors for all vehicles, including emergency and railway vehicles and pedestrian push-buttons.  O. Reg. 612/06, s. 14 (1).

(2)  The minimum standard is to inspect, test and maintain conflict monitors every five to seven months and at least twice a year.  O. Reg. 612/06, s. 14 (2).

(3)  In this section,

“conflict monitor” means a device that continually checks for conflicting signal indications and responds to a conflict by emitting a signal.  O. Reg. 612/06, s. 14 (3).

Bridge deck spalls

15.  (1)  If a bridge deck spall exceeds both the surface area and depth set out in the Table to this section, the minimum standard is to repair the bridge deck spall within the time set out in the Table after becoming aware of the fact.  O. Reg. 612/06, s. 15 (1).

(2)  A bridge deck spall is deemed to be repaired if its surface area or depth is less than or equal to that set out in the Table.  O. Reg. 612/06, s. 15 (2).

(3)  In this section,

“bridge deck spall” means a cavity left by one or more fragments detaching from the paved surface of the roadway or shoulder of a bridge.  O. Reg. 612/06, s. 15 (3).

TABLE
BRIDGE DECK SPALLS

| Class of Highway | Surface Area | Depth | Time |
| --- | --- | --- | --- |
| 1 | 600 cm² | 8 cm | 4 days |
| 2 | 800 cm² | 8 cm | 4 days |
| 3 | 1,000 cm² | 8 cm | 7 days |
| 4 | 1,000 cm² | 8 cm | 7 days |
| 5 | 1,000 cm² | 8 cm | 7 days |

O. Reg. 612/06, s. 15, Table.

Roadway surface discontinuities

16.  (1)  If a surface discontinuity on a roadway, other than a surface discontinuity on a bridge deck, exceeds the height set out in the Table to this section, the minimum standard is to repair the surface discontinuity within the time set out in the Table after becoming aware of the fact.  O. Reg. 22/10, s. 9.

(2)  If a surface discontinuity on a bridge deck exceeds five centimetres, the minimum standard is to deploy resources as soon as practicable after becoming aware of the fact to repair the surface discontinuity on the bridge deck.  O. Reg. 22/10, s. 9.

(3)  In this section,

“surface discontinuity” means a vertical discontinuity creating a step formation at joints or cracks in the paved surface of the roadway, including bridge deck joints, expansion joints and approach slabs to a bridge.  O. Reg. 22/10, s. 9.

TABLE
SURFACE DISCONTINUITIES

| Class of Highway | Height | Time |
| --- | --- | --- |
| 1 | 5 cm | 2 days |
| 2 | 5 cm | 2 days |
| 3 | 5 cm | 7 days |
| 4 | 5 cm | 21 days |
| 5 | 5 cm | 21 days |

O. Reg. 612/06, s. 16, Table.

Sidewalk surface discontinuities

16.1  (1)  The minimum standard for the frequency of inspecting sidewalks to check for surface discontinuity is once per year.  O. Reg. 22/10, s. 10.

(2)  If a surface discontinuity on a sidewalk exceeds two centimetres, the minimum standard is to treat the surface discontinuity within 14 days after becoming aware of the fact.  O. Reg. 22/10, s. 10.

(3)  For the purpose of subsection (2), treating a surface discontinuity on a sidewalk means taking reasonable measures to protect users of the sidewalk from the discontinuity, including making permanent or temporary repairs, alerting users’ attention to the discontinuity or preventing access to the area of discontinuity.  O. Reg. 22/10, s. 10.

(4)  In this section,

“surface discontinuity” means a vertical discontinuity creating a step formation at joints or cracks in the surface of the sidewalk.  O. Reg. 22/10, s. 10.

17.  Omitted (provides for coming into force of provisions of this Regulation).  O. Reg. 612/06, s. 17.

Back to top