Conservation Authorities Act
Loi sur les offices de protection de la nature

R.R.O. 1990, REGULATION 147

Amended to O. Reg. 535/91

FILL, CONSTRUCTION AND ALTERATION TO WATERWAYS — ESSEX REGION

Historical version for the period September 23, 1991 to May 3, 2006.

This Regulation is made in English only.

1.  In this Regulation,

“Authority” means the Essex Region Conservation Authority;

“building or structure” means a building or structure of any kind;

“drainage area” means, for a point, the area which contributes runoff to that point;

“fill” means earth, sand, gravel, building materials, storage materials, rubble, rubbish, garbage or any other material whether similar to or different from any of the aforementioned materials, whether originating on the site or elsewhere, used or capable of being used to raise, lower or in any way affect the contours of the ground;

“fill line” means any line designated as such on the maps referred to in the Schedules;

“mm” means millimetres;

“regional storm” means a storm producing in a forty-eight hour period in a drainage area of,

(a)	twenty-five square kilometres or less, a rainfall that has the distribution set out in Table 1, or

(b)	more than twenty-five square kilometres, a rainfall such that the number of millimetres of rain referred to in each case in Table 1 shall be modified by the percentage amount shown in Column 2 of Table 2 opposite the size of the drainage area set out opposite thereto in Column 1 of Table 2:

Table 1

73 mm of rain in the first 36 hours

6 mm of rain in the 37th hour

4 mm of rain in the 38th hour

6 mm of rain in the 39th hour

13 mm of rain in the 40th hour

17 mm of rain in the 41st hour

13 mm of rain in the 42nd hour

23 mm of rain in the 43rd hour

13 mm of rain in the 44th hour

13 mm of rain in the 45th hour

53 mm of rain in the 46th hour

38 mm of rain in the 47th hour

13 mm of rain in the 48th hour

Table 2

| Column 1 | Column 2 |
| --- | --- |
| Drainage Area (in square kilometres) | Percentage |
| 26 to 45 both inclusive | 99.2 |
| 46 to 65 both inclusive | 98.2 |
| 66 to 90 both inclusive | 97.1 |
| 91 to 115 both inclusive | 96.3 |
| 116 to 140 both inclusive | 95.4 |
| 141 to 165 both inclusive | 94.8 |
| 166 to 195 both inclusive | 94.2 |
| 196 to 220 both inclusive | 93.5 |
| 221 to 245 both inclusive | 92.7 |
| 246 to 270 both inclusive | 92.0 |
| 271 to 450 both inclusive | 89.4 |
| 451 to 575 both inclusive | 86.7 |
| 576 to 700 both inclusive | 84.0 |
| 701 to 850 both inclusive | 82.4 |
| 851 to 1000 both inclusive | 80.8 |
| 1001 to 1200 both inclusive | 79.3 |
| 1201 to 1500 both inclusive | 76.6 |
| 1501 to 1700 both inclusive | 74.4 |
| 1701 to 2000 both inclusive | 73.3 |
| 2001 to 2200 both inclusive | 71.7 |
| 2201 to 2500 both inclusive | 70.2 |
| 2501 to 2700 both inclusive | 69.0 |
| 2701 to 4500 both inclusive | 64.4 |
| 4501 to 6000 both inclusive | 61.4 |
| 6001 to 7000 both inclusive | 58.9 |
| 7001 to 8000 both inclusive | 57.4 |

“river”, “lake”, “creek”, “stream” or “watercourse” means any river, lake, creek, stream or watercourse under the jurisdiction of the Authority.  R.R.O. 1990, Reg. 147, s. 1.

2.  The areas described in the Schedules are areas in which, in the opinion of the Authority, the control of flooding or pollution or the conservation of land may be affected by the placing or dumping of fill.  R.R.O. 1990, Reg. 147, s. 2.

3.  Subject to section 4, no person shall,

(a)	construct any building or structure or permit any building or structure to be constructed in or on a pond or swamp or in any area susceptible to flooding during a regional storm;

(b)	place or dump fill, or permit fill to be placed or dumped, permanently or temporarily, in the areas described in Schedule 1, whether such fill is already located in or upon such area or brought to or on such area from some other place or places; or

(c)	straighten, change, divert or interfere in any way with the existing channel of a river, lake, creek, stream or watercourse.  R.R.O. 1990, Reg. 147, s. 3.

4.  Subject to the Ontario Water Resources Act or to any private interest, the Authority may permit in writing the construction of any building or structure, or the placing or dumping of fill, or the straightening, changing, diverting or interfering with the existing channel of a river, lake, creek, stream or watercourse, to which section 3 applies if, in the opinion of the Authority, the site of the building or structure or the placing or dumping of fill and the method of construction or placing or dumping of fill or the straightening, changing, diverting or interfering with the existing channel will not affect the control of flooding or pollution or the conservation of land.  R.R.O. 1990, Reg. 147, s. 4.

5.  No person shall commence to construct any building or structure, or dump or place fill, or straighten, change, divert or interfere with the existing channel of a river, lake, creek, stream or watercourse, in any area to which section 3 applies before permission to do so has been obtained under section 4.  R.R.O. 1990, Reg. 147, s. 5.

6.  (1)  A signed application for permission to construct a building or structure shall be filed with the Authority and shall include,

(a)	four copies of a plan of the property showing the proposed location of the building or structure, its elevation and the proposed final grade plan;

(b)	four copies of a complete description of the type of building or structure to be constructed, including drainage details and the method of construction;

(c)	four copies of a statement of the dates between which the construction will be carried out; and

(d)	four copies of a statement of the proposed use of the building or structure following completion of the construction.  R.R.O. 1990, Reg. 147, s. 6 (1).

(2)  A signed application for permission to place or dump fill shall be filed with the Authority and shall include,

(a)	four copies of a plan of the property on which the fill is to be placed, showing the proposed location of filling, the depth to which it is proposed to fill and the proposed final grade of the land when filling is completed;

(b)	four copies of a complete description of the type of fill proposed to be placed or dumped and the method of placing or dumping the fill;

(c)	four copies of a statement of the dates between which the placing or dumping will be carried out; and

(d)	four copies of a statement of the proposed use of the land following completion of placing or dumping.  R.R.O. 1990, Reg. 147, s. 6 (2).

(3)  A signed application for permission to straighten, change, divert or interfere in any way with the existing channel of a river, lake, creek, stream or watercourse shall be filed with the Authority and shall include,

(a)	four copies of a plan on which shall be shown in plan view and cross section the details of the straightening, change, diversion or interference;

(b)	four copies of a description of the protective measures to be undertaken and the method to be used to carry out the straightening, change, diversion or interference;

(c)	four copies of a statement of the dates between which the straightening, changing, diverting or interfering will be carried out; and

(d)	four copies of a statement of the purpose of the proposed work.  R.R.O. 1990, Reg. 147, s. 6 (3).

7.  The Authority may, at any time, withdraw any permission given under section 4 if, in the opinion of the Authority, the representations contained in the application for permission are not carried out.  R.R.O. 1990, Reg. 147, s. 7.

8.  Members of the staff of the Authority are appointed as officers to enforce this Regulation.  R.R.O. 1990, Reg. 147, s. 8.

Schedule 1
LITTLE RIVER

That part of the watershed of Little River in the City of Windsor and the townships of Sandwich East and Sandwich South, in the County of Essex, that affects the following lots, as shown delineated by fill lines on the maps filed in the Regional Office of the Ministry of Natural Resources at London, Ontario as Maps ER1-1 to ER1-6, inclusive, and identified by a stamp of the Registrar of Regulations dated the 23rd day of January, 1991.

| Concession |  | Lot |
| --- | --- | --- |
| I. | (Plan of the Township of Sandwich East) | Parts of 133, including part of lots 1 to 71, both inclusive, and part of Block B, Reg. Plan No. 1533, and including parts of lots 1 to 8, both inclusive, Plan M-113 |
|  |  | Parts of 129 West, 132 and 134, including part of Lot 1 and part of lots 3 to 20, both inclusive, Plan No. 1501 |
| II. | (Plan of the Township of Sandwich East) | Parts of 128, including parts of Block A, Plan M-26 and including all of Plan M-142 |
|  |  | Parts of 125, 126, 127, 129, 132 and 133 |
| III. | (Plan of the Township of Sandwich East) | All of 125, 126, 127, 134 and 135 |
| IX. | (Plan of the Township of Sandwich South) | Parts of 14 to 18, both inclusive |
|  |  | All of Lot 19 |

O. Reg. 535/91, s. 1.

Schedule 2
CEDAR, WIGLE AND MILL CREEKS

That part of the watersheds of Cedar, Wigle and Mill Creeks in the Town of Kingsville and the townships of Gosfield South and Colchester South, in the County of Essex, that affects the following lots, as shown delineated by fill lines on the maps filed in the Regional Office of the Ministry of Natural Resources at London, Ontario as Maps ER2-1 to ER2-26, inclusive, and identified by a stamp of the Registrar of Regulations dated the 23rd day of January, 1991:

| Concession | Lot |
| --- | --- |
| Front Concession, Western Division, in the Town of Kingsville | Parts of 1 to 4, both inclusive |
| Front Concession, Western Division (Plan of the Township of Gosfield South) | Parts of 4 to 32, both inclusive |
| 4th Section, Western Division (Plan of the Township of Gosfield South) | Parts of 4, 5, 7, 8, 9, 10, 16 and 17 |
| Front Concession, Eastern Division (Plan of the Township of Gosfield South) | Parts of 1 to 6, both inclusive |
| I. | Parts of 33 to 39, both inclusive |
| (Plan of the Township of Colchester South) |  |
| II. | Parts of 17 to 19, both inclusive |
| (Plan of the Township of Colchester South) |  |
| III. | Parts 14 to 19, both inclusive |
| (Plan of the Township of Colchester South) |  |
| IV. | Parts of 13 to 19, both inclusive |
| (Plan of the Township of Colchester South) |  |

O. Reg. 535/91, s. 2.

Schedule 3
PIKE CREEK, PUCE RIVER, FOURTH CONCESSION DRAIN, BELLE RIVER, DUCK CREEK, MOISON CREEK AND RUSCOM RIVER

That part of the watersheds of Pike Creek, Puce River, Fourth Concession Drain, Belle River, Duck Creek, Moison Creek and Ruscom River in townships of Sandwich South, Maidstone, Rochester and Gosfield North, the Village of St. Clair Beach and the Town of Belle River, in the County of Essex, that affects the following lots, as shown delineated by fill lines on the maps filed in the Regional Office of the Ministry of Natural Resources at London, Ontario as Maps ER3-1 to ER3-16, inclusive, ER3-18, ER3-20, ER3-21, ER3-22, ER3-24, ER3-24a, ER3-25, ER3-26, ER3-28, ER3-29, ER3-30, ER3-31, ER3-32, ER3-34, ER3-35, ER3-36, ER3-37, ER3-39 to ER3-45, inclusive, ER3-47, ER3-48, ER3-50, ER3-51 and ER3-52, and identified by a stamp of the Registrar of Regulations dated the 23rd day of January, 1991:

| Concession | Lot |
| --- | --- |
| X. | Parts of 9 to 13, both inclusive |
| (Plan of the Township of Sandwich South) |  |
| XI. | Parts of 9 to 19, both inclusive |
| (Plan of the Township of Sandwich South) |  |
| XII. | Parts of 9 to 20, both inclusive |
| (Plan of the Township of Sandwich South) |  |
| North Talbot Road Concession (Plan of the Township of Sandwich South) | Parts of 292 to 297, both inclusive |
| West Pike Creek Concession (Village of St. Clair Beach) | Parts of Gore Lot, 1 and 2 |
| West Pike Creek Concession (Plan of the Township of Maidstone) | Parts of 1 to 10, both inclusive |
| East Pike Creek Concession (Plan of the Township of Maidstone) | Parts of 1 to 10, both inclusive |
| North Middle Road Concession (Plan of the Township of Maidstone) | Parts of 1, 2, 11, 12 and 22 |
| South Middle Road Concession (Plan of the Township of Maidstone) | Parts of 1, 2, 11, 12 and 19 to 22, both inclusive |
| North Talbot Road Concession (Plan of the Township of Maidstone) | Parts of 292, 293 and 294 |
| West Puce River Concession (Plan of the Township of Maidstone) | Parts of B and 1 to 7, both inclusive |
| East Puce River Concession (Plan of the Township of Maidstone) | Parts of 1 to 7, both inclusive |
| Concession between the Belle River | Parts of 1 to 3, both inclusive |
| Concession and the East Puce River |  |
| Concession fronting on Lake St. Clair |  |
| (Plan of the Township of Maidstone) |  |
| I. | Parts of A, 1 to 20 and Part of 22, and 24 to 29, all inclusive |
| (Plan of the Township of Maidstone) |  |
| II. | Parts of 11, 12 and 24 |
| (Plan of the Township of Maidstone) |  |
| III. | Part of 11 |
| (Plan of the Township of Maidstone) |  |
| I. | Part of 11 |
| (Plan of the Township of Maidstone) |  |
| V. | Parts of 11 to 19, both inclusive, and Part of 24 |
| (Plan of the Township of Maidstone) |  |
| VIII. | Parts of 11 and 12 |
| (Plan of the Township of Maidstone) |  |
| IX. | Parts of 11 and 19 |
| (Plan of the Township of Maidstone) |  |
| I. | Part of A, 2 and 3 |
| West of Belle River in the Town of Belle River | All of 1 |
| I. | Part of A and 1 |
| East of Belle River in the Town of Belle River | Parts of 2 and 3 |
| Lakeshore Range Concession (Plan of the Township of Rochester) | Parts of 1 to 8, both inclusive |
| West Ruscom River Concession | Parts of A and 3 to 8, both inclusive |
| (Plan of the Township of Rochester) | All of 1 and 2 |
| East Ruscom River Concession (Plan of the Township of Rochester) | Parts of A, 1, 2, 3 and 4 to 8, all inclusive |
| North Middle Road Concession | Parts of 1, 3, 4, and 9 to 13, both inclusive |
| (Plan of the Township of Rochester) | All of 14 to 17, both inclusive |
| South Middle Road Concession (Plan of the Township of Rochester) | Parts of 1, 4, 9 to 11, both inclusive, and 15 to 17, both inclusive |
|  | All of 12 to 14, both inclusive |
| I. | Parts of A, 1 to 20, both inclusive, and 25 to 30, both inclusive |
| (Plan of the Township of Rochester) |  |
| II. | Parts of 11 and 12 |
| (Plan of the Township of Rochester) |  |
| III. | Parts of 11 and 12 |
| (Plan of the Township of Rochester) |  |
| IV. | Parts of 12, 15 to 17, both inclusive |
| (Plan of the Township of Rochester) | Parts of 28 to 30, both inclusive |
| V. | Parts of 12 to 15, and 26 to 28, all inclusive |
| (Plan of the Township of Rochester) |  |
| VI. | Parts of 13, 15 to 21 and 25 to 30, all inclusive |
| (Plan of the Township of Rochester) |  |
| VII. | Parts of 12, 13 and 25 to 30, all inclusive |
| (Plan of the Township of Rochester) |  |
| VIII. | Part of 29 |
| (Plan of the Township of Rochester) | All of 30 |
| North Talbot Road Concession (Plan of the Township of Gosfield North) | Part of 270 |
| VII. | Parts of 15 to 20 and 23 to 25, all inclusive |
| (Plan of the Township of Gosfield North) |  |
| VIII. | All of 12 and 13 to 25, all inclusive |
| (Plan of the Township of Gosfield North) |  |
| IX. | Parts of 12 to 17, and 19 to 21, all inclusive |
| (Plan of the Township of Gosfield North) |  |
| X. | Parts of 13 to 16, both inclusive, and 19 to 25, both inclusive |
| (Plan of the Township of Gosfield North) |  |
| XI. | Parts of 12 to 14, 20 and 21 to 25, all inclusive |
| (Plan of the Township of Gosfield North) |  |

O. Reg. 535/91, s. 2.

Schedule 4
STURGEON CREEK AND SELKIRK DRAIN

That part of the watersheds of Sturgeon Creek and Selkirk Drain in the Town of Leamington and the Townships of Mersea and Gosfield South, in the County of Essex, that affects the following lots, as shown delineated by fill lines on the maps filed in the Regional Office of the Ministry of Natural Resources at London, Ontario as Maps ER4-1 to ER4-6, inclusive, and identified by a stamp of the Registrar of Regulations dated the 23rd day of January, 1991:

| Concession | Lot |
| --- | --- |
| A. | Parts of 11 and 12 |
| (Plan of the Township of Mersea) |  |
| B. | Parts of 10 and 11 |
| (Plan of the Township of Mersea) |  |
| I. | Parts of 7, 8 and 10 to 12, all inclusive |
| (Plan of the Township of Mersea) |  |
| III. | Parts of 1 to 6, both inclusive |
| (Plan of the Township of Mersea) |  |
| South Talbot Road Concession (Plan of the Township of Mersea) | Parts of 236 to 240, both inclusive |
| North Talbot Road Concession (Plan of the Township of Mersea) | Parts of 238 to 240 and 242 to 244, all inclusive |
| North Talbot Road Concession (Plan of the Town of Leamington) | Parts of 243 and 244 |
| III. | Part of 13 |
| (Plan of the Township of Gosfield South) |  |
| Broken Front Concession (Plan of the Township of Mersea) | Parts of 7 and 8 |

O. Reg. 535/91, s. 2.

Schedule 5
CANARD RIVER AND BIG CREEK

That part of the watersheds of Canard River and Big Creek in the County of Essex, that affects the following lots, as shown delineated by fill lines on the maps filed in the Regional Office of the Ministry of Natural Resources at London, Ontario as Maps ER5-1 to ER5-18, inclusive, ER5-20 to ER5-29, inclusive, ER5-31 to ER5-49, inclusive, ER5-49A, ER5-50, ER5-52 to ER5-59, inclusive, and ER5-61 to ER5-67, inclusive, and identified by a stamp of the Registrar of Regulations dated the 23rd day of January, 1991:

1.	In the Township of Malden, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| I. | Parts of 5 to 19, both inclusive |
| II. | Parts of 20, 21 and 24 |
| III. | Parts of 25 to 32, both inclusive |
|  | All of 33 |
|  | Part of Caldwell Grant |
| IV. | Parts of 34, 36, 37 and 38 |
|  | All of 35 |
| V. | Parts of 52, 53 and 54 |
|  | All of 55 and 56 |
| VI. | Parts of 57, 58 and 63 |
| VII. | Parts of 59, 60, 77 and 78 |
| VIII. | Parts of 89 to 94 |
| IX. | Part of 96 |
|  | All of 97 to 101, both inclusive |

2.	In the Township of Anderdon, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| I. | Parts of 28 to 42, both inclusive |
| II. | Parts of 1 to 4 and 10 to 14, all inclusive |
| III. | Parts of 1, 4 to 10, both inclusive, and 14 |
| IV. | Parts of 3 to 7, both inclusive, and 11 to 14, both inclusive |
| V. | Parts of 1 to 4, both inclusive, and 9 to 14, both inclusive |
| VI. | Parts of 1, 2, 5 to 10, both inclusive, 12, 13 and 14 |
| VII. | Parts of 4 to 8, both inclusive, 11 and 12 |
| VIII. | Parts of 1 to 6, both inclusive, 8, 11, 12 and 13 |

3.	In the Township of Sandwich West, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| I. | Parts of 1 to 29, both inclusive |
| II. | Parts of 1 to 34, both inclusive |
| III. | Parts of 1 to 6, both inclusive |
| IV. | Parts of 4 to 6, both inclusive |
| V. | Parts of 1 and 2 |
| VI. | Parts of 1, 2 and 3 |
| VII. | Parts of 1 and 2 |
| VIII. | Part of 1 |

4.	 In the Township of Colchester North, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| South Malden Road | Parts of 1, 2 and 3, 6 to 33, all inclusive |
| North Malden Road | Parts of 1 to 11, both inclusive, and 20 to 31, both inclusive |
| VII. | Parts of 3 to 9, both inclusive |
| VIII. | Parts of 7 to 13, both inclusive, and 18 to 21, both inclusive |
| IX. | Parts of 12, 13 and 17 to 21, both inclusive |
| X. | Parts of 1 to 4 and 17 to 21, all inclusive |
| XI. | Parts of 1 to 5 |
| XII. | Parts of 1 to 5 and 11 to 15, all inclusive |
| XIII. | Parts of 1 to 5, both inclusive, and 13 to 17, both inclusive |
| XIV. | Parts of 1 to 5, both inclusive, and 14 to 17, both inclusive |

O. Reg. 535/91, s. 3.

Schedule 6
TURKEY CREEK

That part of the watershed of Turkey Creek in the County of Essex, that affects the following lots, as shown delineated by fill lines on the maps filed in the Regional Office of the Ministry of Natural Resources at London, Ontario as Maps ER6-1 to ER6-13, inclusive, ER6-14, ER6-15A, ER6-15B, ER6-16, ER6-17, ER6-18, ER6-19, ER6-20, ER6-21A, ER6-21B, ER6-22 to ER6-33, inclusive, ER6-34A, ER6-34B, ER6-35 and ER6-36, and identified by the stamp of the Registrar of Regulations dated the 23rd day of January, 1991:

1.	In the City of Windsor, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| II. | Parts of 44 to 55, both inclusive, and 71 to 109, both inclusive |
| III. | Parts of 63 to 78, both inclusive, and 81 to 102, both inclusive |
| IV. | Parts of 1 to 5, both inclusive |
| North of Talbot Road | Parts of 306 to 308, both inclusive |

2.	In the Township of Sandwich West, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| I. | Parts of 30 to 45, both inclusive |
| II. | Parts of 33 to 40, both inclusive |
| IV. | Parts of 1 and 2 |

O. Reg. 535/91, s. 3.

Schedule 7
HILLMAN MARSH AND MUDDY CREEK

That part of the watersheds of Hillman Marsh and Muddy Creek in the Township of Mersea, in the County of Essex, that affects the following lots, as shown delineated by fill lines on the maps filed in the Regional Office of the Ministry of Natural Resources at London, Ontario as Maps ER7-1 to ER7-6, and identified by the stamp of the Registrar of Regulations dated the 23rd day of January, 1991:

| Concession | Lot |
| --- | --- |
| I. | Parts of 18 to 22, both inclusive |
| (Plan of the Township of Mersea) |  |
| II. | Parts of 16 to 22, both inclusive |
| (Plan of the Township of Mersea) |  |
| III. | Part of 19 |
| (Plan of the Township of Mersea) |  |
| IV. | Parts of 22 to 24, both inclusive |
| (Plan of the Township of Mersea) |  |
| South Talbot Road (Plan of the Township of Mersea) | Parts of 221 to 223 and 225 to 235, all inclusive |
| North Talbot Road (Plan of the Township of Mersea) | Parts of 223 and 227 to 230 and 235, 236, all inclusive |

O. Reg. 535/91, s. 4.

Schedule 8
LAKE ERIE, DETROIT RIVER AND LAKE ST. CLAIR

That part of the watersheds of Lake Erie, the Detroit River and Lake St. Clair, in the County of Essex, that affects the following lots, as shown delineated by fill lines on the maps filed in the Regional Office of the Ministry of Natural Resources at London, Ontario as Maps ER8-1, ER8-2, ER8-5, ER8-8, ER8-9, ER8-14 to ER8-20, inclusive, ER8-23, ER8-27 to ER8-33, inclusive, ER8-35, ER8-38, ER8-43, ER8-44, ER8-45, ER8-46, ER8-47, ER8-48, ER8-50 to ER8-64, inclusive, ER8-64a, ER8-64b, ER8-65, ER8-65a, ER8-66, ER8-66a, and ER8-67 to ER8-75, inclusive, and identified by the stamp of the Registrar of Regulations dated the 23rd day of January, 1991:

1.	In the Township of Mersea, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| I. | Part of Lot 1 |
| II. | Part of the Lot 23 and 24 |
| III. | Part of Lot 22 |
| A. | Part of Lot 22 |
| B. | Part of Lot 11 and 21 |
| C. | Part of Lot 12, 13 and 21 |
| D. | Part of Lot 13 to 21, all inclusive |
| Broken Front Concession | Part of Lot 1 to 6 and 9 to 12, all inclusive |

2.	In the Town of Leamington, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| Broken Front Concession | Part of Lot 6 to 8, all inclusive |

3.	In the Township of Gosfield South, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| Front Concession Eastern Division | Part of Lot 3 to 13, all inclusive |
| Front Concession Western Division | Part of Lot 5 to 32, all inclusive |

4.	In the Town of Kingsville, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| Front Concession Eastern Division | Part of Lot 1 and 2 |
| Front Concession Western Division | Part of Lot 1 to 5, both inclusive |

5.	In the Township of Colchester South, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| Front Concession | Part of Lot 33 to 97, all inclusive |

6.	In the Township of Malden, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| I. | Part of Lot 5 to 19, all inclusive |
| V. | Part of Lot 56 |
| VI. | Part of Lot 57 and 58 |
| VII. | Part of Lot 59 and 60 |
| Plan of the Township of Malden | Part of the Caldwell Grant, including Registered Plans 1103, 1379, 1131, 1191, 795 |

7.	In the Town of Amherstburg, County of Essex, and being comprised of the following lots:

| Concession |  | Lot |
| --- | --- | --- |
| I. | (formerly Malden Township) | Part of Lot 1 to 4 |
| I. | (formerly Anderdon Township) | Part of Lot 1 to 5 |

8.	In the Township of Anderdon, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| I. | Part of Lot 5 to 42 |

9.	 In the Township of Sandwich West, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| I. | Part of Lot 1 to 40, all inclusive |

10.	In the City of Windsor, County of Essex, and being comprised of the following lots:

| Concession |  | Lot |
| --- | --- | --- |
| I. | (formerly Sandwich West) | Part of lot 40 to 128, 129E, 129W and 130 to 149, all inclusive |

11.	In the Town of Tecumseh, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| I. | Part of Lot 150 to 156, all inclusive |

12.	In the Village of St. Clair Beach, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| West Pike Creek | Part of A, 1 and 2 |

13.	In the Township of Maidstone, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| West Pike Creek | Part of Lot 2 and 3, all inclusive |
| All of Broken Front Concession | All lots |
| East Pike Creek | Part of Lot 1 to 3, all inclusive |
| Concession between Pike Creek and Puce River fronting on Lake St. Clair | Part of Lot 1 to 4, all inclusive |
| West Puce River | All of Lot B, Part of Lot 1 and 2 |
| East Puce River | All of Lot B and Lot 1, Part of Lot 2 and 3 |
| Concession between Belle River and Puce River fronting on Lake St. Clair | Part of Lot 1 to 3, all inclusive |
| I. | All of Lot 1, Part of Lot A and 2 |

14.	In the Town of Belle River, County of Essex, and being comprised of the following lots:

| Concession |  | Lot |
| --- | --- | --- |
| I. | West of Belle River | Part of A |
| I. | East of Belle River | Part of A |

15.	In the Township of Rochester, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| I. | Part of Lot A and Lot 1 |
| West Ruscom River | All of Lot A, Lot 1, 2, Part of Lot 3 |
| Concessions between Ruscom River and Belle River fronting on Lake St. Clair | Part of Lot 1 to 8, all inclusive |
| East of Ruscom River | Part of Lot A and Lot 1 to 3, all inclusive |

16.	In the Township of Tilbury North, County of Essex, and being comprised of the following lots:

| Concession | Lot |
| --- | --- |
| Broken Front Concession | Part of Lot 1 to 8, all inclusive |
|  | All of lot 9 to 15, all inclusive |

O. Reg. 535/91, s. 4.

Schedule 9
LITTLE CREEK

That part of the watershed of Little Creek in the Township of Tilbury North, in the County of Essex, that affects the following lots, as shown delineated by fill lines on the maps filed in the Regional Office of the Ministry of Natural Resources at London, Ontario as Maps ER9-1, ER9-2 and ER9-3, and identified by the stamp of the Registrar of Regulations dated the 23rd day of January, 1991:

| Concession |  | Lot |
| --- | --- | --- |
| I. | (Plan of the Township of Tilbury North) | Part of 11 and 12, both inclusive |
| II. | (Plan of the Township of Tilbury North) | Part of 11 and 12, both inclusive |
| III. | (Plan of the Township of Tilbury North) | Part of 11 and 12, both inclusive |
| IV. | (Plan of the Township of Tilbury North) | Part of 11 and 12, both inclusive |

O. Reg. 535/91, s. 4.

Schedule 10
LAKE ERIE — PELEE ISLAND

That part of the watershed of Lake Erie in the Township of Pelee, in the County of Essex, that affects the following lots, as shown delineated by fill lines on the maps filed in the Regional Office of the Ministry of Natural Resources at London, Ontario as Maps ER10-1 to ER10-10, inclusive, and identified by the stamp of the Registrar of Regulations dated the 23rd day of January, 1991:

— All of lots 50 to 52, Registered Plan 338

— All of beach lots ‘A’ to ‘H’, Registered Plan 338

— Part of lots 1, 46, Registered Plan 338

— All of Registered Plan 459

— All of lots ‘M’ to ‘O’, Registered Plan 1240

— All of lots ‘P’ to ‘R’, Registered Plan 1499

— All of lots 70, 367 to 385, Registered Plan 35

— Part of lots 1 to 8, 14, 15 to 21, 23, 27, 29, 30 to 33, 34 to 41, 69, Registered Plan 35

— All of Registered Plan 1011

— Part of lots 15 and 25, Registered Plan 384

— Part of lots 3 to 5, Registered Plan 228

— Part of Block ‘C’, Registered Plan 1180

— All of Registered Plan 1534

— Part of Lot 10, Registered Plan 494

— All of beach lots ‘A’ to ‘D’, Registered Plan 328

— Part of lots 1, 2 and 5, Registered Plan 328

— All of lots 20-9, 20-12, 20-15, 20-17, 20 to 30, Registered Plan 1333

— All of lots 1 to 20, 20-3, 20-4, Registered Plan 438

O. Reg. 535/91, s. 4.

Back to top