Environmental Protection Act
Loi sur la protection de l’environnement

R.R.O. 1990, REGULATION 349

HOT MIX ASPHALT FACILITIES

Historical version for the period December 31, 1990 to June 13, 2011.

No amendments.

This Regulation is made in English only.

1.  In this Regulation,

“aggregate” means any material, including gravel, slag, limestone, crushed rock, sand, hydrated lime, cement, furnace ash, glass or sulphur, used to produce asphalt paving when mixed with bituminous asphalt;

“HMA facility” means a hot mix asphalt facility with equipment designed to heat and dry aggregate and to mix aggregate with bituminous asphalt to produce asphalt paving material;

“portable HMA facility” means an HMA facility that remains at one location for less than one year;

“reference pressure” means a pressure of 101,325 pascals;

“reference temperature” means a temperature of 25 degrees Celsius;

“suspended particulate matter” means any solid or condensible material carried in the exhaust gases from an HMA facility collected and measured by standard sampling and analytical methods described in the “Source Testing Code” published by the Ministry of the Environment.  R.R.O. 1990, Reg. 349, s. 1.

2.  (1)  Except for the period of fifteen minutes immediately following start-up, no person responsible for an HMA facility shall operate or permit the operation of the facility in such a manner as to emit suspended particulate matter into the air at a concentration in excess of 0.230 grams per cubic metre measured dry and undiluted in the exhaust gases and reported at reference temperature and reference pressure.

(2)  This section does not apply to an HMA facility operating under a certificate of approval issued before the 31st day of December, 1985.  R.R.O. 1990, Reg. 349, s. 2.

3.  Except for a period of fifteen minutes immediately following start-up, no person responsible for an HMA facility shall operate or permit the operation of the facility so that visible material including a water plume and fallout of water droplets emitted into the air from the facility impinges on any point beyond the limits of the property on which the facility is located.  R.R.O. 1990, Reg. 349, s. 3.

4.  No person responsible for an HMA facility shall permit visible solid material emitted from any source other than the facility to pass beyond the limits of the property on which the facility is located.  R.R.O. 1990, Reg. 349, s. 4.

5.  In the event that malfunctioning of an HMA facility or any other operating condition occurs that results in the limits prescribed in section 2, 3 or 4 being exceeded, the person responsible for the facility shall,

(a)	immediately take all possible steps to minimize the extent and duration of the event including, when necessary, reducing the rate at which reclaimed asphalt paving is fed to the facility;

(b)	immediately notify the Director and furnish him or her with particulars of the event; and

(c)	at the earliest opportunity, but not later than seven days after the event, furnish the Director with the particulars of the event in writing.  R.R.O. 1990, Reg. 349, s. 5.

6.  Every person responsible for a portable HMA facility shall,

(a)	keep a copy of the certificate of approval issued for the facility available for inspection at the facility site; and

(b)	give notice of any intended relocation of the facility by completing Form 1 and submitting it to the Director at least fifteen days before the intended relocation.  R.R.O. 1990, Reg. 349, s. 6.

Form 1
PORTABLE HOT MIX ASPHALT FACILITY NOTICE OF INTENDED RELOCATION

Environmental Protection Act

Insert regs\graphics\1990\349\349001au.tif

R.R.O. 1990, Reg. 349, Form 1.

Back to top