Highway Traffic Act
Code de la route

R.R.O. 1990, REGULATION 587

EQUIPMENT

Historical version for the period May 2, 2008 to November 9, 2008.

Last amendment:  O. Reg. 115/08.

This Regulation is made in English only.

Brakes

1.  (1)  In making a brake test a Bear Hydraulic Brake Tester, Cowdrey Dynamic Brake Tester, James Decelerometer, Muether Stopmeter, Tapley Brake Testing Meter, Weaver Brake Tester, or another instrument that is approved by the Minister, shall be used.  R.R.O. 1990, Reg. 587, s. 1 (1).

(2)  The brake test may be made with the vehicle loaded or otherwise in the discretion of the person making the test.  R.R.O. 1990, Reg. 587, s. 1 (2).

2.  (1)  Where the vehicle has a clutch, the brake test shall be made with the clutch disengaged.  R.R.O. 1990, Reg. 587, s. 2 (1).

(2)  Where the vehicle has no clutch, the brake test shall be made without motive power being applied to the driving wheels, except in the case of electrical brakes.  R.R.O. 1990, Reg. 587, s. 2 (2).

3.  The brakes required by section 64 of the Act and this Regulation shall be adequate to stop the vehicle or combination of vehicles referred to in Column 1 of the Table within a distance not greater than the distance set opposite the vehicle or combination of vehicles in Column 2 while being operated at a rate of speed of twenty miles per hour on a dry, smooth, hard asphalt or other paved surface free from loose material and having not more than 1 per cent gradient.

TABLE

| Item | Column 1 | Column 2 |
| --- | --- | --- |
|  | Vehicle | Distance |
| 1. | A motor vehicle having a seating capacity for less than 10 persons | 25 feet |
| 1.1 | A motor tricycle | 25 feet |
| 2. | A motorcycle other than a motor tricycle | 30 feet |
| 3. | A commercial motor vehicle having a registered gross weight of 10,000 pounds or less | 30 feet |
| 4. | A commercial motor vehicle having a registered gross weight of more than 10,000 pounds | 40 feet |
| 5. | A combination of a motor vehicle and a trailer where the trailer has a registered gross weight of 3,000 pounds or less | 40 feet |
| 6. | A combination of a motor vehicle and a trailer where the trailer has a registered gross weight of more than 3,000 pounds or a combination of a motor vehicle and more than one trailer | 50 feet |
| 7. | A combination of a motor vehicle and a mobile home | 50 feet |

R.R.O. 1990, Reg. 587, s. 3; O. Reg. 115/08, s. 1.

4.  Every mobile home when on a highway shall be equipped with brakes adequate to stop and to hold the vehicle as required by this Regulation, and all such brakes and braking systems shall be maintained in good working order.  R.R.O. 1990, Reg. 587, s. 4.

5.  (1)  The push rod stroke of the service brake chamber of a vehicle equipped with wheel brake air chambers shall be not more than the push rod stroke listed in Column 2 of the Schedule for the type of chamber listed in Column 1 of the Schedule if the wheel brake has cam or disc type brakes.  O. Reg. 229/95, s. 1; O. Reg. 276/99, s. 1 (1).

(2)  The push rod stroke of the service brake chamber of a vehicle equipped with wheel brake air chambers shall be not more than the vehicle manufacturer’s maximum push rod stroke if the brake chamber type does not appear in Column 1 of the Schedule.  O. Reg. 229/95, s. 1; O. Reg. 276/99, s. 1 (2).

(3)  If the wheel brakes of a vehicle equipped with wheel brake air chambers have wedge type brakes, the combined movement of both brake shoe linings shall not exceed one-eighth of an inch.  O. Reg. 229/95, s. 1.

(4)  Measurements of wheel brakes under subsections (1), (2) and (3) shall be taken with the vehicle engine turned off, an initial air system pressure between 90 and 100 psi, the park brakes released and the service brake actuator fully applied.  O. Reg. 229/95, s. 1.

(5)  All the wheels of a vehicle manufactured after April 30, 1995 that is equipped with wheel brake air chambers shall have wheel brakes each of which is automatically adjustable.  O. Reg. 229/95, s. 1.

(6)  Each wheel brake referred to in subsection (5) that is equipped with an external adjustment mechanism and has an exposed push rod shall have an indicator that indicates the condition of service brake under-adjustment.  O. Reg. 229/95, s. 1.

(7)  The indicator referred to in subsection (6) must be visible to a person with 20/40 vision who is adjacent to or underneath the vehicle.  O. Reg. 229/95, s. 1.

(8)  No wheel brake shall be removed, rendered partly or wholly inoperable, modified so as to reduce its effectiveness or shall operate improperly.  O. Reg. 229/95, s. 1.

(9)  Brakes shall be adjusted so that the braking power is applied as equally as possible to the wheels on opposite sides of the vehicle.  O. Reg. 229/95, s. 1.

(10)  A motorcycle shall be equipped with two independently actuated service brake systems, one applying at least the front wheel brakes and the other applying at least the rear wheel brakes, unless the motorcycle was manufactured solely with a split-service brake system, within the meaning of Canada Motor Vehicle Safety Standard 122 of the Motor Vehicle Safety Regulations (Canada), and the split-service brake system,

(a)	met the requirements of that Standard at the time it was manufactured;

(b)	has a single actuator; and

(c)	has been maintained in its original condition.  O. Reg. 115/08, s. 2.

Reflectors

6.  A motor vehicle or trailer having a width in excess of eighty inches may display a reflector approved by the Ministry instead of a clearance lamp on the rear of the vehicle.  R.R.O. 1990, Reg. 587, s. 6.

7.  (1)  A trailer shall be equipped with conspicuity markings as follows:

1.	On and after June 1, 2000 and before January 1, 2002, a trailer manufactured after December 1, 1993 whose overall width is greater than 2.05 metres and whose gross vehicle weight rating is greater than 4,500 kilograms shall be equipped with conspicuity markings in accordance with United States Federal Motor Vehicle Safety Standard 108 S. 5.7 that became effective on December 1, 1993 or with Canadian Motor Vehicle Safety Standard 108 that became effective on January 24, 1997.

2.	On and after January 1, 2002, every trailer whose overall width is greater than 2.05 metres and whose gross vehicle weight rating is greater than 4,500 kilograms shall be equipped with conspicuity markings in accordance with United States Federal Motor Vehicle Safety Standard 108 S. 5.7 that became effective on December 1, 1993 or with Canadian Motor Vehicle Safety Standard 108 that became effective on January 24, 1997.  O. Reg. 305/00, s. 1.

(2)  Subsection (1) does not apply to a trailer designed exclusively for living or office use.  O. Reg. 305/00, s. 1.

Non-Application of Section 103 of the Act

8.  Subsection 103 (1) of the Act does not apply to,

(a)	a commercial motor vehicle, or a trailer drawn by it, registered in the name of or operated under any form of contract on behalf of,

(i)	a department of a provincial government,

(ii)	a department of the government of Canada,

(iii)	a board or commission the members of which are appointed by the Governor General in Council or a Lieutenant Governor in Council,

(iv)	a municipality,

(v)	a board or commission the members of which are appointed by a municipality, or

(vi)	a public service corporation, board or commission,

if the vehicle or trailer bears a mark or sign indicating ownership;

(b)	a commercial motor vehicle registered in the name of or operated under any form of contract on behalf of a funeral director;

(c)	a motor vehicle classified for registration purposes as a dual-purpose vehicle other than one licensed as a public vehicle or public truck;

(d)	a commercial motor vehicle operated under a written lease or a written agreement for the exclusive use of a person other than the owner where the vehicle bears a mark or sign indicating ownership;

(e)	a motor vehicle designed, equipped and used exclusively for living accommodation, commonly known as a motorized mobile home;

(f)	a commercial motor vehicle to which is attached temporarily or permanently, a structure designed, used and maintained as a mobile dwelling unit commonly known as a camper; or

(g)	a commercial motor vehicle having a registered gross weight of up to and including 18,000 pounds, other than one licensed as a public vehicle or a public truck.  R.R.O. 1990, Reg. 587, s. 8.

Non-Application of Section 78 of the Act

9.  Section 78 of the Act does not apply to a television receiving set or a television set where either is used only,

(a)	as an aid for the safe and efficient operation of a motor vehicle; or

(b)	in carrying out a service or conducting a business where the use,

(i)	does not involve recreation or entertainment, and

(ii)	does not affect the safe operation of the motor vehicle.  R.R.O. 1990, Reg. 587, s. 9.

Daytime Running Light Systems

10.  (1)  No person shall sell or offer for sale a daytime running light system that does not conform to the requirements of the Canadian Standards Association Standard CAN/CSA-D603-88, Daytime Running Light Systems.  R.R.O. 1990, Reg. 587, s. 10 (1).

(2)  For the purpose of subsection (1), a daytime running light system means a group of electrical components including the control unit designed for installation on a motor vehicle to automatically activate lights used to improve the conspicuity of the vehicle viewed from the front at all times other than when headlights are required.  R.R.O. 1990, Reg. 587, s. 10 (2).

11.  (1)  In this section,

“motor tricycle” means a motorcycle that,

(a)	is designed to travel on three wheels in contact with the ground,

(b)	has seating on which all occupants must sit astride,

(c)	has not more than four designated seating positions,

(d)	has a manufacturer’s gross vehicle weight rating of 1,000 kilograms or less,

(e)	has a minimum wheel rim diameter of 250 millimetres,

(f)	has a minimum wheel base of 1,016 millimetres, and

(g)	does not have a structure partially or fully enclosing the driver and passenger, other than that part of the vehicle forward of the driver’s torso and the seat backrest.  O. Reg. 115/08, s. 3.

(2)  For the purpose of determining the number of wheels on a motorcycle, two wheels are considered to be one wheel if they are mounted on the same axle and the distance between the centres of their areas of contact with the ground is less than 460 millimetres.  O. Reg. 115/08, s. 3.

(3)  A motorcycle with two front wheels shall not be operated on the highway unless it is a motor tricycle that,

(a)	bears the manufacturer’s compliance label issued under section 6 of the Motor Vehicle Safety Regulations (Canada) specifying the type of vehicle as “TRI” for motor tricycle or, in the case of an imported motor tricycle, a compliance label or other label as provided for in section 12 of those Regulations; and

(b)	continues to meet the regulatory standards that applied to it at the time it was manufactured or imported.  O. Reg. 115/08, s. 3.

(4)  A motor tricycle shall not have more seating positions than it had when originally manufactured.  O. Reg. 115/08, s. 3.

(5)  A motor tricycle originally manufactured for sale in Canada shall not have more than two seating positions unless the motor tricycle,

(a)	was originally manufactured with more than two seating positions; and

(b)	bears the manufacturer’s compliance label issued under section 6 of the Motor Vehicle Safety Regulations (Canada) specifying the type of vehicle as “TRI” for motor tricycle.  O. Reg. 115/08, s. 3.

(6)  An imported motor tricycle shall not have more than two seating positions unless the motor tricycle,

(a)	was originally manufactured as a motor tricycle with more than two seating positions; and

(b)	bears a compliance label or other label to prove conformity as provided for in section 12 of the Motor Vehicle Safety Regulations (Canada).  O. Reg. 115/08, s. 3.

(7)  A motor tricycle shall be fitted with at least two mirrors that conform to the requirements set out in Canada Motor Vehicle Safety Standard 111 under the Motor Vehicle Safety Regulations (Canada).  O. Reg. 115/08, s. 3.

(8)  Every motor tricycle shall have a parking brake adequate to hold the vehicle stationary for a few seconds when tested at light throttle in low forward gear and in reverse.  O. Reg. 115/08, s. 3.

(9)  None of the original controls of the brake system of a motor tricycle, including the anti-lock brake controls, shall have been removed, modified or replaced so that their effectiveness is reduced.  O. Reg. 115/08, s. 3.

(10)  In the case of a motor tricycle originally fitted with an electronic stability control system, such system shall not be missing and  there shall be no indication of a malfunction in the system.  O. Reg. 115/08, s. 3.

Schedule 1

| Column 1 |  | Column 2 |
| --- | --- | --- |
| Service Brake Chambers |  | Push Rod Travel |
| Clamp Type Brake Chamber Data |  |  |
| Type | Outside Diameter |  |
| 6 | 4½"(114.3mm) | 1¼"(31.75mm) |
| 9 | 5¼"(133.35mm) | 1⅜"(34.93mm) |
| 12 | 5 11/16"(144.46mm) | 1⅜"(34.93mm) |
| 12 Long Stroke | 5 11/16"(144.46mm) | 1¾"(44.45mm) |
| 16 | 6⅜"(161.93mm) | 1¾"(44.45mm) |
| 16 Long Stroke | 6⅜"(161.93mm) | 2"(50.8mm) |
| 20 | 6 25/32"(172.24mm) | 1¾"(44.45mm) |
| 20 Long Stroke | 6 25/32"(172.24mm) | 2"(50.8mm) |
| 24 | 7 7/32"(183.36mm) | 1¾"(44.45mm) |
| 24 Long Stroke | 7 7/32"(183.36mm) | 2"(50.8mm) |
| 24 Long Stroke with Square Inlet Port or with Square Raised Embossment on Lid | 7 7/32"(183.36mm) | 2½"(63.5mm) |
| 30 | 8 3/32"(205.58mm) | 2"(50.8mm) |
| 30 Long Stroke with Square Inlet Port or with Square Raised Embossment on Lid | 8 3/32"(205.58mm) | 2½"(63.5mm) |
| 36 | 9"(228.6mm) | 2¼"(57.15mm) |
| Bolt Type Chamber Data |  |  |
| Type | Outside Diameter |  |
| A | 6 15/16"(176.21mm) | 1⅜"(34.93mm) |
| B | 9 3/16"(233.36mm) | 1¾"(44.45mm) |
| C | 8 1/16"(204.79mm) | 1¾"(44.45mm) |
| D | 5¼"(133.35mm) | 1¼"(31.75mm) |
| E | 6 3/16"(157.16mm) | 1⅜"(34.93mm) |
| F | 11"(279.4mm) | 2¼"(57.15mm) |
| G | 9⅞"(250.83mm) | 2"(50.8mm) |
| Rotochamber Type Chamber Data |  |  |
| Type | Outside Diameter |  |
| 9 | 4 9/32"(108.74mm) | 1½"(38.1mm) |
| 12 | 4 13/16"(122.23mm) | 1½"(38.1mm) |
| 16 | 5 13/32"(137.32mm) | 2"(50.8mm) |
| 20 | 5 15/16"(150.81mm) | 2"(50.8mm) |
| 24 | 6 13/32"(162.72mm) | 2"(50.8mm) |
| 30 | 7 1/16"(179.39mm) | 2¼"(57.15mm) |
| 36 | 7⅝"(193.68mm) | 2¾"(69.85mm) |
| 50 | 8⅞"(225.43mm) | 3"(76.2mm) |
| Tie Rod Piston Type Chamber Data |  |  |
| Type | Outside Diameter |  |
| 30 Long Stroke with Square Inlet Port | 6½"(165.1mm) | 2½"(63.5mm) |
| DD-3 Type Chamber Data |  |  |
| Type | Outside Diameter |  |
| 30 | 8⅛"(206.37mm) | 2¼"(57.15mm) |

O. Reg. 276/99, s. 2.

Back to top