Trades Qualification and Apprenticeship Act
Loi sur la qualification professionnelle et l’apprentissage des gens de métier

R.R.O. 1990, REGULATION 1077

SHEET METAL WORKER

Historical version for the period December 7, 2010 to December 31, 2010.

Last amendment:  O. Reg. 481/10.

This Regulation is made in English only.

1.  In this Regulation,

“certified trade” means the trade of sheet metal worker;

“sheet metal worker” means a person who,

(a)	manufactures, fabricates, assembles, handles, erects, installs, dismantles, reconditions, adjusts, alters, repairs or services all ferrous and nonferrous sheet metal work of No. 10 U.S. Gauge or of any equivalent or lighter gauge and all other materials used in lieu thereof, and

(b)	reads and understands shop and field sketches used in fabrication and erection, including those taken from original architectural and engineering drawings or sketches or a person who fabricates or installs metal cladding over insulation,

but does not include a person employed in production commonly known as mass production.  R.R.O. 1990, Reg. 1077, s. 1; O. Reg. 246/92, s. 1.

Note:  On January 1, 2011, section 1 is revoked and the following substituted:

Designation as certified trade

1.  The trade of sheet metal worker is designated as a certified trade for the purposes of the Act.  O. Reg. 481/10, s. 1.

See:  O. Reg. 481/10, ss. 1, 9.

2.  The trade of sheet metal worker is designated as a certified trade for the purposes of the Act.  R.R.O. 1990, Reg. 1077, s. 2.

Note:  On January 1, 2011, section 2 is revoked and the following substituted:

Branches of certified trade

2.  The trade of sheet metal worker is composed of the following branches:

1.	Branch 1 (sheet metal worker).

2.	Branch 2 (residential (low rise) sheet metal installer).  O. Reg. 481/10, s. 1.

See:  O. Reg. 481/10, ss. 1, 9.

3.  An apprentice training program is established for the certified trade and shall consist of five periods of related training and work experience training of 1,800 hours for each period,

(a)	in full time educational day courses provided at a college of applied arts and technology, or in courses that in the opinion of the Director, are equivalent thereto in the subjects contained in Schedule 1; and

(b)	in practical training and instruction provided by the employer of the apprentice in the subjects contained in Schedule 2.  R.R.O. 1990, Reg. 1077, s. 3.

Note:  On January 1, 2011, section 3 is revoked and the following substituted:

Scope of trade

3.  (1)  Subject to subsection (2), the performance of any of the following activities constitutes work that is part of the trade of sheet metal worker:

1.	The manufacturing or fabrication of all ferrous and nonferrous sheet metal work of No. 10 U.S. Gauge or any equivalent or lighter gauge and all other materials used in lieu of it.

2.	The assembly, handling, erection, installation, dismantling, reconditioning, adjustment, alteration, repair or servicing of all ferrous and nonferrous sheet metal work of No. 10 U.S. Gauge or any equivalent or lighter gauge and all other materials used in lieu of it.

3.	Reading and interpreting all shop and field sketches used in fabrication and erection, including those taken from sketches or original architectural and engineering drawings.

4.	The fabrication or installation of metal cladding over insulation.  O. Reg. 481/10, s. 1.

(2)  Any activities described in subsection (1) that are performed in production commonly known as mass production do not constitute work that is part of the trade of sheet metal worker.  O. Reg. 481/10, s. 1.

(3)  A person who holds a certificate of qualification in Branch 1 of the trade (sheet metal worker) may perform any of the activities described in subsection (1).  O. Reg. 481/10, s. 1.

(4)  Subject to subsection (5), a person who holds a certificate of qualification in Branch 2 of the trade (residential (low rise) sheet metal installer) may only perform the activities described in paragraphs 2 and 3 of subsection (1) and may only do so with respect to the installation of air handling or ventilation systems.  O. Reg. 481/10, s. 1.

(5)  A person described in subsection (4) may only perform the activities if the following criteria are satisfied:

1.	The work is performed,

i.	within a self-contained single family dwelling such as a single-detached house, semi-detached house, town house or linked house consisting of wood-frame construction, or

ii.	within a low rise residential building,

that does not have common conditioned areas and is four stories or fewer.

2.	The work is performed on a low velocity system with static pressure of no more than one inch water column and maximum air flow of 2,000 cubic feet per minute or on a high velocity system with static pressure of no more than three inches water column and maximum air flow of 2,500 cubic feet per minute.

3.	The air handling or ventilation system does not penetrate any fire-rated separations.  O. Reg. 481/10, s. 1.

See:  O. Reg. 481/10, ss. 1, 9.

4.  The subjects of examination for an apprentice in the certified trade are the subjects contained in Schedules 1 and 2.  R.R.O. 1990, Reg. 1077, s. 4.

Note:  On January 1, 2011, section 4 is revoked and the following substituted:

Apprentice training program

4.  (1)  An apprentice training program is established for the trade of sheet metal worker in accordance with this section.  O. Reg. 481/10, s. 2.

(2)  The apprentice training program for the trade of sheet metal worker shall include the following components:

1.	Classes that provide training and instruction.

2.	On the job work experience.  O. Reg. 481/10, s. 2.

(3)  A period of apprentice training referred to in subsection (4) may include either or both of the components referred to in subsection (2).  O. Reg. 481/10, s. 2.

(4)  The apprentice training program for the trade of sheet metal worker shall consist of the following periods of apprentice training:

1.	In the case of Branch 1 of the trade, five periods, with each period consisting of 1,800 hours of apprentice training.

2.	In the case of Branch 2 of the trade, three periods, with each period consisting of 1,500 hours of apprentice training.  O. Reg. 481/10, s. 2.

(5)  An employer shall not establish an apprentice training program in either Branch of the trade of sheet metal worker unless the program is approved by the Director.  O. Reg. 481/10, s. 2.

See:  O. Reg. 481/10, ss. 2, 9.

5.  The rate of wages for an apprentice in the certified trade, whether for regular daily hours of work or for hours of work in excess of regular daily hours of work, shall be not less than,

(a)	40 per cent during the first period;

(b)	50 per cent during the second period;

(c)	60 per cent during the third period;

(d)	70 per cent during the fourth period; and

(e)	80 per cent during the fifth period,

of the average hourly rate of wages or its equivalent for journeypersons employed by the employer in that trade and with whom the apprentice is working.  R.R.O. 1990, Reg. 1077, s. 5; O. Reg. 74/05, s. 1.

Note:  On January 1, 2011, section 5 is revoked and the following substituted:

Apprentice wages

5.  (1)  The rate of wages to be paid by an employer to an apprentice working in Branch 1 of the trade (sheet metal worker) during the apprentice training program shall not be less than the following percentage of the average hourly rate of wages for journeypersons employed by the employer in that branch of the trade:

1.	During the first 1,800 hours of apprentice training, 40 per cent.

2.	During the second 1,800 hours of apprentice training, 50 per cent.

3.	During the third 1,800 hours of apprentice training, 60 per cent.

4.	During the fourth 1,800 hours of apprentice training, 70 per cent.

5.	During the fifth 1,800 hours of apprentice training, 80 per cent.  O. Reg. 481/10, s. 3.

(2)  The rate of wages to be paid by an employer to an apprentice working in Branch 2 of the trade (residential (low rise) sheet metal installer) during the apprentice training program shall not be less than the following percentage of the average hourly rate of wages for journeypersons employed by the employer in that branch of the trade:

1.	During the first 1,500 hours of apprentice training, 40 per cent.

2.	During the second 1,500 hours of apprentice training, 60 per cent.

3.	During the third 1,500 hours of apprentice training, 80 per cent.  O. Reg. 481/10, s. 3.

See:  O. Reg. 481/10, ss. 3, 9.

Number of apprentices employed by employer

6.  The number of apprentices who may be employed by an employer in the certified trade shall not exceed,

(a)	where the employer is a journeyperson in the trade, one apprentice plus an additional apprentice for every three journeypersons employed by that employer in the trade and with whom the apprentice is working; and

(b)	where the employer is not a journeyperson in the trade, one apprentice for the first journeyperson employed by the employer plus an additional apprentice for each additional three journeypersons employed by that employer in the trade and with whom the apprentice is working.  O. Reg. 74/05, s. 2.

Exemption, s. 10 (2) of Act

7.  A person is exempt from subsection 10 (2) of the Act if he or she,

(a)	applies in the prescribed form for apprenticeship in the certified trade; and

(b)	works in that trade for three months or less.  R.R.O. 1990, Reg. 1077, s. 7.

8.  The Director shall issue a progress record book to each apprentice and the apprentice shall record therein the time that the apprentice spends in related training and work experience and shall be responsible for the safekeeping of the progress record book.  R.R.O. 1990, Reg. 1077, s. 8.

Note:  On January 1, 2011, section 8 is revoked.  See:  O. Reg. 481/10, ss. 4, 9.

9.  An applicant for a certificate of qualification in the certified trade shall submit to the Director evidence satisfactory to the Director of,

(a)	the successful completion of the apprenticeship training program described in section 3; or

(b)	his or her engagement in the certified trade as a journeyperson in Ontario or elsewhere for a period at least equivalent to the total number of hours of which the apprenticeship training program consists.  R.R.O. 1990, Reg. 1077, s. 9; O. Reg. 74/05, s. 3.

Note:  On January 1, 2011, section 9 is revoked and the following substituted:

Application for certification

9.  An applicant for a certificate of qualification in either Branch of the certified trade shall submit to the Director evidence satisfactory to the Director of,

(a)	the successful completion of the applicable apprentice training program described in section 4; or

(b)	his or her engagement in the applicable Branch of the certified trade as a journeyperson in Ontario or elsewhere for a period at least equivalent to the total number of hours of which the apprentice training program consists.  O. Reg. 481/10, s. 5.

See:  O. Reg. 481/10, ss. 5. 9.

Note:  On January 1, 2011, the Regulation is amended by adding the following section:

Transition

10.  (1)  A person who holds a certificate of qualification in the trade of sheet metal worker on December 31, 2010 is deemed on and after January 1, 2011 to hold a certificate of qualification in Branch 1 of the trade of sheet metal worker (sheet metal worker).  O. Reg. 481/10, s. 6.

(2)  A person who holds an expired certificate of qualification in the trade of sheet metal worker on December 31, 2010 is deemed to hold a certificate of qualification in Branch 1 of the trade of sheet metal worker (sheet metal worker) as of the date the person renews their certificate of qualification and pays the fee set out in subsection 27 (1) of Regulation 1055 of the Revised Regulations of Ontario, 1990 (General) made under the Act.  O. Reg. 481/10, s. 6.

(3)  If a certificate of apprenticeship in the trade of sheet metal worker that was issued before January 1, 2011 is still valid immediately before that day, it is deemed on and after January 1, 2011 to be a certificate of apprenticeship in Branch 1 of the trade of sheet metal worker (sheet metal worker).  O. Reg. 481/10, s. 6.

(4)  If a contract of apprenticeship in the trade of sheet metal worker was filed with the Director under clause 9 (1) (b) of the Act on or before December 31, 2010 and, on or before that day, is not fully executed and has not been terminated, cancelled or transferred under section 15 of the Act,

(a)	the contract is deemed on and after January 1, 2011 to be a contract of apprenticeship in Branch 1 of the trade of sheet metal worker (sheet metal worker); and

(b)	every reference in the contract to the trade of sheet metal worker is deemed on and after January 1, 2011 to be a reference to Branch 1 of the trade of sheet metal worker (sheet metal worker).  O. Reg. 481/10, s. 6.

(5)  For the purpose of issuing a certificate of apprenticeship under section 16 of the Act on and after January 1, 2011, a person who completed an apprentice training program in the trade of sheet metal worker before that day is deemed to have completed an apprentice training program in Branch 1 of the trade of sheet metal worker (sheet metal worker).  O. Reg. 481/10, s. 6.

See:  O. Reg. 481/10, ss. 6, 9.

Schedule 1
SHEET METAL WORKER

In-School Training

| Item | Column 1 | Column 2 | Column 3 |
| --- | --- | --- | --- |
|  | Course | Subject | Instruction to be Given |
| 1 | Mathematics (Trade Related) | Mathematics | Addition, subtraction, multiplication, division of whole numbers, fractions, decimals. Metric system; conversion methods. Weights and measures. Ratio and proportion. Percentage, discounts, simple interest. Areas, volumes, linear, perimeter, angular mensuration. Square root. Scale conversion. Algebra fundamentals: simple equations, formulae, shop calculations. Trigonometry; right angled and oblique triangles, formulae, shop calculations. |
| 2 | Science (Trade Related) | Physics | Air; properties, pressure, vacuum, pressure measurement; duct pressures, pressure variation effects on equipment. Water; physical and chemical properties, density, specific gravity, pressure at depth, capillarity. Heat; temperature scale, transmission, effects, measurement, specific heat of metals and air, heat capacity, humidity, expansion. Sound; transmission, acoustic insulation, decibel ratings. Properties of materials; tensile and compressive stress. Basic metallurgy; ferrous and non-ferrous metals, corrosion, electrolysis, electropotential series. Principle of machines; mechanical advantage, efficiency, work, energy and power. Fan motor horse-power requirements. |
| 3 | English | Usage and Business Communication | Reading comprehension. Trade terminology, usage. Sentence, paragraph structure. Letter, report writing. Work and parts orders. Interpretation and use of manufacturer’s manuals and job specifications. Oral communication. |
| 4 | Drafting and Blue Print Reading | Basic Drafting and Interpretation | Use of lines, views, projections, sections, developments, dimensions, lettering. Threads and fasteners. Material specifications. Reading and interpretation of frame, masonry and concrete construction plans; materials, construction members, dimensioning, sections, elevations, details, schedules, standard architectural symbols. Preparation of elementary trade related working drawings, dimensioned sketches of duct work layouts, jigs and fixtures. Material estimates and labour costs. |
| 5 | Trade Practice General | Safety | Safety rules and safe operating procedures. Protective clothing and equipment. First aid. Fire prevention; location, use and maintenance of fire-fighting equipment. The Workers’ Compensation Act. The Building Code Act. The Occupational Health and Safety Act. Handling and storage of flammable liquids, gases, acids and sealants. Safe use of lifting and hoisting equipment, pneumatic and electrical tools and equipment, welding equipment. Good housekeeping. |
|  |  | Hand Tools | Care and use of hammers, mallets, chisels, files, stakes, dies, rivetsetts, dollies, bucking bars, punches, drifts, pliers, hand shears, drills, reamers, taps and dies, hacksaws, pop rivetters. |
|  |  | Power Tools, Equipment and Rigging | Care and use of portable pneumatic and electric drills, grinders, circular and sabre saws, unishears or nibblers. Impact and powder actuated tools. Power shears, bench-masters, brakes, rolls. Edging, forming, locking, beading, swaging, wiring equipment. Bench grinder, drill press, punch press, rivetting tools. Spot, oxyacetylene, arc and carbon arc welding equipment. Materials handling devices, scaffolds, ladders, ropes, slings, hoists. |
|  |  | Measuring and Layout Devices | Care and use of rules, tapes, squares, straightedges, protractors, compasses, dividers, scribers and trammels. Sheet metal and wire gauges. Micrometers, levels, plumb bobs, chalk lines. Patterns and templates. |
| 6 | Trade Practice Pattern Development | Parallel Line Method | Developing patterns for profile, rectangular, round and elliptical forms. Blueprint use for size and profile. Stretch-out pattern methods. Seam, fold and joint allowances. Determining miter line. Types of notches. |
|  |  | Radial Line Method | Developing patterns for regular conical forms and pyramids. Blueprint use for size and profile. Conical form types. Plan and elevation relationship. Determining apex. Seam allowance. Types of notches and pyramid forms. |
|  |  | Triangulation Method | Developing patterns for irregular tapered forms. Blueprint use for form and size. Tapered form types. Plan and elevation relationship to half-pattern. Determining solid and broken lines, hypotenuse for rue lengths and stretchout relationship. Seam and lock allowances. Types of notches. |
|  |  |  | Transition patterns; blueprint use for offset forms and sizes. Types of transitions. Plan and elevation relationship for full pattern. |
|  |  |  | Double angle form patterns; blueprint use for true angle of auxiliary forms. Types of twisted shapes. Plan, elevation, auxiliary and double auxiliary views. |
| 7 | Trade Practice Cutting Methods | Straight Shearing | Use of regular and combination pattern hand shears. Slitting; use of hand, special slitting, lever slitting, hand and power rotary slitting shears, portable unishears and nibblers. Use of double cut pipe shears for split shearing. Square shearing; use of foot and power operated squaring, gap squaring shears and attachments. Inside cuts; starting methods, use of bench, slitting and scroll shears. Shearing bolts, rivets, etc. with cold chisels. |
|  |  | Curved Shearing | Types and use of hand shears. Circle shearing; use of hand and power operated circle shears. Inside cuts; use of hand and power ring and circle shears. Use of power elbow shear for compound curves. |
| 8 | Trade Practice Forming Procedures | Edges (Bench Tools) | Types, uses and methods of forming hemmed, open, burred, turned, flanged and crimped edges. Edge allowances. Use of brakes, bar folders, bending bars, hand and power burring, turning, flanging and crimping equipment. |
|  |  | Shapes (Hand and Bench Tools) | Rolling; rolled form types. Use of solid or slip rolls and funnel forming equipment. Forming on stakes; stake types, uses. Hand tool types and use with stakes. Profile bending; types of profiles and forms, use of hand and power cornice brake. Panning; methods and use of panning equipment. Raising, bumping and stretching; uses of bumping and stretching hammers and dies. Determining layout for bumping. Metal yield point. Annealing methods. |
|  |  | Beading and Swaging | Beading and swaging roll types; reinforcing with beads and swaging, swaging for stops. Use of hand and power beading and swaging equipment. |
|  |  | Wired Edges | Wiring straight edges; edge allowance, wire sizes and gauges, measurement. Use of hand and power wiring, wire and brace bending equipment. False wiring edges of round, rectangular and square containers; false wire allowance. Use of hand and power false wiring and turning equipment. |
|  |  | Reinforcements | Diagonal bending; methods and use for stiffening. Reinforcing methods; use of band, angle, tee and channel iron. Reinforcement fastening methods. |
|  |  | Reinforced Fibreglass | Fibreglass cloth, resins and catalysts. Pot-life. Lay-up and finishing techniques for duct work, fittings and joints. Joint tool use. |
| 9 | Trade Practice Joining Methods | Lock Seams | Groove seaming; seam allowance, groove width, internal and external seams. Use of hand and machine groovers. Brake, mallet and stake grooved seams. Setting down and peening methods; use of hand and power setting down equipment. |
|  |  |  | Double seaming; location and type, seam allowance. Use of hand and power double seaming equipment, slide and snap locks. |
|  |  |  | Box locking; location, box lock allowance and forming methods. |
|  |  |  | Pittsburgh locking; allowance, use of brake and Pittsburgh lock former. Locking methods. |
|  |  |  | Elbow edging; roll types, edging allowance. Use of hand and power elbow edging machines. Elbow seam closing; closing methods for rigid or adjustable elbows. Use of hand and machine closing equipment. |
|  |  |  | Standing seams; types, allowances, fastening devices. Use of seam closing devices. |
|  |  |  | Collar locking; types of collar locks, locking methods, allowances. Use of hand and power collar locking equipment. |
|  |  | Rivetted Seams | Hole spacing and punching methods; use of hand and power punching equipment. |
|  |  |  | Hole drilling methods; drill bit number, letter, fractional sizing methods, angles and clearances. Use of hand and power drilling equipment. |
|  |  |  | Rivetting methods; rivet types, sizing methods, setting and heading. Rivet dollies. Use of hand and power rivetting equipment. |
|  |  | Welding | Spot welding; material preparation, locating and spacing welds. Tips, polarity, metal thickness and amperage settings. Use of portable and stationary spot welding equipment. |
|  |  |  | Oxy-acetylene; regulator pressures, flame and gas mixture types, purpose, adjustment. Welding and brazing rods, fluxes. Basic weld joints; material preparation, allowances, positions. Welding, cutting and brazing techniques. |
|  |  |  | Arc welding; types of equipment, terminology, polarity and A.C./D.C. current requirements, advantages and limitations. Electrodes and filler rods. Flat and horizontal fillet welding techniques. Carbon-arc cutting and brazing. |
|  |  | Soldering | Copper heating methods, equipment. Correct temperature. Forging methods; copper types, shapes, weights. Use of hammer and anvil. Tinning; surface cleaning requirements, dip and tinning solutions. Fluxing; flux types, uses, applications. Defluxing after soldering. Acid use precautions. Solder application; solder types, uses, flow and penetration. Preheating requirements. Soldering types of positioned joints. Surface tinning methods. Testing for leaks. Cleaning soldered joints by hand and power filing, sanding, buffing methods and equipment. |
|  |  | Cementing | Cleaning surfaces for caulking; cleaning material types, uses, application methods. Sealant types and use for high and low pressure duct systems; underground ducts of transite, vitreous clay, plastic; high temperatures. Sealant application methods. |
| 10 | Trade Practice Job-site Assembly and Erection Techniques | Metal Roofing and Copings | Styles of roofs, monitors, towers, minarets, dormers, copings, pediments and corrugated roofs. Metal and plastic roofing types, rubber, lead and canvas. Joints and cleats. Fastening types and methods. Waterproofing methods using grouting, plastic sealants, solder. Vapour barrier types and installation methods. Expansion allowance. Installation of curbs, saddles and roof sleeves. Use of special roofing tools. Safe roof work practices and erection of scaffolding. |
|  |  | Flashings and Skylights | Flashing types, purpose and installation; soakers, step, cape, hip, valley, ridge, gravel stops and nosing. Joint and seam making methods. Caulking, grouting and sealing methods. Types and uses of fasteners and plugs. Skylight types and installation techniques. |
|  |  | Gutters and Downspouts | Gutter types and materials; ogee, round, inlaid. Flat and rake miter making methods. End and outlet installation. Expansion allowance methods. Downspout types, shapes, sizes, materials. Assembly of conductor heads, elbows, shoes, splash pans and scuppers. Installation of gutters and downspouts; erection of scaffolding, types and uses of hangers, spikes, ferrules, straps. Use of plastic and solder sealants. |
|  |  | Plenums and Ducts | Plenum and casing types, construction materials. Types of joints. Stiffening and reinforcing methods. Fastener types and uses. Assembly and installation techniques. Correct relationship to air-handling equipment. Provision of access doors and openings. Gaskets and sealants. |
|  |  | (Square Ducts) | Material types, weights and gauges. Types of cleats. Jointing methods; lapping and fastening. Hanging device types and uses; masonry anchors, bolts, screws, rivets, nails. Use of powder actuated stud guns. Hanger positioning and installation methods. Duct assembly and installation techniques. Positioning and installation of fire dampers. Taping and sealant use. |
|  |  | (Round Ducts) | Common and special material types, including plastics. Jointing methods and joining devices. Hanger and support types, uses and installation techniques. Sealant and taping use. Plastic welding methods. Installation techniques for collector systems, flues, stacks, chimneys and breechings. |
|  |  | (Internal Insulation) | Types, uses and methods of applying duct insulation for acoustic lining, thermal resistance, condensation prevention. Use of sound baffles and manufactured silencers. |
|  |  | (Air Flow Measurement and Balancing) | Normal velocities in low and high pressure duct systems. Average velocities at grilles, through air handling equipment. Material conveying velocities. Static and velocity pressures. Effects of poorly designed elbows, fan fittings, branch take-offs and transitions. |
|  |  | Roof Ventilators | Types and uses; stationary, cowls, gravity, ventilators, louvred penthouse. Motorized roof vents and fan discharge heads. Installation techniques. Use of bird and fly screens. |
|  |  | Exhaust Hoods | Types and uses; standard canopy, fume cabinets. Types and uses of spray booths, drying booths, machine exhaust hoods. Interior and exterior hood materials. Plastic hoods. Methods of jointing and finishing seams, butt and welded joints. Use of hood filters. Hood assembly and installation methods in accordance with applicable codes. Condensation control. |
|  |  | Air Handling Equipment (Fans and Blowers) | Types, functions and characteristics. Installation and mounting methods. Component alignment. Use of manufacturers drawings and specifications for installations. |
|  |  | (Direct Fired Heating Equipment) | Types and characteristics; gravity and forced air units. Heat measurement; quantity (B.T.U.), intensity (temperature). Characteristics of solar, waste products, coal, wood, oil, gas and electrical heating systems. Comfort factors. Boiler and furnace room ventilation. Installation of heating units, ducts and vents to applicable safety codes. |
|  |  | (Components) | Types, characteristics and installation of air washers, humidifiers, de-humidifiers, spray eliminators and dust collectors. Filters; replaceable media, washable, absolute, electronic precipitators and activated carbon types. Types and installation of preheat coils, reheat coils, direct expansion coils for cooling and de-humidifying and electric duct heaters. |
|  |  | (Dampers) | Types, uses and installation of hand and automatically controlled parallel and opposed blade, face, face and by-pass dampers, blast gates and switches. |
|  |  | (Indicating Devices and Controls) | Types and characteristics. Installation methods. Care and use of filter gauge and manometers. |
|  |  | (Louvres and Grilles) | Types and installation of fixed and adjustable louvres, shutters, bird and fly screens, grilles and ceiling outlets. |

R.R.O. 1990, Reg. 1077, Sched. 1.

Note:  On January 1, 2011, Schedule 1 is revoked.  See:  O. Reg. 481/10, ss. 7, 9.

Schedule 2
SHEET METAL WORKER

Work Experience Training

| Item | Column 1 | Column 2 | Column 3 |
| --- | --- | --- | --- |
|  | Course | Subject | Work Instruction and Experience |
| 1 | Trade Practice General (As detailed in Schedule 1) | Safety | Safety rules and removal of all hazards. The Workers’ Compensation Act. The Building Code Act. The Occupational Health and Safety Act. Care and use of hand and power tools and equipment, measuring and layout devices. |
| 2 | Trade Practice | Methods and Applications | Parallel line method; developing patterns for profile, rectangular, round and elliptical forms. Stretchout patterns. |
|  |  |  | Radial line method; developing patterns for regular conical forms and pyramids. |
|  |  |  | Triangulation method; developing patterns for irregular tapered forms, transition patterns, double angle form patterns. |
| 3 | Trade Practice | Straight Shearing | Use of regular and combination pattern hand shears. Slitting. Split shearing. Square shearing. Inside cuts. Use of bench, slitting and scroll shears, portable unishears and nibblers. Shearing bolts, rivets, etc. with cold chisels. |
|  |  | Curved Shearing | Circle shearing; use of hand and power operated circle shears. Inside cuts; use of hand and power ring and circle shears. Use of power elbow shear for compound curves. |
| 4 | Trade Practice Forming Operations | Edges (Bench Tools) | Forming hemmed, open, burred, turned, flanged and crimped edges. Use of brakes, bar folders, bending bars, hand and power burring, turning, flanging and crimping equipment. |
|  |  | Shapes (Hand and Bench Tools) | Rolling; use of solid or slip rolls and funnel forming equipment. Forming on stakes. Profile bending; use of hand and power cornice brake. Panning; use of panning equipment. Raising, bumping and stretching. Annealing. |
|  |  | Beading and Swaging | Reinforcing with beads and swaging, swaging for stops. Use of hand and power beading and swaging equipment. |
|  |  | Wired Edges | Wiring straight edges. Use of hand and power wiring, wire and brace bending equipment. False wiring edges of round, rectangular and square containers. Use of hand and power false wiring and turning equipment. |
|  |  | Reinforcements | Diagonal bending. Reinforcing use of band, angle, tee and channel iron. Reinforcement fastening. |
|  |  | Reinforced Fibreglass | Lay-up and finishing of fibreglass ductwork, fittings and joints. |
| 5 | Trade Practice Joining Operations | Lock Seams | Groove seaming; internal and external seams. Use of hand and machine groovers. Brake, mallet and stake grooved seams. Use of hand and power setting down equipment. |
|  |  |  | Double seaming; allowance. Use of hand and power double seaming equipment, slide and snap locks. |
|  |  |  | Box locking; forming methods. Pittsburgh locking; use of brake and Pittsburgh lock former. Elbow edging. Use of hand and power elbow edging machines. Elbow seam closing; rigid and adjustable elbows. Use of hand and machine closing equipment. Standing seams. Use of seam closing devices. Collar locking. Use of hand and power collar locking equipment. |
|  |  | Rivetted Seams | Hole spacing, punching, drilling. Rivetting operations. Use of hand and power punches, drilling and rivetting equipment. |
|  |  | Welding | Spot welding; material preparation, locating and spacing welds. Use of portable and stationary spot welding equipment. |
|  |  |  | Oxy-acetylene; basic weld joints, material preparation allowances, positions. Welding, cutting and brazing operations. Arc welding. Flat and horizontal fillet welding. Carbon-arc cutting and brazing. |
|  |  | Soldering | Copper heating. Forging. Tinning. Fluxing. Defluxing after soldering. Acid use precautions. Solder application. Preheating. Soldering positioned joints. Surface tinning. Testing for leaks. Cleaning soldered joints by filing, sanding, buffing. |
|  |  | Cementing | Cleaning surfaces. Sealant use for high and low pressure duct systems; underground ducts of transite, vitreous clay, plastic; high temperatures. |
| 6 | Trade Practice Job-site Assembly and Erection Operations | Metal Roofing and Copings | Installation of sheet metal for roofs, monitors, towers, minarets, dormers, copings, pediments and corrugated roofs. Plastic roofing, rubber, lead and canvas. Waterproofing with grouting, plastic sealants, solder. Vapour barrier installation. Expansion allowance. Installation of curbs, saddles and roof sleeves. Safe roof work practices. |
|  |  | Flashings and Skylights | Installation of soakers, step, cape, hip, valley, ridge, gravel stops and nosing. Caulking, grouting and sealing. Skylight installation. |
|  |  | Gutters and Downspouts | Assembly of conductor heads, elbows, shoes, splash pans and scuppers. Installation of gutters and downspouts; use of plastic and solder sealants. |
|  |  | Plenums and Ducts | Assembly, installation and sealing. Provision of access doors and openings. |
|  |  | (Square Ducts) | Hanger positioning and installation. Duct assembly and installation. Fire dampers. Taping and sealing. |
|  |  | (Round Ducts) | Hanger and support installation. Sealing and taping. Plastic welding. Installation of collector systems, flues, stacks, chimneys and breechings. |
|  |  | (Internal Insulation) | Installation of duct insulation for acoustic lining, thermal resistance, condensation prevention. Sound baffles and silencers. |
|  |  | (Air Flow Measurement and Balancing) | Familiarization with normal velocities in low and high pressure duct systems. Average velocities at grilles, through air handling equipment. Material conveying velocities. Static and velocity pressures. Effects of poorly designed elbows, fan fittings, branch take-offs and transitions. |
|  |  | Roof Ventilators | Installation of stationary and gravity types, cowls, ventilators, louvred penthouses. Motorized roof vents and fan discharge heads. Bird and fly screens. |
|  |  | Exhaust Hoods | Assembly and installation of standard canopy, fume cabinets. Spray booths, drying booths, machine exhaust hoods. Interior and exterior hoods. Plastic hoods. Hood filters. Condensation control. Applicable codes. |
|  |  | Air Handling Equipment (Fans and Blowers) | Installation and mounting. Component alignment. Use of manufacturers drawings and specifications. |
|  |  | (Direct Fired Heating Equipment) | Familiarization with heat measurement; solar, waste products, coal, wood, oil, gas and electrical heating systems. Comfort factors. Boiler and furnace room ventilation. Installation of gravity and forced air heating units, ducts, vents and stacks to applicable safety codes. |
|  |  | (Components) | Installation of air washers, humidifiers, de-humidifiers, spray eliminators and dust collectors. Filters, electronic precipitators and activated carbon types. Preheat coils, reheat coils, direct expansion coils and electric duct heaters. |
|  |  | (Dampers) | Installation of hand and automatically controlled parallel and opposed blade, face, face and by-pass dampers, blast gates and switches. |
|  |  | (Indicating Devices and Controls) | Installation and adjustment. Use of filter gauges and manometers. |
|  |  | (Louvres and Grilles) | Installation of fixed and adjustable louvres, shutters, bird and fly screens, grilles and ceiling outlets. |

R.R.O. 1990, Reg. 1077, Sched. 2.

Note:  On January 1, 2011, Schedule 2 is revoked.  See:  O. Reg. 481/10, ss. 7, 9.

Back to top